3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500106r02

# E-meeting, Jan 20 – 24, 2025		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0524 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Multihop U2U Relay for non-IP type PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The “multi-hop indicator per RSC” is provisioned in the Relay Discovery Parameters, but it is unclear whether the values of RSC for multi-hop and for single-hop are different or they could be the same. It is preferred to clarify that they are different so that SA3 could reuse legacy security methods defined in R17. - For Discovery of non-IP PDU type with Model A, the description “For each protected user info in the received Direct Discovery set, …the relay will add the protected user info into its own Direct Discovery set” is misleading that the Relay could parse and re-generate a DDS. - There is no step 5 in Figure 6.3.2.6.3.2-1, the figure and descriptions need some update. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the values of RSC for multi-hop and for single-hop are different, in Policy/Parameter provisioning for multi-hop UE-to-UE Relay. - Clarify that in the non-IP type Model A Discovery procedure, the Announcement message may include a list of protected user info with corresponding path info, rather than modify and generating its own DDS. - Update step number of Figure 6.3.2.6.3.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and unclear description for multihop U2U Relay non-IP type PDU related procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5.1a, 6.3.2.6.3.1, 6.3.2.6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.1.5.1a	Policy/Parameter provisioning for 5G ProSe Layer-3 multi-hop UE-to-UE Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay:

-	when the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 End UEs supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay accessing 5G ProSe Layer-3 UE-to-UE Relays supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay over PC5 reference point.

-	when the UE is not "served by NG-RAN":

-	Indicates whether the UE is authorized to relay traffic for 5G ProSe Layer-3 End UEs supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay accessing 5G ProSe Layer-3 UE-to-UE Relays supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay over PC5 reference point.

-	The authorisation for a UE to act as a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-UE Relay Layer indicator indicates the associated RSC is offering 5G ProSe Layer-3 UE-to-UE Relay service. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Layer-3 multi-hop UE-to-UE Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement message and sending/receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 multi-hop UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to use a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay also authorizes the use of 5G ProSe Layer-3 UE-to-UE Relay discovery with Model A and Model B.

NOTE 2:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery:

-	Includes the parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery as a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay and for enabling the UE to connect to the 5G ProSe Layer-3 UE-to-UE Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery parameters (Relay Service Code(s), UE-to-UE Relay Layer indicator, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-UE Relay Layer indicator indicates the associated RSC is offering 5G ProSe Layer-3 UE-to-UE Relay service. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Layer-3 multi-hop UE-to-UE Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

-	Default Destination Layer-2 ID(s) for sending/receiving Relay Discovery Solicitation messages and receiving Relay Discovery Announcement message;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 multi-hop UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-UE Relay supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay as well as in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 End UE supporting 5G ProSe Layer-3 multi-hop UE-to-UE Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-UE Relay:

1)	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

NOTE 3:	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Discovery in clause 5.1.2.1 are expected to be aligned for direct and relayed discovery for UE to UE communication.

2)	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 4:	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay communication when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Communication in clause 5.1.3.1 are expected to be aligned for direct and relayed UE to UE communication.

NOTE 5: The RSC used for multi-hop service is different from the RSC used for single-hop service.

# * * * * Second change * * * *

6.3.2.6.3.1	Procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model A

Depicted in Figure 6.3.2.6.3.1-1 is the procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model A.

Figure 6.3.2.6.3.1-1: 5G ProSe multi-hop UE-to-UE Relay Discovery of non-IP PDU type with Model A

1.	The 5G ProSe UE-to-UE Relay has discovered End UEs in proximity and obtains the Direct discovery set from End UEs in proximity per RSC. (e.g. via a previous 5G ProSe UE-to-UE Relay Discovery or via secure PC5 connection between 5G ProSe U2U Relay and 5G ProSe End UE (refer to TS 33.503 [29])). 5G ProSe UE-to-UE Relay may also get the Hop-Limit from End UEs.

2.	The 5G ProSe UE-to-UE Relay sends a UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message contains the User Info ID of itself, a list of protected user info of End UEs, a list of paths indicated by an (ordered) list of User Info ID of Relays (initially it only contains the User Info ID of UE-to-UE Relay1 in the figure), hop count (which is set to 1 initially) corresponding to each protected user info in the message and optionally Hop-Limitcorresponding to each protected user info in the message. Each path corresponds to a user info in the Direct Discovery set.

The UE-to-UE Relay Discovery Announcement message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

3.	After receiving UE-to-UE Relay Discovery Announcement message(s) from another 5G ProSe UE-to-UE relay, a 5G ProSe UE-to-UE Relay checks the information associated to the received protected User Info(s). For each protected user info in the received message, if the corresponding path does not include the User Info ID of the 5G ProSe UE-to-UE Relay and the hop count is less than the corresponding Hop-Limit, the relay will add the protected user info into its own list of protected user info to be sent (if it is not added before) and add its own User Info ID into the path(s) of the corresponding protected user info(s) and increase the hop count by 1. If the relay already has the protected user info in its own list of protected user info to be sent with the hop count less than or equal to the received one, then it is up to relay's implementation to determine if add the protected user info again.

NOTE:	Based on implementation, Relay UEs can determine if two protected user infos are identical, e.g. by comparing the protected user info bit by bit if it is encrypted.

A 5G ProSe UE-to-UE Relay may associate the User Info ID and Layer-2 ID of neighbour UE-to-UE Relay(s) according to the Announcement message. The association may be used in the subsequent Link Management procedures.

4.	A 5G ProSe End UE or a 5G ProSe UE-to-UE Relay monitors Announcement message from a 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs and 5G ProSe UE-to-UE Relays determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

# * * * *Third change * * * *

6.3.2.6.3.2	Procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model B

Depicted in Figure 6.3.2.6.3.2-1 is the procedure for Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model B.

Figure 6.3.2.6.3.2-1: Multi-hop 5G ProSe UE-to-UE Relay Discovery of non-IP PDU type with Model B

1.	The Source End UE determines the Hop-Limit for discovery based on policy configuration (i.e. a mapping between Hop-Limit and RSC) or QoS parameters.

If the Hop-Limit is determined based on RSC, the Source End UE may not include the Hop-Limit in the Solicitation message. Otherwise, the Source End UE includes the Hop-Limit in the Solicitation message.

2a.	The Source End UE sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the information as described in clause 5.8.6.3.

3a.	If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s) of a 5G ProSe UE-to-UE Relay, the 5G ProSe UE-to-UE Relay may decide to send a 5G ProSe UE-to-UE Relay Discovery Solicitation message.

The 5G ProSe UE-to-UE Relay should drop the received Solicitation message when its own User Info ID is contained in the Path information of received Solicitation message, or if the hop count (i.e. the number of Relays included in the message) has reached the Hop-Limit of the received Solicitation message. The Hop-Limit may be obtained from the Solicitation message or may be decided based on RSC if it is not obtained from the message.

If the same Direct Discovery Set is received from different 5G ProSe-enabled UEs, the 5G ProSe UE-to-UE Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g. hop count, delay, channel quality of received messages, etc.).

NOTE 1:	If the Source End UE does not receive any response after a timeout, based on application requirement, it may increase the Hop-Limit and send the discovery message again.

NOTE 2:	Based on implementation, Relay UEs can decide to send the discovery message with the same direct discovery set at most once in order to control the total number of discovery messages transferred among Relays. To determine the identical direct discovery set, the Relay UE can compare the direct discovery set bit by bit if it is encrypted, or check the source End UE and target End UE User info if the direct discovery set is not encrypted.

4a.	A 5G ProSe UE-to-UE Relay sends a Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID of Relays in the path that has sent the Solicitation message. The hop count is increased by 1.

2b.-5b.	(Optional) Another transmission path of the Solicitation message.

6-7a. If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s) of the Target 5G ProSe End UE and the Target 5G ProSe End UE matches the user info (i.e. Application Layer ID) of the discoveree 5G ProSe End UE contained in the solicitation message, then the Target 5G ProSe End UE responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the information as described in clause 5.8.6.3.

The Target 5G ProSe End UE may choose a path and sends Response message to 5G ProSe UE-to-UE Relay(s) based on e.g. the PC5 signal strength of each solicitation message received, the number of hops to the Source End UE, the path information, order in which solicitation message were received, etc.

8a.	A 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Response message. A 5G ProSe UE-to-UE Relay may associate the User Info ID and Layer-2 ID of neighbour UE-to-UE Relay(s) according to the Response message. The association may be used in the subsequent Link Management procedures.

7b.-9b. (Optional) Transmission of another Response message corresponding to another path.

10.	The Source End UE may perform the UE-to-UE Relay(s)/path selection for subsequent Link Management procedures based on e.g. the PC5 signal strength of the received Response messages, the number of hops to Target 5G ProSe End UE.

# * * * * Fourth changes * * * *

#### 5.8.6.3	Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of non-IP PDU type

The 5G ProSe UE-to-UE Relay Discovery Announcement message (Model A) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and contains the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

-	For each user info in the Direct Discovery set:

-	Hop count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop.

-	(Optional) Hop-Limit: value that indicates the limit of the number of hops of the message.

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the source 5G ProSe End UE and includes the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

-	Hop count: indicates the number of hops that the message is already relayed. It is set to 0 initially by the Discoverer End UE and is increased by 1 per hop.

-	(Optional) Hop-Limit: value that indicates the limit of the number of hops of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than or equal to the (pre)configured maximum number of hops.

The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC and includes the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

-	Hop count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop.

-	(Optional) Hop-Limit: value that indicates the limit of the number of hops of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than or equal to the (pre)configured maximum number of hops.

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer UE.

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

-	Hop count: indicates the number of hops that the message is already relayed.

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the target 5G ProSe End UE matching the RSC includes the following the following parameters for multi-hop relaying in addition to as described in clause 5.8.4:

-	Hop count: indicates the number of hops that the message is already relayed.

-	Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE.

# * * * * End of changes * * * *