e3GPP SA WG2 Meeting #166-AdHoc-e	 S2-2500113

# Electronic Meeting, 20-24 January 2025								                 (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0154 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to NTZ support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Principles for NTZ support have been agreed in S2-2412987 CR#0136. The principles introduce normative behavior with “shall” statements that would require tests to be defined for the UE. However, the principles are very high level and do not allow the definition of detail normative aspects, and do not allow other WGs to take the necessary actions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies the UE procedures for NTZ enforcement in the UAV UE across NAS and AS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normative text for the support of NTZs cannot be supported in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[5]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[7]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[11]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[12]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[13]	ASTM F3411.19: "Standard Specification for Remote ID and Tracking".

[14]	ASD-STAN prEN 4709-002:2022-03: "Aerospace series - Unmanned Aircraft Systems - Part 002: Direct Remote Identification".

[15]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[16]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[17]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[18]	3GPP TS 26.502: "5G multicast-broadcast services; User Service architecture".

[19]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[20]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[21]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[22]	ECC Decision (22)07 (cept.org) https://docdb.cept.org/download/4240: "Harmonised technical conditions for the usage of aerial UE for communications based on LTE and 5G NR in the bands 703-733 MHz, 832-862 MHz, 880-915 MHz, 1710- 1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570- 2620 MHz harmonised for MFCN".

[23]	3GPP TS 38.331: "Radio Resource Control (RRC) protocol specification".

[24]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[25]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[X]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[Y]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3 .

[Z]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

# >>>> NEXT CHANGES<<<<

## 5.15	Support of No-Transmit Zones

The following principles are applied to support of NTZ for UAV UEs.

1)	NTZ enforcement applies to the UE that is a UAV UE (i.e. an aerial UE that is a UE with aerial subscription).

2)	UAV UEs supporting NTZ regulations are configured/provisioned with NTZ assistance information comprising of a mandatory list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and altitude restrictions) and optional time restriction, so that these UAV UEs are aware of the presence of NTZs.

3)	NTZ enforcement by UAV UE applies for both LTE and NR.

4)	An NTZ can map to one or more cells or a fraction of one or more cells, or overlap different cells in a mobile operator network.

NOTE 1:	Replanning of existing tracking areas and cells based on the presence of NTZs is not assumed.

NOTE 2:	NTZ mapping to cells will have no impacts on 3GPP specifications, hence application of NTZ to cells is UE implementation-specific.

5)		UAV UE shall neither transmit nor attempt to transmit in the NTZ (i.e. in the geographic area of the NTZ, within the restricted frequency bands, and within the time restrictions when provided), regardless of the service type including emergency services. NAS layer and AS layer in the UAV UE as defined in TS 24.501 [Y], TS 24.301 [X], TS 38.331 [23] and TS 36.331 [Z] are used to prevent the UAV UE to send data or signalling while in the NTZ

6)	For UAV UEs outside of the NTZ, the normal EPS/5GS procedures apply.

7)	UAV UEs supporting NTZ regulations are configured/provisioned with NTZ assistance information, so that these UAV UEs are aware of the presence of NTZs.

8)	A UAV UE configured with the NTZ assistance information is expected to perform the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	does not transmit any signalling or data within the restricted frequency bands upon applying NTZ enforcement.

c)	does not select/reselect any cells impacted entirely by an NTZ.

The UAV UE(s) configured with the NTZ assistance information is responsible to provide such assistance information to the AS layer in the UE  for the AS layer to perform NTZ enforcement in implementation dependent ways.

NOTE 3:	It is assumed that UAV UE is configured with NTZ assistance information before registering for UAS services with the 3GPP system, if the local regulation requires NTZ.

The above NTZ assistance information is made available to the UAV UE in following ways:

-	pre-configured;

-	configured by using mechanism that is out of 3GPP scope.

The UAV UE(s) configured with the NTZ assistance information is expected to perform the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	does not transmit any signalling or data within the restricted frequency bands upon applying NTZ enforcement.

c)	does not select/reselect any cells impacted entirely by an NTZ.

# >>>>END OF CHANGES<<<<