e3GPP SA WG2 Meeting #166-AdHoc-e	 S2-2500113r05

# Electronic Meeting, 20-24 January 2025								                 (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0154 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to NTZ support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Principles for NTZ support have been agreed in S2-2412987 CR#0136. The principles introduce normative behavior with “shall” statements that would require tests to be defined for the UE. However, the principles are very high level and do not allow the definition of detail normative aspects, and do not allow other WGs to take the necessary actions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies the UE procedures for NTZ enforcement in the UAV UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normative text for the support of NTZs cannot be supported in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

## 5.15	Support of No-Transmit Zones

The following principles are applied to support of NTZ for UAV UEs.

1)	NTZ enforcement applies to the UE that is a UAV UE (i.e. an aerial UE that is a UE with aerial subscription).

2)	UAV UEs supporting NTZ regulations are configured/provisioned with NTZ assistance information comprising of a mandatory list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and altitude restrictions) and optional time restriction, so that these UAV UEs are aware of the presence of NTZs.

3)	NTZ enforcement by UAV UE applies for both LTE and NR.

4)	An NTZ can map to one or more cells or a fraction of one or more cells, or overlap different cells in a mobile operator network.

NOTE 1:	Replanning of existing tracking areas and cells based on the presence of NTZs is not assumed.

NOTE 2:	NTZ mapping to cells will have no impacts on 3GPP specifications, hence application of NTZ to cells is UE implementation-specific.

5)		UAV UE shall neither transmit nor attempt to transmit in the NTZ (i.e. in the geographic area of the NTZ, within the restricted frequency bands, and within the time restrictions when provided), regardless of the service type including emergency services. NAS layer and AS layer in the UAV UE without using AS signaling, are used to prevent the UAV UE to send data or signalling while in the NTZ

6)	For UAV UEs outside of the NTZ, the normal EPS/5GS procedures apply.

7)	UAV UEs supporting NTZ regulations are configured/provisioned with NTZ assistance information, so that these UAV UEs are aware of the presence of NTZs.

The UAV UE(s) configured with the NTZ assistance information is responsible to provide such assistance information to the AS layer in the UE  for the UE’s AS layer to perform NTZ enforcement in implementation dependent ways.

NOTE 3:	It is assumed that UAV UE is configured with NTZ assistance information before registering for UAS services with the 3GPP system, if the local regulation requires NTZ.

The above NTZ assistance information is made available to the UAV UE in following ways:

-	pre-configured;

-	configured by using mechanism that is out of 3GPP scope.

The UAV UE(s) configured with the NTZ assistance information is expected to perform the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	does not transmit any signalling or data within the restricted frequency bands upon applying NTZ enforcement.

c)	does not select/reselect any cells impacted entirely by an NTZ.

A UAV UE configured with the NTZ assistance information is expected to perform the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	does not transmit any signalling or data within the restricted frequency bands upon applying NTZ enforcement.

c)	when in an NTZ, does not select/reselect any cells in restricted frequency bands.

d)	when in an NTZ, does not send measurement reports to the RAN about cells in restricted frequency bands.

# >>>>END OF CHANGES<<<<