3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500140

# Online, , 20th Jan 2025 - 24th Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1315 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Updates to VFL inference procedures e.g to address trusted AF and handling of parameters from analytics request |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  | Date: |  | 2025-01-08 |
| Category: | B |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |

| Reason for change: | - It is FFS whether additional parameters are needed to send from the VFL server to VFL client, e.g. parameters used in training phase and parameters from Analytics request. A. It is suggested to allow for the Feature ID to be provided for robustness. Otherwise a client can only participate in one VFL training operation and subsequent inference operations for an analytics ID (as it might be assigned to different features in different training operations), and using different clients than used for the model training is also precluded. B. The analytics request can contain the following parameters (as defined in Clause 6.1.3 of TS 23.228) that most likely cannot be handled by the VFL server alone but need to be forwarded to the VFL clients : - Preferred level of accuracy; - Reporting Thresholds; - Maximum number of objects requested (max); - Maximum number of SUPIs requested (SUPImax); - Time when information is needed; - Analytics Metadata Request; - Data time window; - Consumer NF's serving area or NF ID; - It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying There is a risk of inaccurate results when omitting certain VFL clients and thus VFL features in the inference phace. To enable a logging and improvment of the accuracy of results when problems are detected the information about features used to infere analytics results should be stored by the VFL server together with a timestmp to identiufy the analytics results. - Editor's note: It is FFS additional parameters are needed to send from the VFL client to VFL server. The VFL server may need or wish to provide the following parameters as defined in Clause 6.1.3 of TS 23.228) to Analytics consumers and needs related input from VFL cliemts to derive them: Analytics Metadata, Timestamp of intermediate result generation (required when ADRF is deployed), and optionally Validity period, and Confidence - Naf_VFLInference and Nnef VFLInference are meant to be request with AF VFL client as service producer. They are not meant for AF VFL server as service producer. Separate service operations with other parameters would be required. Also both subscription and request/reponse should be supported |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | . - Add Feature ID and parameters from the Analytics request to inference request - The VFL server stores information about Features used to derive analytics result to provide tracing and explainability of the results. - Add parameters from Analytics notification/response (as defined oin Clause 6.1.3) to Inference response/notification - Naf_Analytics and Nnef Analytics are used to trigged AF VFL server to perform inference. |  |  |  |
| Consequences if not approved: | - The VFL server cannot handle Analytics requests with many of the parameters defined in Clause 6.1.3. - The origin of VFL inference errors of not all features are used cannot be traced or explained. - The VFL server cannot provide Analytics trsponses/notifications with many of the parameters defined in Clause 6.1.3. - Unsuitable service operations are used to trigged AF VFL server to perform inference. |  |  |  |
| Clauses affected: | 6.2H.2.4.1, 6.2H.2.4.2 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications | TS/TR ... CR ... |
| affected: |  | x | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | x | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

##### 6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF is acting as VFL server

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period and Analytics Filter) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF determines the VFL Server for the requested analtyics, sends a request to NWDAF VFL server using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period and Analytics Filter or other parameters as defined in Clause 6.1.3.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. VFL Server selects clients(s) using information stored in the VFL training process. The server may select some or no clients, e.g. depending on the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients. If the server does not select VFL clients for all features, it shall store information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results, for instance in the ADRF, to anable to subsequently trace the origin of analytics results.

When no VFL Clients are selected, the VFL server may generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 - 6 (and 7 if step 1 was also skipped).

VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients including the Target of VFL inference = e.g. UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used and optionlly VFL inference filter, VFL Feature ID, Preferred level of accuracy, Reporting Thresholds, Maximum number of objects requested, Maximum number of SUPIs, Time when information is needed, Analytics Metadata Request, Data time window,	Consumer NF's serving area or NF ID (see Clause 6.1.3 for a definition of those parameters);.

2a.	For each NWDAF VFL client, the VFL Server NWDAF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference_Subscribe or Nnef_VFLInference_Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	VFL Client sends the client intermediate results to the VFL server and if requested in step 2 includes Analytics Metadata, Timestamp of intermediate result generation (required when ADRF is deployed), and optionally Validity period, and Confidence (see Clause 6.1.3 for a definition of those parameters).

The intermediate results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results. The server also aggregates possibly received analytics metadata, Timestamp of intermediate result generation (required when ADRF is deployed), Validity period, and Confidence.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF.

5b.	Each trusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference_Request response to the NWDAF VFL server.

6.	The VFL server may collect its local data and generate the intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate results when generates the combined inference output.

7.	Depending on request, the NWDAF VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

##### 6.2H.2.4.2	Inference procedure for vertical federated learning when AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when AF is acting as VFL server

The inference procedure when untrusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggerd by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped. The inference procedure triggerd by internal service logic of the AF acting as VFL server is out of 3GPP scope.

0.	[optional] Same as step 0 in clause 6.2H.2.4.1.

1.	[optional] Same as step 1 in clause 6.2H.2.4.1, to NEF using Nnef_Analytics_subscribe/request. NEF forwards the subscription request to AF using Naf_Analytics_subscribe/request.

Editor's note:	When the AnLF determine the VFL server AF is FFS. For example, during the training phase.

Editor's note:	The service name between the AnLF and untrusted AF as VFL server is FFS.

2.	Same as step 2 in clause 6.2H.2.4.1, to NEF using Nnef_VFLInference_subscribe. An untrusted AF includes the external NWDAF ID and sends the request to the NEF.

NEF converts any received external identifiers to internal identifiers and forwards the subscription request to NWDAF using Nnwdaf_VFLInference_subscribe.

Editor's note:	It is FFS additional parameters are needed to send from the VFL server to VFL client.

3.	Same as step 3 in clause 6.2H.2.4.1.

4.	Same as step 4 in clause 6.2H.2.4.1.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	Same as step 5 in clause 6.2H.2.4.1. , to NEF using Nnwdaf_VFLInference_Notify.

NEF converts any received internal identifiers to external identifiers and forwards the subscription notify to the untrusted AF using Nnef_VFLInference_Notify.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	Same as step 6 in clause 6.2H.2.4.1.

7.	[optional] Same as step 7 in clause 6.2H.2.4.1, to NEF using Naf_Analytics_Reply/Notify.

NEF converts any received internal identifiers to external identifiers and forwards the subscription Notify to NWDAF using Nnef_ Analytics_Reply/Notify.

8.	[optional] Same as step 8 in clause 6.2H.2.4.1.