3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500153r04

# Online, , 20th Jan 2025 - 24th Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5200 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4 Resolve open issues for mitigation actions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some procedures in Clause 4.17.9 are marked as tentative by ENs. However those procedures are in line with mitigation actions listed in 23.288: Table 6.22.3-3: Example mechanisms to mitigate and prevent the signalling storm Cause of the signalling storm; Example actions of NFs. NF abnormal signalling; SCP blocks or throttles messges originating from an NF/service with abnormal signalling.. NF abnormal signalling; SCP redirects requests towards an NF service producer with abnormal signalling towards another service producer.. NF abnormal signalling; SCP notifies communication peers of an NF with abnormal signalling about abnormal condition, e.g. high load.. Rejecting requesst and informing NF consumers allows peers to perform mitigation actions even if not all NFs subscribed to signalling storm analytics. |
| Summary of change: |  | Confirm tentative text bin Ens to allign with TS 23.288 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Open issues and missalignment between specifications. More NFs need to subscribe to signalling storm analytics to perform mitigation actions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.17.9	Delegated service discovery when NF service consumer and NF service producer are in same PLMN

Figure 4.17.9-1: Delegated NF service discovery when NF service consumer and NF service producer are in same PLMN

1.	The NF service consumer intends to communicate with an NF service producer. The NF service consumer sends the service request to an SCP. The request may include discovery and selection parameters necessary to discover and select a NF service producer instance. The discovery and selection parameters are included in the request by the NF service consumer in a way that the SCP does not need to parse the request body.

2.	The SCP may perform discovery upon the request either by interacting with an NRF using Nnrf_NFDiscovery service NRF or may use information collected during the previous interactions with an NRF (by the Nnrf_NFDiscovery service or Nnrf_NFManagement_NFStatusNotify service operation). The SCP together with the NRF authorizes the request. The SCP may subscribe at the NWDAF for analytics related to abnormal network behaviour/signalling storm and then obtain related information from the NWDAF as specified in TS 23.288 [50]. This subscription and related notifications may already have occurred before step 1 and the SCP may cache such information. The SCP selects the target NF service producer and may take the analytics information obtained from the NWDAF into consideration for that selection.

The SCP can reject request targeting NF service producers that may suffer a signalling storm or cause signalling storm with appropriate error information to enable NF service consumers to take mitigation actions, and the SCP can also updates its profile at the NRF to indicate that certain NF sets or SCP domains are no longer served by the SCP.

NOTE 1:	If the discovery and selection parameters in the request include a UE identity, e.g. SUPI or IMPI/IMPU, the SCP can resolve the requested NF's Group ID corresponding to the UE identity and then invoke the Nnrf_NFDiscovery service, as defined in clause 6.3.1 of TS 23.501 [2].

NOTE 2:	Rejecting requests can lead to service failures if the NF service consumer is not able to perform appropriate mitigation actions and thus needs to be performed with caution.

3.	If the NF service consumer is authorized to communicate with the NF service producer, the SCP forwards the request to the selected NF service producer according to the configuration of the Network Slice, e.g. the expected NF instances are only reachable by NFs in the same network slice.

Editor´s Note: If is ffs whether rhe SCP may use overload control defined in TS 29.500 [17] due to that the NF service consumer may suffer or cause a signalling storm, to enable the NF service producer to take mitigation actions.

4.	The NF service producer sends a response to the SCP. If the request in step 3 creates a resource in the NF service producer, such as depicted in Figure 4.17.9-1, the NF service producer responds with resource information identifying the created resource.

The SCP may use overload control defined in TS 29.500 [17] due to that the NF service producer may suffer or cause a signalling storm, to enable the NF service consumer to take mitigation actions.

5.	The SCP routes the response to the NF service consumer.

If the NF service consumer receives a resource address, it uses it for subsequent requests regarding the concerned resource. Otherwise, the procedure ends here.

6.	On a subsequent operation on the created resource, the NF service consumer addresses the resource via the resource address returned by the NF service producer at step 4.

7.	The SCP resolves the NF service producer address and selects a target NF service producer instance. The SCP then routes the request to the selected NF service producer instance. See the clause 6.3.1.0 of TS 23.501 [2] for the details of selection of a target NF service producer instance by SCP.

8.	The SCP delivers the request to the NF service producer.

9.	The NF service producer sends a response to the SCP. The NF service producer may respond with an updated resource information different to the one received in the previous response.

10.	The SCP sends a response to the NF service consumer. If the resource information was updated, the NF service consumer uses the received resource information for subsequent operations (requests) on the resource.

NOTE 2:	In the similar manner of handling the resource information, a binding indication may be also provided by the NF service producer and used for the subsequent requests of the NF service consumer. See more details in clause 4.17.12.