3GPP TSG-WG SA2 Meeting #166-AHE                                       	S2-2500157r01

# Online, January 20-24, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5202 | rev | --- | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Allow NSSF to provide slice validity time during slice selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-1-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is a mirror CR of a corresponding R18 CR. Currently, NSSF can provide the AMF slice validity time updates only in notification of Nnssf_NSSAIAvailability service. However, it cannot provide the AMF the initial/current value of slice validity time in a response to slice selection GET request. This is not a healthy situation when AMF (and consequently the UE) does not receive from NSSF a complete picture about a slice, but rather needs to rely on using a separate service (Nnssf_NSSAIAvailability) to obtain additional basic information about the slice. For example, when NS selection is triggered by UE registration, we may end up in an unstable state where the UE receives from the AMF Allowed NSSAI and configured NSSAI in the registration accept message without receiving slice validity time for one or more slices that are included in the Allowed NSSAI or/and configured NSSAI. This CR proposes to fix this situation. R01 proposes to update the Subscribe service operation in Nnssf_NSSAIAvailability service such that AMF can include immediate report flag when it subscribes to slice validity time at the NSSF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to allow the NSSF to provide the AMF slice validity time in a response to slice selection GET request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NSSF will not be able to provide the AMF slice validity time in a response to slice selection GET request. Consequently, the UE may not be aware of the validity time of a slice. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.16.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 5.2.16.2	Nnssf_NSSelection service

##### 5.2.16.2.1	Nnssf_NSSelection_Get service operation

Service operation name: Nnssf_NSSelection_Get

Description: This service operation enables Network Slice selection in both the Serving PLMN and HPLMN. It also enables the NSSF to provide to the AMF the Allowed NSSAI and the Configured NSSAI for the Serving PLMN. It allows also to provide the NSAG information which is applicable (clause 5.15.14 of TS 23.501 [2]).

It may be invoked during Registration procedure, during inter-PLMN mobility procedure, during PDU Session Establishment procedure or during UE Configuration Update procedure. When invoked during Registration procedure it may possibly trigger AMF re-allocation. When invoked during PDU Session Establishment procedure it may be invoked in the VPLMN or in the HPLMN; if invoked in the VPLMN it returns the hNRF selected by the hNSSF and if applicable, the value of the HPLMN NSI ID. When invoked during UE Configuration Update procedure or inter-PLMN mobility procedure it may be invoked in the Serving PLMN.

NOTE 1:	The list of events, which trigger invoking of the Nnssf_NSSelection_Get service operation, is not exhaustive.

NOTE 2:	The NSSF can determine the serving network and Access Type from the TAI, as described in TS 29.571 [70].

Inputs, Required: None.

Inputs, Conditional Required:

If this service operation is invoked during Registration procedure for Network Slice selection or UE Configuration Update procedure, then the following inputs are required:

-	Subscribed S-NSSAI(s) with the indication if marked as default S-NSSAI, PLMN ID of the SUPI, TAI, NF type of the NF service consumer, Requester ID.

If this service operation is invoked during Registration procedure to get the slice mapping information in the case of Indirect Network Sharing as specified in clause 5.18.5 of TS 23.501 [2], then the following inputs are required:

-	HPLMN S-NSSAIs, PLMN ID of the SUPI.

If this service operation is invoked to derive the S-NSSAI for the serving PLMN (as described in clause 4.11.1.3.3), the following inputs are required:

-	S-NSSAIs for the HPLMN associated with established PDN connection, PLMN ID of the SUPI, NF type of the NF service consumer, Requester ID.

If this service operation is invoked by target AMF during inter-PLMN mobility procedure, the following inputs are required:

-	S-NSSAIs for the HPLMN, PLMN ID of the SUPI, TAI.

If this service operation is invoked during PDN Connection Establishment in the Serving PLMN in EPS by a SMF+PGW-C, the following inputs are required:

-	Subscribed S-NSSAIs for the UE, PLMN ID of the SUPI, NF type of the NF service consumer, Requester ID.

If this service operation is invoked during PDU Session Establishment procedure in the Serving PLMN then the following inputs are required:

-	S-NSSAI, non-roaming/LBO roaming/HR roaming indication, PLMN ID of the SUPI, TAI, NF type of the NF service consumer, Requester ID.

Inputs, Optional:

If this service operation is invoked during Registration procedure for Network Slice selection or UE Configuration Update procedure, then the following inputs are provided if available:

-	Requested NSSAI, Mapping Of Requested NSSAI, Default Configured NSSAI Indication, NSSRG Information, UE support of subscription-based restrictions to simultaneous registration of network slice feature Indication, UDM indication to provide all subscribed S-NSSAIs for UEs not indicating support of subscription-based restrictions to simultaneous registration of network slices, Allowed NSSAI for current Access Type, Allowed NSSAI for the other Access Type and the corresponding Mapping Of Allowed NSSAIs for current Access Type and other Access Type, Rejected S-NSSAI(s) for RA, UE support of NSAG Information.

If this service operation is invoked during PDU Session Establishment procedure, then the following input is optional:

-	HPLMN S-NSSAI that maps to the S-NSSAI from the Allowed NSSAI of the Serving PLMN.

Outputs, Conditional Required:

If this service operation is invoked during Registration procedure for Network Slice selection or UE Configuration Update procedure, then one or more of the following outputs are required:

-	Allowed NSSAI, Configured NSSAI; Target AMF Set or, based on configuration, the list of candidate AMF(s).

If this service operation is invoked during Registration procedure to get the slice mapping information in the case of Indirect Network Sharing as specified in clause 5.18.5 of TS 23.501 [2], then the following outputs are required:

-	HPLMN S-NSSAIs, Mapping of HPLMN S-NSSAIs in the hosting operator's network.

If this service operation is invoked during inter-PLMN mobility procedure, then one or more of the following outputs are required:

-	Allowed NSSAI.

If this service operation is invoked to derive the S-NSSAI for the serving PLMN (as described in clause 4.11.1.3.3), the following output is required:

-	S-NSSAIs for the HPLMN associated with established PDN connection, Mapping of S-NSSAIs associated with established PDN connection in the Serving PLMN.

If this service operation is invoked during PDN Connection Establishment in the Serving PLMN in EPS by a SMF+PGW-C, the following outputs are required:

-	Subscribed S-NSSAIs for the UE, Mapping of S-NSSAIs associated with the subscribed S-NSSAIs for the UE in the Serving PLMN.

If this service operation is invoked during PDU Session Establishment procedure, then the following outputs are required:

-	The NRF to be used to select NFs/services within the selected Network Slice instance.

Outputs, conditional Optional:

If this service operation is invoked during UE Registration procedure or UE Configuration Update procedure, then one or more of the following outputs are optional:

-	Mapping Of Allowed NSSAI, Mapping Of Configured NSSAI, NSI ID(s) associated with the Network Slice instances of the Allowed NSSAI, NRF(s) to be used to select NFs/services within the selected Network Slice instance(s) and NRF to be used to determine the list of candidate AMF(s) from the AMF Set, rejected S-NSSAI with cause of rejection, Target NSSAI, the NSAG information (defined in clause 5.15.14 of TS 23.501 [2]), validity time for each S-NSSAI in the Allowed NSSAI and/or Configured NSSAI as applicable.

If this service operation is invoked during inter-PLMN mobility procedure, then the following output is optional:

-	Mapping Of Allowed NSSAI.

If this service operation is invoked during PDU Session Establishment procedure, then the following output is optional:

-	NSI ID associated with the S-NSSAI provided in the input.

##### 5.2.16.3.4	Nnssf_NSSAIAvailability_Subscribe service operation

Service operation name: Nnssf_NSSAIAvailability_Subscribe

Description: This service operation enables a NF Service Consumer (e.g. AMF) to subscribe to a notification of any changes in status of the NSSAI availability information (e.g. S-NSSAI validity time, S-NSSAIs and optionally NSI IDs available per TA and the restricted S-NSSAI(s) and optionally NSI IDs per PLMN in that TA in the serving PLMN of the UE) upon this is updated by another AMF. This service also enables the NF Service Consumer (e.g. AMF or NSSF in the VPLMN) to subscribe to a notification of Network Slice Replacement and Network Slice Instance Replacement.

Inputs, Required: Callback URI of the NF Service Consumer:

Inputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following inputs are required:

-	list of TAIs supported by the NF service consumer, event to be subscribed.

If this service operation is invoked to subscribe for changes in status of the NSSAI validity time information, then the following inputs are required:

-	The S-NSSAIs, event to be subscribed.

-	Immediate reporting flag.

If this service operation is invoked to subscribe to a notification for Network Slice Replacement, then the following inputs are required:

-	for the S-NSSAI of the serving PLMN: set of S-NSSAIs served by the NF Service Consumer that may be replaced, NF type of the NF Service Consumer (e.g. AMF), Requester ID, event to be subscribed.

-	for HPLMN S-NSSAI: S-NSSAIs for the HPLMN, NF type of the NF Service Consumer (e.g. AMF or NSSF), Requester ID and PLMN ID, event to be subscribed.

If this service operation is invoked to subscribe to a notification when the Network Slice instance is congested or no longer available, then the following inputs are required:

-	The S-NSSAIs, NSI IDs, event to be subscribed.

Inputs, Optional: Expiry time.

Outputs, Required: Subscription Correlation ID.

Outputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following outputs are required:

-	Expiry time (if present in the request, may be included in the response based on operator's policy and taking into account the expiry time present in the request (i.e. should be less than or equal to that value); if not present in the request, may be included in the response based on operator's policy. Whatever the case, if not included in the response, this means that the subscription is valid without an expiry time).

If Immediate reporting flag was provided and if this service operation is invoked by subscription for event notification for update of slice validity time, then the following inputs are required:

-	The S-NSSAI(s) and associated validity time for each S-NSSAI.

Outputs, Optional:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following outputs are optional:

-	A list of TAIs and for each TAI, the S-NSSAIs and optionally NSI IDs supported by the AMF and 5G-AN and authorized by the NSSF for the TAI and a list of S-NSSAIs and optionally NSI IDs restricted per PLMN for the TAI. The NSSS indicates to the AMF that the S-NSSAI(s) are not available in the corresponding TAIs.

# *** End of Change ***