3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500159r09

# Electronic Meeting, 20-24 January 2025								                 (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5893 | rev | 1 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Joint admission control for multi-modal services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R2-2409272, RAN2 requested SA2 to provide the Multi-Modal Service ID (MMSID) to NG-RAN, e.g.,to enable joint admission control for multi-modal services. While it is generally feasible to provide the MMSID to NG-RAN, it is important to consider clarity of specifications given that the MMSID is a parameter that has been specified in Rel-18 and as per Rel-18 is not provided to NG-RAN. SA4 clarified in S4-242223 that “not all applications require all QoS flows to be granted. […] For instance, it may be acceptable to an application if the QoS flow for video cannot be established with the requested QoS requirements but it may not be acceptable if the QoS flow for audio cannot be established”. Furthermore, as clarified by SA4 in S4-242223, “the conditions under which particular QoS flows are considered essential for an application to continue are application specific and cannot be determined by 5GS.”. Thus, the CR proposes providing the MMSID to RAN but its use should be carefully considered by RAN groups. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5GS may optionally support MMSID awareness at RAN If the AF provides the Multi-modal Service ID, then the PCF may include the Multi-modal Service ID in the PCC rules generated for these data flows. If a Multi-modal Service ID is included in PCC rules, then the SMF may provide the Multi-modal Service ID to NG-RAN when establishing QoS flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of MMSID RAN awareness for multi-modal services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.1, 5.37.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

5.37.1	General

This clause provides an overview of 5GS functionalities for support of XR services (AR/VR applications) and interactive media services that require high data rate and low latency communication, e.g. cloud gaming and tactile/multi-modal communication services according to service requirements documented in TS 22.261 [2]. The standardized 5QI characteristics for such interactive services are provided in Table 5.7.4-1 and TSCAI is used to describe the related traffic characteristics as defined in clause 5.27.2. Further enhancements for these interactive media services are as follows:

-	The 5GS may support QoS policy control for multi-modal traffic, see clause 5.37.2.

-	The 5GS may support network information exposure which can be based on ECN markings for L4S, see clause 5.37.3 or 5GS exposure API, see clause 5.37.4.

-	The 5GS may support PDU Set based QoS handling including PDU Set identification and marking, see clause 5.37.5.

-	The 5GS may ensure that the UL and DL packets together meet the requested round trip delay and also update the delay for UL and DL considering QoS monitoring results, see clause 5.37.6.

-	The 5GS may perform per-flow Packet Delay Variation (PDV) monitoring and policy control according to AF provided requirements, see clause 5.37.7.

-	The 5GC may provide traffic assistance information to the NG-RAN to enable Connected mode DRX power saving, see clause 5.37.8.

-	The 5GS may consider dynamically changed traffic characteristics for better resource management, see clause 5.37.10.

-	The 5GS may support traffic identification for multiplexed media flows in the same transport layer connection, see clause 5.37.11.

-	The 5GC may perform PDU Set Importance based transport level packet marking, see clause 5.8.2.7.

-	The 5GS may support Multi-modal Service ID (MMSID) awareness at RAN for multi-modal flows, see clause 5.37.2.

5.37.2	Policy control enhancements to support multi-modal services

A multi-modal service is a communication service that consists of several data flows that relate to each other and that are subject to application coordination. The data flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources(e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs).

For the single UE case, it is expected that those data flows are closely related and require strong application coordination for the proper execution of the multi-modal application and therefore, all those data flows are transmitted in a single PDU session.

The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each data flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements:

-	The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application.

-	The AF may provide QoS monitoring requirements for data flows associated to a multi-modal service to the PCF . The PCF generates the authorized QoS Monitoring policy for each data flow.

NOTE:	In order to start the QoS monitoring for the data flows associated to a multi-modal service within a certain period of time, the PCF needs to receive the QoS monitoring requirements for those data flows from AF within a single request or, in case of multiple requests, within a short period of time.

5GS may optionally support MMSID-awareness at RAN for multi-modal services,

If the AF provides a Multi-modal Service ID for data flows to 5GS, then the PCF includes it in PCC rules, and the SMF forwards it to NG-RAN in N2 SM information when establishing QoS flows.

NOTE y:	 Multi-modal Service ID is provided by the AF and is recommended that unique values are used within 5GS. Uniqueness can be ensured through an SLA between operator and application service provider, and the format of the unique values are left to Stage 3.

NOTE x:	 The use of MMSID by NG-RAN is described in RAN specifications.

In addition to the features that are provided for the case that the data flows are associated with a single UE, the following features are provided for the case where the data flows are associated with more than one UE:

-	The same DNN/S-NSSAI combination for the multi-modal service should be selected by each of the involved UEs. The URSP Rule evaluation framework is used to ensure that the same DNN/S-NSSAI is selected.

-	The AF should use the same Multi-modal Service ID in the interactions with the PCF(s) for all the involved UEs that relate to a multi-modal service. The PCF may take this information into account (e.g. to apply a specific QoS policy) when processing each AF request independently. The data flows contribute to the service experience, but are still valid stand-alone, as they are transmitted over separate PDU Sessions to/from the involved UEs.

-	If multiple PCFs are involved, the PCFs take policy decisions according to the input provided by the AF. There is no support for policy coordination among the multiple PCFs in this Release of the specification. Policy decisions are taken by each PCF separately on a per PDU Session basis.

# >>>>END OF CHANGES<<<<