3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500171r02

# Electronic Meeting, 20-24 January 2025								                 (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1533 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PS Data Off status change reporting to DCSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to support the report of PS Data Off status to DCSF, the following change to Nimsas_SessionEventControl service operation has been approved accompanied with an EN: The AS may notify the DCSF about the UE 3GPP PS Data Off status. Editor's note: The service design for AS notifying DCSF needs more consideration. It is not clear, however, how the DCSF is notified when the user changes the PS Data Off status during the period of IMS Session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The change clarifies that the IMS AS will notify the DCSF when the user changes the PS Data Off status during the period of IMS Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompletion of PS Data Off status reporting to DCSF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.3.2, AA.2.4.2.1, AC.2.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES <<<<

## X.3.2	Network Enforcement of SIP-Based 3GPP PS Data Off Exempted Services

Application Servers implementing the SIP-based services shall enforce the SIP based 3GPP PS Data Off Exempted services for all UEs.

Each Application Server shall be configured with up to two lists of 3GPP PS Data Off Exempt Services, one list for non-roaming users, and the other list for users roaming in the various VPLMNs with whom roaming agreements exist.

The AS shall become aware of the UE 3GPP Data Off status (active/inactive) at IMS (re-)Registration through third party registration. If the UE has changed its 3GPP PS Data Off status from inactive to active, the AS shall ensure that only SIP-based services which are part of the SIP-based 3GPP PS Data Off Exempt Services are permitted.

In the context of IMS Data Channel, the AS may further notify the DCSF about the UE’s 3GPP PS Data Off status and subsequently notifies the DCSF about any changes to the UE’s 3GPP PS Data Off status.

If the UE has changed its 3GPP PS Data Off status from active to inactive, the AS shall also let through the terminating requests to the UE for services that were not Data Off exempt.

NOTE 1:	The AS could be implemented in an existing AS e.g. SCC AS or distributed across other AS's.

NOTE 2:	The operator needs to ensure coordinated lists of 3GPP Data Off Exempt services provisioned in the UE and configured the network.

NOTE 3:	The notification about the UE 3GPP PS Data Off status to the DCSF can be used by operators for further control, which is out of scope.

# >>>>NEXT CHANGE<<<<

#### AA.2.4.2.1	General

Service description: This service enables the consumer to be notified about session events when served IMS subscribers take part in IMS sessions.

The following operations are available for this service:

-	Explicit subscription to receive session events. This is not specified in this Release.

-	Notifying IMS session control events of a specific IMS subscriber to NFs, e.g. DCSF, based on local configuration in the IMS AS.

IMS AS processing of the session event is paused until the consumer acknowledges reception of the session event notification. The consumer is given the possibility to apply session media control and policies by invoking the Nimsas_MediaControl service before giving IMS AS a session event notification acknowledgement.

If the consumer does not apply media control/policies to a session event, IMS AS continues processing and forwards the event as is (including the media state) to calling/called party.

The AS may notify the DCSF about the UE’s 3GPP PS Data Off status and subsequently notifies the DCSF about any changes to the UE’s 3GPP PS Data Off status based on implicit subscription to the PSDataOffStatusChangeEvent.

# >>>>NEXT CHANGE<<<<

#### AA.2.4.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber that has IMS data channel subscription.

Inputs, Required: Session ID, Event ID,

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator, 3GPP PS Data Off Status, Media info list, Media information set.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session case indicates if this is an originating or terminating IMS session. Event initiator indicates initiator of the event, i.e. 'served IMS subscriber' vs 'remote IMS subscriber'. 3GPP PS Data Off Status is the active/inactive of 3GPP PS Data Off of the UE the network is serving.

Media info list includes for each media in the list:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS,

-	media specification: This depends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type: DC, Audio, or Video.

When the media type is "DC", the elements below are derived from the SDP received by the IMS AS in an SIP INVITE or a re-INVITE related to an IMS Data Channel and the corresponding DTLS connection.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security set up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

Media information set:

-	For Event ID "3rdPartySessionCreation", the media information set is described in clause AA.2.4.4.2.

-	For Event ID "3rdPartySessionUpdate", the media information set is described in clause AA.2.4.4.3.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported EventIDs and related parameters.

Table AA.2.4.2.2-1: List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| SessionEstablishmentProgressEvent | Media info list |
| SessionEstablishmentAlertingEvent | Media info list |
| SessionEstablishmentSuccessEvent | Media info list |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | Event initiator, Media info list, Calling ID, Called ID, Session case |
| MediaChangeSuccessEvent | Media info list |
| MediaChangeFailureEvent | Media info list |
| SessionTerminationEvent | Session case |
| 3rdPartySessionUpdate | Media information set |
| 3rdPartySessionCreation | Media information set |
| PSDataOffStatusChangeEvent | 3GPP PS Data Off Status |

# >>>>END OF CHANGES<<<<