SA WG2 Meeting #166-AdHoc-e	S2-2500175r01

E-meeting, 20 – 24 January 2025	(revision of S2-24xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5897 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Parameters for EIF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This paper focuses on the EIF discovery and selection with adding the parameters which are used for EIF selection/discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding DNN, S-NSSAI, serving area and supported services to be considered by the NF consumer for EIF selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to select/discover the EIF is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change* * *

6.3.27	EIF discovery and selection

The NF consumers (e.g. NEF, AF, PCF) may utilise the NRF services to discover EIF instance(s) or the EIF selection information may be locally configured in NF consumers.

The following factors may be considered by the NF consumer for EIF selection:

-	EIF Serving Area information.

-	DNN.

-	S-NSSAI.

-	Event ID(s) (e.g., UE or PDU session granularities)and services supported by an EIF.

NOTE: For requirement of UE level energy consumption information exposure, the DNN and S-NSSAI are not used.

* * * End of Change * * *