3GPP TSG-SA2 Meeting #166AH-E	S2-2500209r01

# January 20-24,   2025, Electronic				 							  (revision of S2-25CCCCC)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1541 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Update to Network Capability Exposure Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Resolve Editor notes, and make some corrections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Expand the procedure updating an existing IMS session for a Data Channel UpdateAdd the procedures of IMS data channel capabilies exposure. - Complete the procedure for creating a stand alone IMS DC - Complete the procedure for terminating an IMS DC - Correct the Session delete procedue. The deletion request need not be reported to the DCSF. There is no reason why the DCSF would reject a session deletion request. Also unclear what will the DC AS do in case of a rejection. - Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing and incorrect specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.2.2, AG.1, AG.2.1, AG.2.2, AG.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### AA.2.4.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber requesting use of IMS data channel media.

Inputs, Required: Session ID, Event ID,

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator, Media info list.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session case indicates if this is an originating or terminating IMS session. Event initiator indicates initiator of the event, i.e. 'served IMS subscriber' vs 'remote IMS subscriber'. 

Media info list includes for each media in the list:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS,

-	media specification: This depends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type: DC, Audio, or Video.

When the media type is "DC", the elements below are derived from the SDP received by the IMS AS in an SIP INVITE or a re-INVITE related to an IMS Data Channel and the corresponding DTLS connection.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security set up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported EventIDs and related parameters.

Table AA.2.4.2.2-1: List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| SessionEstablishmentProgressEvent | Media info list |
| SessionEstablishmentAlertingEvent | Media info list |
| SessionEstablishmentSuccessEvent | Media info list |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | Event initiator, Media info list, Calling ID, Called ID, Session case |
| MediaChangeSuccessEvent | Media info list |
| MediaChangeFailureEvent | Media info list |
| SessionTerminationEvent | Session case |
| ExternalSessionUpdateEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| ExternalSessionCreateEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |

# * * * * Next change * * * *

#### AA.2.4.4.3	Nimsas_ImsSessionManagement_Update service operation

Service operation name: Nimsas_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the operated media is Data Channel.

-	Media Parameter: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it indicates the Data Channel Type (ADC or BDC):

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P), Application Binding Information, and if P2A ADC is selected, the party which this request is targeted.

-	If P2A or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: Calling ID, Called ID, Notification Target Address and Notification Correlation ID, IMS AS instance serving the target subscriber.

Calling ID: It is the public identity of calling IMS Subscriber.

Called ID: It is the public identity of called IMS Subscriber.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1.

Outputs, Required: Result indication.

Outputs, Optional: None.

# * * * * Next change * * * *

## AG.2.3	Terminating standalone Data Channel session

Figure AG.2.3-1 shows the terminating procedure of a standalone data channel session. A standalone data channel can be a standalone bootstrap or standalone application data channel.

Figure AG.2.3-1: Terminating of a standalone data channel Session

1.	The DC AS sends a Nnef_ImsSessionManagement_Delete request to the NEF requiring terminating a standalone DC session with UE A. The request incudes the Session ID to be terminated.

2.	Unless provided in the incoming request to the NEF, the NEF queries the HSS for the address of the IMS AS-2 serving the specific UE if needed.

3.	The NEF sends a Nimsas_ImsSessionManagement_Delete request to the IMS AS-2 requiring to terminate the standalone data channel session identified by the Session ID.

4.	The IMS AS-2 returns a successful Nimsas_ImsSessionManagement_Delete response to the NEF.

5.	The NEF returns a successful Nnef_ImsSessionManagement_Delete response to the DC AS.

6.	The IMS AS-2 sends the BYE request according to existing IMS procedures to terminate the data channel session.

7.	The IMS AS-2 sends a Nimsas_SessionEventControl_Notify request including the event SessionTerminationEvent to notify  DCSF-2 that a session is terminated.

8.	If the DC AS included a subscription to the DC session events to the Nnef_ImsSessionManagement_Delete request in Step 1, the IMS AS notifies the NEF for the progress of the session deletion with the Nimsas_SessionManagement_Notify service operation, based on Table AA.2.4.4.5-1.

9.	The NEF notifies the DC AS for the progress of the session deletion with the Nnef_SessionManagement_Notify service operation.

# * * * * End of changes * * * *