3GPP TSG-SA2 Meeting #166AH	S2-2500219r07

Emeeting, Elbonia,  20 – 24th January, 2025

# Title:	[draft] Reply LS on Clarification request on usage and control of UE Local Configuration.

# Response to:	LS (C1-247121/ S2-2500034) on Clarification request on usage and control of UE Local Configuration

# Release:	Rel-19

# Work Item:	TEI19

Source:                  [Nokia, Will be] SA2

To:                          CT1, GSMA URSPWI

Cc:                          GSMA TSG

Contact Person:

Srinivas Garikipati

Srinivas.Garikipati@Nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

None

1. Overall Description:

SA2 thanks CT1 for their LS on Clarification request on usage and control of UE Local Configuration.

SA2 provides the following updates to CT1 and GSMA URSPWI.

CT1 Query Details (GSMA URSPWI):

Q1. Can UE local configuration entries be created both by the operator and the user?

Answer: As per 3GPP TS 23.503, UE Local Configuration is defined as:

UE Local Configuration: Information about the association of an application to either a PDU Session or to non-seamless Offload is configured in the Mobile Termination (MT) and in the Terminal Equipment (TE). For example, UE Local Configuration can include operator specific configuration (e.g. operator provided S-NSSAI(s)), or application specific parameters to set up a PDU Session or end user configuration for specific applications.

Based on the above definition, both the operator and end user can create UE local configuration entries.

If so

Q1.1 can the operator also create configuration for non seamless non-3GPP offload or only configuration for associating an application to a PDU session association?

Answer: Yes, the operator can also create configuration for non-seamless non-3GPP offload.

Q1.2. Should the entries in the UE Local Configuration entered by one entity e.g., the user be protected so that the other entities e.g., operator or application cannot delete or modify them?

Answer: In general, the entries in UE local configuration need to be maintained in a consistent manner. Ensuring that information in UE local configuration is maintained in a consistent manner is out of 3GPP scope and up to UE implementation.

Q2. If the operator can populate the UE local configuration for non seamless non-3GPP offload (see question 1.1.), what is the intended use of the RSD parameter "Non Seamless Offload Indication" defined in the URSP rule?

Answer: UE local configuration is optional and may or may not be configured on the MT or TE. If UE local configuration is configured on the MT or TE, it takes precedence over URSP rules (for non-seamless non-3GPP offload). However, if UE local configuration is not configured on the MT or TE, the operator can request non seamless non-3GPP offload by use of the RSD parameter "Non Seamless Offload Indication" defined in the URSP rule. Further, operator specific UE local configuration is not needed for the case when the non-seamless non-3GPP offload does not go through the ePDG or N3IWF entities that are defined by 3GPP.

CT1 asks SA2 to confirm the above response.

SA2 Answer: SA2 concurs with the answer provided by CT1. Furthermore, SA2 would like to point out that the indication in the RSD for Non-Seamless is used by the UE to determine that the matching application should be non-seamlessly offloaded to non-3GPP access (i.e. outside of a PDU Session), and that this traffic is not routed via ePDG or N3IWF.

Q3. Any particular reason why what seem to be three distinct sets of UE local configurations are captured in the same object?

Answer: CT1 asks SA2 to provide appropriate response.

SA2 Answer:

As per 3GPP TS 23.503, UE local configuration can include three distinct sets of configurations, namely,  operator specific configuration, application specific configuration and end user configuration. This offers a flexible tool to address all possible deployment cases. However, it does not mean that all three sets of configurations need to be present for a particular UE.

In certain implementations, these entries may be managed by an operating system or other higher layer software on the UE. The UE local configuration can possibly include overlapping and conflicting entries and as such to maintain these entries in a consistent manner based on end user preferences they may be included in the same object. How the information is organized and maintained in UE local configuration is up to UE implementation.

2. Actions:

To CT1 group and GSMA URSPWI.

ACTION:

SA2 kindly asks CT1 and GSMA URSPWI to take above SA2 information into account.

3. Date of Next SA2 Meetings:

|  | DATES | LOCATION | CITY |
| --- | --- | --- | --- |
| SA2#167: | 17 - 21 February, 2025 | Athens | Greece |
| SA2#168: | 07 - 11 April, 2025 | Goteborg | Sweden |

SA2 Meeting calendar can be found at:

https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/