3GPP TSG-SA2 Meeting #166-Ad Hoc-e	S2-2500220r03

# Online, , 20th Jan 2025 - 24th Jan 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5902 | rev |  | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On MWAB mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR removes an editor's note and just keeps reference to RAN TSs for the mobility behaviour when the UE moves with a MWAB. Ther is no impact to SA2 specifications. In addition, some reference to RAN specifications for the gNB behaviour when the UE needs to change AMF due to mobility is added. The main rationale for this Cr is linked to the incoming LS from RAN3 in S2-2500053 where RAN 3 confirms that mobility without CN specifications impacts is possible even when AMF change needs to occur and from the action on SA2 they are interested in avoiding impact to CN specifications, hence system-wide it could be sufficient to refer to the behaviour in RAN specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editor's note is removed in clause 5.49.5.2 and reference to RAN specifications for the gNB behaviour when the UE needs to change AMF due to mobility is added in clause . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.49.5	Mobility support of MWAB

5.49.5.1	UE mobility when a MWAB-gNB cell is involved

The existing UE mobility procedures without MWAB UE specific enhancements can be reused to handle a UE mobility to and from a MWAB-gNB while the UE is in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED modes, irrespective of whether the MWAB is static or the MWAB is moving, i.e.:

-	For UEs in RRC_IDLE and RRC_INACTIVE state, procedure for cell (re) selection as specified in TS 38.304 [50] and TS 23.122 [17] for RRC_IDLE and RRC_INACTIVE is used irrespective of cells are belonging to a MWAB-gNB or a gNB.

-	For UEs in RRC_CONNECTED state, the MWAB-gNB or gNB triggers the handover procedures to the neighbouring cells as specified in TS 38.300 [27] and TS 23.502 [3] irrespective of whether they are belonging to a MWAB-gNB or a gNB.

5.49.5.2	UE mobility when the UE is moving together with a MWAB cell

For a UE served by an MWAB-gNB cell, the MWAB-gNB may trigger the handover procedures for UEs in RRC_CONNECTED mode as specified in TS 38.401 [42]. For UEs in RRC_IDLE and RRC_INACTIVE state, existing NAS mobility procedures and Connection Resume procedure are used as specified in TS 23.502 [3].

5.49.5.3	Impact of MWAB mobility

The mobility of MWAB includes the MWAB-UE and the MWAB-gNB aspect.

-	The mobility of the MWAB-UE reuses the exiting UE mobility procedures specified for normal UE.

-	During Mobility, a MWAB-gNB can obtain or apply a new configuration as specified in TS 38.401 [42] from the OAM server of MWAB Broadcasted PLMN and, if any new AMF was provided as part of the new configuration, the MWAB-gNB sets up the N2 connection with the new AMF and it releases the N2 connection with any old AMF removed from the MWAB-gNB configuration.

-	The MWAB-UE may request new or modify existing BH PDU Sessions for the corresponding N2, N3 and Xn interfaces, upon request from the MWAB-gNB as it moves in the network and the MWAB-gNB updates the TNL association as specified in clause 5.21.1 and TS 38.401 [42] to use the new TNL information associated with the new BH PDU Session(s) as needed.

-	SSC mode 3 may be applied to the BH PDU Sessions to minimize traffic interruption when UPF re-allocation for these BH PDU Sessions is required during the MWAB mobility.

-	When it is required to change AMF for the UEs served by the MWAB-gNB upon mobility of a MWAB into a new region, the TAC(s) and Cell ID(s) the MWAB-gNB announces also change. The MWAB-gNB, for UEs in RRC_CONNECTED mode, behaves as specified in TS 38.401 [42].

End of changes