3GPP SA WG2 Meeting #166-AdHoc-e	S2-2500221

# Online, 20 – 24 January, 2025       	                     	                     	 (revison of S2-250xxxx)

Title:	[draft] Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility)

Response to:	LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility) (S2-2500053/R3-247910)

Release:	Rel-19

Work Item:	VMR_Ph2

Source:	SA WG2

To:	RAN WG3

Cc:	RAN WG2

Contact Person:

Name:	Alessio Casati

Tel. Number:	-

E-mail Address:	alessio.casati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR 5902 (S2-2500220) and CR 5934 (S2-2500368)

1. Overall Description:

SA2 thanks RAN3 for their LS (S2-2500053/R3-247910) on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility).

SA2 Answer on the two logical gNB solution:

SA is happy to endorse the two logical gNB solutions as there is no impact to existing behaviour in the CN and systemwide.

SA2, additionally, has discussed the following solutions in the LS and has the SA2 answer below to the question in the action to SA2.

Solution 1: Using single WAB-gNB with single cell and changing TAC

- The WAB-gNB establishes a separate NG-C connection with the new AMF.

- It reports the initial TAC, TAC1, only to the initial AMF and the new TAC, TAC2, only to the new AMF.

- The WAB-gNB cell updates the SI from TAC1 to TAC2.

- For UE in RRC CONNECTED state:

  - When receiving the SI update, the UE performs an MRU, which is forwarded by the WAB-gNB to the initial AMF. The WAB-gNB includes TAC2 as part of the User Location Information to the initial AMF. Based on TAC2 of the received ULI, the initial AMF initiates an AMF reallocation to the new AMF. There is no handover procedure to be performed for the purpose of UE’s AMF change.

- For UE in RRC IDLE/INACTIVE state:

  - When receiving the SI update, the UE performs an MRU, which is forwarded by the WAB-gNB to the new AMF. The new AMF retrieves the UE’s context from the initial AMF.

Solution 2: Using single WAB-gNB with two cells and different TACs

- The WAB-gNB establishes a separate NG-C connection with the new AMF.

- It instantiates a second cell whose SI includes only TAC2, while the first cell’s SI only includes TAC1.

- It reports TAC1 only to the initial AMF and TAC2 only to the new AMF.

- For UE in RRC CONNECTED state:

  - The WAB-gNB initiates an NG handover from the first cell to the second cell for the UE. When the initial AMF receives the HO Required message, it decides to conduct an inter-AMF handover to the new AMF based on the TAC2 included in the Target ID IE of the HO Required message, and forwards the HO signalling to the new AMF. After all UEs have been handed over, the WAB-gNB discontinues operation of the first cell.

- For UE in RRC IDLE/INACTIVE state:

  - When operation of the first cell is discontinued, the UE reselects the second cell and performs an MRU, which is forwarded by the WAB-gNB to the new AMF. The new AMF retrieves the UE’s context from the initial AMF.

SA2 Answer on the single gNB solutions:

For solution1, for the support of a UE in RRC_CONNECTED state, SA2 believes the intended behaviour is not supported by the CN hence it has CN specifications impacts, and SA2 would not endorse proceeding with this approach.

For solution 2, SA2 considers that the solution#2 can address the UE mobility handling as long as there are two cells/TACs during the MWAB node mobility with AMF change. The existing UE mobility procedures (e.g. NGAP procedure or NAS procedure) can be re-used to handle the UEs connected via MWAB node. However, SA2 considers that single gNB connects to multiple AMF sets and reporting different TACs to different AMF sets are rather abnormal setup with additional logics in gNB.

Based on above, SA2 has agreed the attached CR indicating the re-use of existing mobility procedures for UE mobility when it is connected via MWAB node. And SA2 considers nothing prevents RAN3 discussing the one logic node solution if they consider there are other benefits from RAN3 point of view without CN impact.

2. Actions:

To RAN3 group:

ACTION: 	SA2 kindly asks RAN3 to take the above information into account and update corresponding specifications.

3. Date of Next SA2 Meetings:

SA2#167                           17 - 21 February 2025		Athens, GR

SA2#168                           07 - 11 April 2025		Goteborg, SE