3GPP TSG WG2 Meeting #166 AH-E	S2-2500237r06

20 - 24 January, 2025 (e-meeting)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5216 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Any_UE and Group_ID data collection targets |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections and clarifications for Any_UE and Group_ID data collection targets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes: - Added the missing case of subscription to UPF event for Any_UE and Group_ID to the information flows for subscription indirectly via SMF during UPF relocation and PDU Session release. - Added the missing case of subscription to UPF event for Any_UE to the information flows for subscription directly to UPF during N4 session release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and unclear specification for the cases missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.5, 4.15.4.5.6, 4.15.4.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * start of changes * * * *

##### 4.15.4.5.5	Information flow for direct subscription to UPF event exposure service for certain UE using UE's IP address

Figure 4.15.4.5.5-1: Subscription to UPF event exposure service for certain UE(s) directly using UE's IP address

In the case, UPF event consumer (e.g. AF (when in trusted domain) or NEF) is aware of the UE IP address and the UPF Event ID(s) that are to be subscribed meets the criteria for direct subscription to UPF as defined in clause 5.8.2.17 of TS 23.501 [2]), then UPF Event consumer can subscribe to UPF directly for Event Exposure using the following procedure:

1.	The UPF event consumer triggers the UPF discovery to NRF by Nnrf_NFDiscovery_Request providing the UE IP address information and optionally the DNN, S-NSSAI, IP domain if IPv4 address is provided. UE IP address provided to NRF maybe the IP address as seen by the DN (e.g. a Public IP address)

2.	The NRF responds with Nnrf_NFDiscovery_Response message including the information of UPF for the provided UE IP address.

If step 2 does not include any discovered UPF, the information flow described in clause 4.15.4.5.2 can be used instead.

3.	The UPF event consumer sends the Nupf_EventExposure Subscription request for the UPF Event(s) including the following information:

-	Notification Target Address (UPF event consumer address), Notification Correlation Information.

-	UPF Event Id.

-	Event Filter Information, i.e. one or more of the following parameters: S-NSSAI, DNN, either Application Id(s) or Traffic Filtering Information.

-	Target of Event Reporting: UE IP address.

-	Reporting suggestion information.

-	Type of Measurement and Granularity of Measurement.

4.	As per the information provided in the previous step, UPF reports the collected data to the UPF event consumer by invoking Nupf_EventExposure Notify.

NOTE:	In the case there is a NAT, NAPT (Network Address Port Translation) deployed in UPF, the trusted AF can use the Nupf_GetUEPrivateIPaddrAndIdentifiers service before step 1 to get the private IP address of the UE.

# * * * * Next change * * * *

4.15.4.5.6	Information flow for subscription to UPF event exposure service via SMF during UPF relocation and PDU Session release

Figure 4.15.4.5.6-1: Subscription to UPF event exposure service via SMF during UPF relocation and PDU Session release

This information flow is used in the scenario of UPF relocation, e.g., the source UPF is I-UPF or L-PSA which is relocated in SSC mode 1, or the source UPF is PSA UPF which is relocated in SSC mode 3. The information flow also applies to PDU Session release.

When the UPF event consumer targets a specific UE, the procedure is as follows:

1.	The UPF event consumer (e.g. NWDAF) subscribes to the UPF data via SMF by invoking Nsmf_EventExposure_Subscribe service operation, and the Target of Event Reporting indicates a specific UE. The consumer may include a Remaining data reporting indication, which indicates how to handle the data collected for that specific event(s) by the source UPF when the N4 session is released (e.g., due to UPF relocation or PDU session release), i.e. either  to discard the collected data or send the collected data to the consumer.

2.	The SMF selects the source UPF and subscribes to the source UPF by invoking Nupf_EventExposure_Subscribe service operation or via N4 Session Modification, depending on the event. The SMF forwards the Remaining data reporting indication to the source UPF if received in step 1.

3.	When the notification event is triggered, the UPF sends the Nupf_EventExposure_Notify service operation to the UPF event consumer, including the locally collected UPF data.

4.	The SMF determines that the UPF needs to be relocated or PDU Session needs to be released.

5.	In the case of UPF relocation, the SMF initiates a PDU Session update to relocate UPF (for example procedure in clause 4.3.5.3) including N4 session release in source UPF. In the case of PDU Session release, the SMF initiates a PDU Session release procedure, including N4 session release.

6.	When (source) UPF terminates the subscription upon release of the UE related N4 Session (step 5), the (source) UPF may send any collected data not yet sent to the UPF event consumer taking into account local configuration and/or the Remaining data reporting indication (if received) for an event(s) from SMF. For example, for periodic reporting type of the UPF event exposure service when the UPF changes, the (source) UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change. Or for event triggered reporting with Reporting suggestion information, when the UPF changes, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet.

NOTE:	Step 6 takes place as part of step 5. The UPF may send any collected data not yet sent to the UPF event consumer before N4 session released.

7.	In the case of UPF relocation, to continue the subscription of the UPF event consumer, if the UPF related information is matched with the subscription request in step 1, the SMF may select a target UPF and may re-subscribe to the target UPF based on the subscription from UPF event consumer in step 1. Depending on the event, SMF may have sent the subscription to target UPF in step 5 instead.

8.	When the notification event is triggered, the UPF sends the Nupf_EventExposure_Notify service operation to the UPF event consumer, including the locally collected UPF data.

In the case of PDU Session release, steps 7-8 do not apply.

When the UPF event consumer targets Any UE, the same procedure applies with the following changes:

-    In step 1 target is Any UE.

-    Step 2 and step 3 are performed for each of the UEs selected by the SMF to serve the request received in step 1.

-    Step 4 to step 8 may apply to any of the PDU sessions selected by the SMF to serve the request received in step 1. In the case of PDU Session release, steps 7-8 do not apply.

4.15.4.5.7	Information flow for subscription directly to UPF event exposure service during N4 session release

Figure 4.15.4.5.7-1: Subscription directly to UPF event exposure service during N4 session release

When the UPF event consumer targets a specific UE, the procedure is as follows:

1.	If the subscribed UPF events allows to directly subscribe to UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), steps 1-2 as described in Figure 4.15.4.5.5-1 are performed.

2.	The UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to the discovered UPF, and the Target of Event Reporting contains a single UE identified by the UE IP address. The consumer may include a Remaining data reporting indication for an Event ID(s), which indicates how to handle the data collected for that event(s) by the source UPF when the N4 session of the UPF is released (e.g. due to UPF relocation or PDU session release), i.e. either to discard the collected data or send the collected data to the consumer. The consumer may include a Subscription termination reporting indication, requesting the UPF to notify subscription termination to the consumer when N4 session is released.

3.	When the notification event is triggered, the UPF invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

4.	When the N4 session corresponding to the UE is released (e.g., the SMF initiates a PDU Session update to relocate UPF including N4 session release in source UPF, or the SMF initiates a PDU Session release procedure including N4 session release), the UPF determines that the subscription to the UPF event has been terminated.

5.	If step 2 included Subscription termination reporting indication, the UPF sends a last notification to the UPF event consumer by invoking Nupf_EventExposure_Notify service operation, which includes a subscription termination indication to notify that the subscription to the UPF event(s) has been terminated, and may include a cause code specifying the reason for subscription termination, e.g. N4 session release. If step 2 included Remaining data reporting indication and/or based on local configuration, the UPF may report to the consumer any data for the subscribed event, that has been collected but not sent to the consumer yet.

NOTE:	N4 session release may be  part of UPF relocation within the same PDU Session (see clause 4.3.5.3). The UPF event consumer that knows the new UE IP address allocated to the UE could send a new subscription to the target UPF as described in clause 4.15.4.5.5.

NOTE:	Step 5 takes place as part of step 4. The UPF may send any collected data not yet sent to the UPF event consumer before N4 session released.

When the UPF event consumer targets Any UE, the same procedure applies with the following changes:

-    In step 2, target is Any UE. The Subscription termination reporting indication does not apply to this case. If included, UPF ignores it.

-    Steps 3 to 5 applies for each of the N4 Sessions applicable to the subscription request received in step 2.

-    A release of any of the applicable N4 Sessions has no impact on the subscription to the UPF event(s).

# * * * * end of changes * * * *