3GPP TSG SA2 Meeting #166 AH-E	S2-2500238r01

20 - 24 January 2025 (e-meeting)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5217 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Handling of Headers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | clarifications needed in header Handling functionality |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following corrections have been introduced: - Information added by SMF to subscribe to notifications - the event is the enforcement (i.e. occurrence) of a requested action. The information specific for this event report is also listed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of Headers event may be unclear misleading implementations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.12.1, 4.15.12.3, 5.2.26.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.15.12	UPF Event Exposure Direct Reporting

#### 4.15.12.1	General

UPF may expose information directly via a service based interface by means of the UPF Event Exposure service described in clause 5.2.26.2 and as described in clause 5.6.17 of TS 23.501 [2].

Exposure of QoS Monitoring via direct interaction between UPF and Local NEF/AF is described in clause 4.15.12.2 and Notification of Handling of Payload Headers events in clause 4.15.12.3.

UPF event exposure direct reporting as part of UPF data collection procedures is described in clause 4.15.4.5.

#### 4.15.12.2	QoS monitoring exposure

This clause contains the description and the procedure for the exposure of QoS monitoring information via direct interaction between UPF (L-PSA UPF) and Local NEF/AF.

Editor's note:	It is FFS whether Local NEF is introduced or NEF is used instead.

Figure 4.15.12-1: Event exposure using Local NEF

1.	The UPF obtains QoS monitoring information as defined in clauses 5.8.2.18 and 5.45 of TS 23.501 [2].

2.	The UPF sends the notification related with QoS monitoring information over Nupf_EventExposure_Notify service operation according to the configuration from SMF as defined in clause 5.8.2.18 of TS 23.501 [2]. The notification is sent to the Notification Target Address that may correspond (4a) to the AF or (4b) to the Local NEF.

3.	If the Local NEF is used, it reports the QoS monitoring information to the AF by invoking Nnef_EventExposure_Notify service operation.

#### 4.15.12.3	Notification of Handling of Payload Header Events

When AF subscribes to UPF event for Handling of Payload Headers, it provides reporting control information (i.e. Header Handling Reporting Endpoints, etc) as described in clause 5.6.17 of TS 23.501 [2] and PCF includes it in the PCC Rule.

When SMF receives a PCC Rule with Header Handling Control information, if it is including reporting control information for Handling of Payload Header Events, SMF includes it in the instructions to UPF as described in clause 5.8.5.6 of TS 23.501 [2]. The SMF may also decide based on local configuration to subscribe itself to notifications of Handling of Payload Header event and if so, it provides the Header Handling Reporting Endpoint information and Header Handling Reporting for the actions it is interested in. For more information see clause 5.6.17 of TS 23.501 [2].

When a notification reporting trigger is met, UPF notifies the performed action events to the notification endpoint(s) (i.e. AF/NEF and/or SMF) using Nupf_EventExposure_Notify service operation according to the instructions received from SMF (see clause 5.8.2.22 of TS 23.501 [2]). Figure 4.15.12.3-1 shows the notification procedure.

Figure 4.15.12.3-1: Notification of Handling of Payload Header Event

In this procedure, UPF has received instructions for Handling of Payload Headers including reporting control information as described in clause 5.8.5.6 of TS 23.501 [2]. At some moment in time:

1.	The conditions for notification of Handling of Payload Headers event is met.

2.	UPF sends notification(s) to the notification endpoint(s) received in the instructions from SMF by means of Nupf_EventExposure_Notify:

2a.	to SMF, if SMF has subscribed to Handling of Payload Headers and it has included its own Notification Target address;

2b.	to AF, if AF has subscribed to Handling of Payload Headers event and the Notification Target address that UPF receives is the AF;

2c.	to NEF, if AF has subscribed to Handling of Payload Headers event and the Notification Target address that UPF receives is the NEF.

NOTE:	Trusted AFs receive notifications directly from UPF. If AF is untrusted, it receives the notifications via NEF.

3.	If NEF receives a Nupf_EventExposure_Notify from UPF (step 2c), it notifies to AF by means of Nnef_TrafficInfluence_Notify.

# * * * * Next change * * * *

##### 5.2.26.2.1	General

Service description: This service can expose UPF related information to other NFs. There are several operations for this service:

-	Notifying events on the PDU Session to the NFs.

-	Allow consumer NFs to subscribe and unsubscribe for an Event ID on UPF.

The following events can be notified to a NF consumer:

-	QoS Monitoring. This event provides QoS Flow level performance information and can be used for direct event notification of QoS Monitoring reports to a different NF than the SMF as defined in clause 5.8.2.18 of TS 23.501 [2], or it may be for UPF Data collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.5.

Subscription to this event is always indirect via SMF. The subscription specifies the type of measurement that is being requested. A combination of the information listed below can be requested.

UPF and SMF interact using Session Reporting Rules as defined in clause 5.8.5.11 of TS 23.501 [2].

The event notification may contain following information:

-	QoS monitoring result for the QoS monitoring parameter(s) defined in clause 5.45 of TS 23.501 [2], e.g. UL packet delay, DL packet delay, or round trip packet delay.

-	Indication of QoS Flow associated with the default QoS Rule (if requested by SMF, see clause 4.15.4.5.1).

-	User Data Usage Measures or User Data Usage Trends. These events provide information about traffic matching Event Filter Information (see below) for a user's PDU Session or for each PDU Session served by the UPF. It can be used for UPF Data Collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.5.

SMF shall use Service Based Interface subscription service operation to subscribe this UPF event. Other direct consumers can subscribe directly to UPF under the conditions defined in clause 5.8.2.17 of TS 23.501 [2]. The subscription request contains the Type of Measurement that is being requested, the Target of Event Reporting and the Event Filter Information (which describes the target traffic) and the required Granularity of Measurement. The Target of Event Reporting can be either a specific PDU Session or "any UE". The Event Filter Information may contain one or more of the following parameters: DNN, S-NSSAI, either Application ID(s) or Traffic Filtering Information. The Granularity of Measurement can indicate per data flow, per application, or per PDU Session. A combination of the information listed below for User Data Usage Measures can be requested.

For User Data Usage Measures, the event notification may contain following event related information:

-	Volume Measurement: measurements of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) determined for the requested Granularity of Measurement.

-	Throughput Measurement: measurements of data throughput (UL and DL) determined for the requested Granularity of Measurement.

-	Application related Information: URL(s) and/or Domain information (domain name and protocol) detected for the target traffic. This Type of Measurement requires that Application Id(s) or Traffic Filtering Information is provided (i.e. this measurement is not possible to be applied for all traffic handled by the UPF).

For User Data Usage Trends, the event notification may contain following event related information:

-	Throughput Statistic Measurement (average and/or peak throughput) over the measurement period determined for the requested Granularity of Measurement.

For User Data Usage Measures and User Data Usage Trends, if the subscription is targeting any UE, the UPF shall perform the requested measurements for every PDU Session that matches the Event Filter Information and send the information per PDU Session, including the DNN and S-NSSAI and optionally, the sampling ratio achieved by the UPF. When the subscription requests that information is provided per data flow, the notification includes the Packet Filter Set and the Applications Identifier, if available.

NOTE 1:	If the Target of Event Reporting is "any UE", care needs to be taken with regards to load and major signalling impacts and it is expected that the subscription request contains Event Filter Information and appropriate Event Reporting Information (like Sampling ratio and Partitioning criteria, see Table 4.15.1-1) in order to minimize the UPF performance impact of the per PDU Session measurement.

NOTE 2:	The UPF can combine information for different PDU Sessions into one notification message (see TS 29.564 [93]).

-	TSC management information (UMIC, PMIC, NW-TT port number) as defined in clause 5.8.5.14 of TS 23.501 [2].

TSC management information event can be the result of an implicit subscription of the PCF on behalf of the TSNAF/TSCTSF via the Npcf_SMPolicyControl service as described in clause 5.28 of TS 23.501 [2].

-	UE NAT mapping. This event supports the NF consumer to subscribe this UPF event to obtain the NATed UE public IP address and port number with the parameters of UE (private) IP address, IP address of the remote end, DNN and S-NSSAI associated with the AF ID, as well as the IP domain, as defined in clause 5.8.2.17 of TS 23.501 [2].

-	Handling of Payload Headers. This event reports that a Handling of Payload Header requested action has been enforced by UPF (i.e. an occurrence of the requested action) as defined in clause 5.6.17 of TS 23.501 [2].

For this event, UPF reports which action has been enforced and the relevant header information.

Handling of Payload Headers event results out of an implicit subscription received by UPF over N4 triggered by an AF subscription request to PCF via the Npcf_SMPolicyControl service or by the decision in SMF to request notifications of the enforcement of the Handling of Payload Headers instructions provided to UPF.

If the consumer of UPF service is NWDAF and the target of UE is any UE, according to the Analytic ID from consumer, the NWDAF can decide which kind of information should be collect from UPF and event ID to use. Subscription may be performed directly towards UPF or via SMF as described in clause 5.8.2.17 of TS 23.501 [2]. The UPF collects the data according to the event ID and exposes the related information directly regardless of whether the subscription has been relayed by the SMF.

# * * * * End of changes * * * *