3GPP TSG-WG2 Meeting #166-Ad Hoc-e	S2-2500258

# Electronic meeting, 2025-01-20 – 2025-01-24

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5223 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reporting PCF authorization failure to the AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI19 |  |  |  |  |  | Date: |  |  | 2025-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 4.15.6.10 states that the NEF stores the service parameters in the UDR, then “If a group of UEs or any UE is requested, each individual UE authorization is performed at a later stage by PCF”. The authorization performed by the PCF may fail, as such the PCF may report a failure of the individual UE authorization to the AF, this is currently not shows in the procedures. In roaming scenarios, when the AF provides Service Parameters to the VPLMN, the Target UE is the PLMN ID(s) of inbound roamers, then the V-PCF provides the service parameters to the H-PCF of the inbound roamer, then the H-PCF validates the service parameters based on the subscribed values, as described in TS 23.503 clause 6.1.2.2.4: “The H-PCF generates new or updated URSP Rules using the application guidance on URSP Rule determination…. The H-PCF provides RSD values in the URSP Rules that are within the subscribed values defined in the S-NSSAI subscription information defined in Table 6.2.1.3-1”. This is also not shown in the procedures either. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedures for service parameter provisioning are updated to show that the PCF authorization may fail, when the service parameters provided by the AF are not in subscribed values, then the PCF notifies of the failure of delivery of UE Policies to a UE to the AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to inform the AF when the delivery of URSP Rules failed when the PCF determines that service parameters are not subscribed for a SUPI. Misalignment with 23.503, clause 6.1.2.2.4 Distribution of VPLMN specific URSP rules to the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.7.2, 4.15.6.7.3, 4.15.6.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st change **************

##### 4.15.6.7.2	Service specific parameter provisioning by AF to HPLMN

Figure 4.15.6.7.2-1 shows procedure for service specific parameter provisioning. The AF uses Nnef_ServiceParameter service to provide the service specific parameters to the HPLMN and the UE. In the roaming case, PCF is replaced by H-PCF, the AMF interacts with the V-PCF which interacts with H-PCF.

Figure 4.15.6.7.2-1: Service specific information provisioning by AF to HPLMN

0a.	The AMF establishes UE Policy Association as specified in clause 4.16.11.

0b.	PCF requests notifications from the UDR on changes in UE policy information.

1.	To create a new request, the AF invokes an Nnef_ServiceParameter_Create service operation. The request may include subscription information to the report of the outcome of UE Policy delivery.

To update or remove an existing request, the AF invokes an Nnef_ServiceParameter_Update or Nnef_ServiceParameter_Delete service operation together with the corresponding Transaction Reference ID which was provided to the AF in Nnef_ServiceParameter_Create response message.

The content of this service operation (AF request) includes the information described in clause 5.2.6.11.

2.	The AF sends its request to the NEF. The NEF authorizes the AF request. The NEF performs the following mappings:

-	Map the AF-Service-Identifier into DNN and S-NSSAI combination, determined by local configuration.

-	Map the External Application Identifier into the corresponding Application Identifier known in the core network.

2a.	The NEF may invoke Nudm_SDM_Get service operation to perform the following mappings:

-	Map the GPSI in Target UE Identifier into SUPI, according to information received from UDM.

-	Map the External Group Identifier in Target UE Identifier into Internal Group Identifier, according to information received from UDM.

If the AF subscribed to the outcome of UE Policy delivery, the AF indicates where the AF receives the corresponding notifications.

(in the case of Nnef_ServiceParameter_Create): The NEF assigns a Transaction Reference ID to the Nnef_ServiceParameter_Create request.

2b.		(in the case of Nnef_ServiceParameter_Create or Update): The NEF may need to authorize the service specific parameter provisioning request with the UDM by sending a Nudm_ServiceSpecificAuthorisation_Create service operation as defined in clause 4.15.6.7a.

NOTE 2:	The NEF skips the mapping of GPSI or External Group Identifier in step 2a if it needs to authorize the service specific parameter provisioning request with the UDM as the response of the authorization request from UDM includes the SUPI or Internal Group Identifier.

(in the case of Nnef_ServiceParameter_delete): The NEF requests the UDM to remove the authorization of the service specific parameters provisioned by sending a Nudm_ServiceSpecificAuthorisation_Remove service operation.

3.	(in the case of Nnef_ServiceParameter_Create or Update): The NEF stores the AF request information in the UDR as the "Application Data" (Data Subset setting to "Service specific information") together with the assigned Transaction Reference ID.

(in the case of Nnef_ServiceParameter_delete): The NEF deletes the AF request information from the UDR.

4.	The NEF responds to the AF. In the case of Nnef_ServiceParameter_Create response message, the response message includes the assigned Transaction Reference ID.

If the UE is registered to the network and the PCF performs the subscription to notification to the data modified in the UDR by invoking Nudr_DM_Subscribe (AF service parameter provisioning information, SUPI, Data Set setting to "Application Data", Data Subset setting to "Service specific information") at step 0, the following steps are performed:

5.	The PCF(s) receive(s) a Nudr_DM_Notify notification of data change from the UDR. The PCF checks whether application guidance on URSP determination applies for the SUPI as specified in clause 6.1.3.18 of TS 23.503 [20], if this check fails the PCF notifies the failure with an appropriate cause and no delivery of UE Policies to the UE is performed.

NOTE 3:	PCF does not have to subscribe for each UE the application specific information, e.g. if PCF has already received the application specific information for a group of UE or for a DNN by a subscription of other UE. The same application specific information is delivered to every UE in a group or a DNN.

For PIN service, PCF generates the URSP rules with PIN ID in the Traffic Descriptor as specified in Table 6.6.2.1-2 of TS 23.503 [20].

6.	The PCF initiates UE Policy delivery as specified in clause 4.2.4.3.

7.	If the AF subscribed to notifications about the outcome of UE Policies delivery due to Service specific parameter provisioning and the PCF is notified of UE Policy Container from the AMF, the PCF notifies the UE Policy delivery result contained in the UE Policy container as the outcome of the procedure to NEF by sending Npcf_EventExposure_Notify including the SUPI, the list of GPSI(s) if available, and if provided in the step 2a, the Internal-Group-Id.

If the PCF is notified about UE Policy delivery failure from the AMF due to UE not reachable, the PCF may determine to retry step 6 of this procedure when the UE becomes reachable. In such a case, the PCF may report the interim status i.e. UE is temporarily unreachable as the outcome of the procedure to NEF by sending Npcf_EventExposoure_Notify.

If the PCF determines the failure of the UE Policies delivery procedure, the PCF notifies the failure with an appropriate cause such as UE is unreachable, DNN not authorized as the outcome of the procedure to NEF by sending Npcf_EventExposure_Notify.

If the PCF determines that it cannot yet deliver URSP Rules that are based on the service parameters from the AF, then the PCF may report in the interim status that URSP Rules have not yet been delivered by the PCF as the outcome of the procedure to NEF by sending Npcf_EventExposure_Notify.

The content of event reporting in this step is described in clause 6.1.3.18 of TS 23.503 [20].

8.	When the NEF receives Npcf_EventExposure_Notify, the NEF performs information mapping (e.g. AF Transaction Internal ID provided in Notification Correlation ID to AF Transaction ID, SUPI to GPSI, Internal-Group-Id to External-Group-Id, etc.) and triggers the appropriate Nnef_ServiceParameter_Notify message.

##### 4.15.6.7.3	Service specific parameter provisioning by AF to VPLMN

Figure 4.15.6.7.3-1 shows procedure for service specific parameter provisioning by the AF to VPLMN.

Figure 4.15.6.7.3-1: Service specific information provisioning by AF to VPLMN

0a.	Same as in step 0a of Figure 4.15.6.7.2-1.

0b-0c.	The V-PCF may request to V-UDR on changes in UE policy information and H-PCF may subscribe to H-UDR.

1-2.	Steps 1-2 of Figure 4.15.6.7.2-1 apply with the following differences:

-	The AF and NEF belong to the VPLMN. The AF may belong to third party with agreement with VPLMN.

-	When the AF provides application guidance on URSP Rule determination to the VPLMN, it will target "PLMN ID(s) of inbound roamers". The NEF in the VPLMN rejects any request for a GPSI or an External-Group-ID of a different PLMN.

3-5.	Steps 3-5 of Figure 4.15.6.7.2-1 apply with the following differences:

-	AF, NEF and UDR belong to VPLMN. The AF may belong to third party with agreement with VPLMN.

-	The UDR in the VPLMN notifies the V-PCF(s) that have subscribed to the reception of application guidance on URSP determination.

-	In step 5, the V-PCF receives updates on application guidance on URSP determination for the PLMN ID of a SUPI that has a UE Policy Association established. The PLMN ID of the SUPI is included in the target "PLMN ID(s) of inbound roamers" in step 2. In this case, the V-PCF checks whether application guidance on URSP determination applies for the SUPI as specified in clause 6.1.2.2.4 of TS 23.503 [20], if this check fails, step 7 and step 8 follow, the V-PCF notifies the failure with an appropriate cause such as e.g. DNN not authorized.

5a.	The V-PCF sends the Service Parameters including the mapped HPLMN S-NSSAI values to the H-PCF and subscribes to the result of the delivery of UE Policies if the delivery result was requested by the AF, using the event reporting on "Notification on outcome of UE Policies delivery" described in clause 6.1.3.18 of TS 23.503 [20].

NOTE:	The AMF determines whether LBO is allowed and performs SMF selection to select the SMF in VPLMN for LBO case as described in clause 6.3.2 of TS 23.501 [2].

The H-PCF requests V-PCF to notify the result of UE policy delivery to the UE.

6.	The H-PCF generates new or updated URSP Rules considering the Service Parameters received from the V-PCF in step 5a as specified in clause 6.1.2.2.4 of TS 23.503 [20].

7-8.	Steps 7-8 of Figure 4.15.6.7.2-1 apply with the following differences:

-	Notification is sent from V-PCF to the AF belonging to the VPLMN or third party with agreement with VPLMN.

#### 4.15.6.10	Application guidance for URSP determination

This clause describes the procedures to allow an AF to provide guidance for URSP determination to 5G system via NEF. The AF may belong to the operator or to an external party. The PCF may be in the Home PLMN, as it is the PCF that determines the URSP for the UE, or in the VPLMN and then the Application guidance for URSP determination is provided to the PCF in the HPLMN via the PCF of the VPLMN. The PCF in the VPLMN translates the Service Parameters values provided by the AF for inbound roamer to values applicable to the HPLMN, e.g. S-NSSAI as described in TS 23.503 [20].

NOTE 1:	The operator can negotiate with external party (typically a Corporate represented by an AF) dedicated DNN(s) and/or S-NSSAI(s) for the traffic of UE(s) of this external party. UE(s) of the external party can be identified by a group identifier.

The guidance for URSP determination may be used to provide 5GC with guidance for the URSPs depending on the UE location. This is further described in TS 23.548 [74].

For providing guidance for URSP determination, the procedure defined in clause 4.15.6.7 is performed with the following considerations:

1)	Service Description indicates an AF Identifier.

2)	Service Parameters.

Information on the AF guidance for URSP determination which consists of a list of URSP rules that associate an application traffic descriptor with requested features for the candidate PDU sessions the application traffic may use:

-	An application traffic descriptor, whose definition corresponds to that of the URSP Traffic Descriptors (as defined for the URSP rule in TS 23.503 [20] Table 6.6.2.1-2). When AF provides application guidance for URSP determination for PIN, the application traffic descriptor shall include PIN ID.

-	one or more sets of Route selection parameters, each parameter may correspond to:

-	(DNN, S-NSSAI). This may be provided by the AF or determined by the NEF based on the AF Identifier when it is not provided by the AF and the AF provides only one instance of AF guidance for URSP determination. In the case of AF guidance for URSP determination for PIN, this shall be provided by the AF.

-	Requested PDU session type.

-	a default Route selection precedence value to be used for the application traffic when Route selection precedence with a corresponding spatial validity condition is not provided.

-	Route selection precedence with a corresponding spatial validity condition that indicates where the Route selection parameters apply. This may correspond to a geographical area (e.g. a civic address or shapes).

NOTE 2:	The different sets of Route selection parameters indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic descriptor. Each set is meant to apply for a specific (set of) spatial validity condition. Each set is associated with a Route selection precedence to cope with the case where multiple spatial validity conditions overlap.

-	VPLMN ID(s) that indicates the PLMN(s) where the AF guidance on URSP determination and all its RSD(s), applies.

If the AF provides a geographical area as spatial validity condition, it is up to the NEF to transform this information into 3GPP identifiers (e.g. TAI(s)).

An AF sets the Requested PDU session type if the AF requests to change the PDU session type of the URSP rules.

NEF may, based on local configuration, complement missing service parameters. Additionally, based on operator's local policy, NEF may request UDM for service specific authorization for the service parameters for an individual UE (e.g. to authorize the Corporate or MTC provider represented by the AF and the requested DNN, S-NSSAI for the related UE) before storing the service parameters into the UDR. If the request is targeting a group of UEs, NEF may also request UDM for service specific authorization for the group related data (see table 4.15.6.3b-1), i.e. the DNN, S-NSSAI associated to the group. If the request is targeting any UE (all UEs), NEF authorizes the request based on local policy (e.g. based on AF Id) without requesting for any service specific authorization from UDM. NEF requests UDM for service specific authorization for the service parameters provisioned via the Nudm_ServiceSpecificAuthorisation_Create service operation as defined in clause 4.15.6.7a.

If a group of UEs or any UE, possibility of the PLMN when roaming in other PLMN, is requested, each individual UE authorization is performed at a later stage by PCF, the PCF may report a failure of the individual UE authorization to the AF as described in clause 4.15.6.7.2 and in clause 4.15.6.7.3.

NOTE 3:	The operator needs to ensure the consistency between the group related data and the UE group members subscription data, i.e. if a group is authorized for a given DNN/S-NSSAI as defined in the group related data, it needs to be ensured that all UE members of the group are provisioned with such DNN/S-NSSAI, since no individual UE check is required to be done by NEF against UDM.

NOTE 4:	AF guidance for application traffic is not related with 5G VN group.

3)	The Target UE identifier(s) that may be a specific UE, identified by a GPSI, or a group of UE(s), identified by an External-Group-ID, or any UE of the PLMN of the NEF, or the PLMN ID(s) of inbound roamers that the AF request may be associated with.

The information on the AF guidance for URSP determination provided by the AF may be associated to:

a)	UEs of the PLMN (of the NEF) when roaming in other PLMNs. In this case, the AF guidance for URSP determination targets to a specific UE, a group of UEs or any UE of the PLMN. In this case, the AF guidance for URSP determination associated to a specific UE, a group of UEs or any UE of the PLMN shall be also associated with the corresponding VPLMN(s) where the AF guidance for URSP determination shall be applied if the UE roams to that VPLMN(s). The list of VPLMN ID(s) is included in the Service Parameters.

b)	An inbound roamer from one or more PLMN(s). In this case, the AF targets the AF guidance for URSP determination only with the inbound roamers of corresponding PLMN(s). The PLMN ID is included in the Service Parameters.

NOTE 5:	Wildcarding of "PLMN ID of inbound roamers" will be handled by stage 3.

4)	Subscription to events.

The AF may subscribe to notifications about the outcome of the UE Policies delivery due to application guidance for URSP determination.

The usage of the AF guidance for application traffic is described in clause 6.6 of TS 23.548 [74].

************** End of changes **************