3GPP TSG-SA WG2 Meeting #166E	S2-2500290r03

# EMeeting, 20 - 24 January, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1545 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | De-multiplexing ADC in case same stream ID used by both UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In S2-2500289, the case of same stream ID used for application data channels may be occurred. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new de-multiplex criteria based on same stream ID used; Add new sub-clause to show the procedure accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multiplexing application data channels with same stream ID may be error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.10.1, AC.7.10.3.1, AC.7.10.4.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

## AC.7.10	Multiplexing multiple DC streams over single SCTP connection

### AC.7.10.1	General

When multiple data channels are used in an IMS session, the UE and IMS network may support SDP negotiation to multiplex multiple data channel streams into a single data channel SDP media description m line and transport streams of multiple applications in the same SCTP connection, to save the number of m lines used by the UE and to improve the efficient usage of media resources.

The UE and IMS network also need to support data channel de-multiplexing when supporting data channel multiplexing. If the UE receives streams targeting to different applications over a single SCTP connection, it identifies the applications and delivers the data of the streams to corresponding applications. If the IMS network receives multiplexed streams which have different remote endpoints, e.g. a local bootstrap data channel and a remote bootstrap data channel of a UE, the MF terminates the multiplexing and sends different streams to corresponding endpoints based on instructions of DCSF.

The following multiplexing scenarios are supported in this Release:

-	Multiplexing local bootstrap data channel and remote bootstrap data channel between the UE and its home IMS network using a single SDP m line media description if both support data channel multiplexing capability;

-	Multiplexing different application data channels for applications with compatible QoS requirements between the UE and its IMS network using a single SDP m line media description if both support multiplexing capability;

-	Multiplexing different application data channels for applications with compatible QoS requirements between the originating IMS network and terminating IMS network using a single SDP m line media description if both support DC multiplexing capability.

Multiplexing the data channels using the same stream ID are not supported.

The following de-multiplexing scenarios are supported in this Release:

-	The supporting IMS network de-multiplexes local bootstrap data channel and remote bootstrap data channel from a SDP media description towards a UE that does not support multiplexing;

-	The supporting IMS network de-multiplexes application data channels from an SDP media description, if the data channels have different endpoints e.g. a P2P application and a P2A application or towards a UE that does not support multiplexing;

-	The supporting IMS network de-multiplexes application data channels associated with stream IDs from an SDP media description, if other application data channels associated with the same stream IDs are already multiplexed in the SDP media description originated by the served UE, to avoid multiplexing application data channels with same stream ID;

-	The supporting originating IMS network de-multiplex data channels towards remote network if the remote network or remote UE does not support DC multiplexing.

If the UE decides to use data channel multiplexing, when the UE generates SDP offer in INVITE or re-INVITE request, the DC stream IDs associated with the data channels to be multiplexed and the binding information combined with the data channels are included in the SDP offer for a single m line.

NOTE:	How the application id and the DC stream IDs are included in the SDP is specified in SA WG4.

The IMS AS notifies the event of data channel multiplexing along with the data channel media information and binding information to the DCSF if data channel multiplexing is allowed for the subscriber.

The DCSF determines whether and how the data channels are multiplexed. When the DCSF determines that some data channels are allowed to be multiplexed, it instructs the IMS AS to multiplex the specific data channel streams to the same m line on the MF. The IMS AS further instructs MF to reserve a single media termination for the streams to be multiplexed.

If the DCSF determines that a data channel needs to be de-multiplexed from a SCTP connection, it instructs the IMS AS to de-multiplex the specific data channel stream on the MF. Based on the instruction of the DCSF, the IMS AS further indicates the MF to reserve separate media termination for the de-multiplexed stream in stead of using the same media termination with other multiplexed streams.

If the IMS AS is instructed by DCSF to use data channel multiplexing, when IMS AS generates SDP offer in INVITE or re-INVITE request to remote network, the DC stream IDs associated with the data channels to be multiplexed and the binding information combined with the data channels are included in the SDP offer for a single m line.

## ---Start of the 2nd Change---

#### AC.7.10.3.1	Determination of multiplexing and de-multiplexing

When the IMS AS receives the INVITE request with data channel multiplexing, if data channel multiplexing is supported, it reports the event to the DCSF for data channel management. The DCSF determines whether the data channel multiplexing is allowed and whether serving network needs to de-multiplex the data channel traffic based on subscription data, binding information, the endpoints of data channels, whether same stream ID is already used for other multiplexed application data channels, and capability of terminating network.

## ---Start of the 3rd Change (All new text) ---

##### AC.7.10.4.2.x	Application data channel de-multiplexing to avoid multiplexing application data channels with same stream ID

Figure AC.7.10.4.2.x-1 shows the procedure of multiplexing application data channels when both originating and terminating networks support data channel multiplexing.

In this example procedure, the UE initiates an IMS session with audio and DC media components. The UE multiplexes application data channels for app#2, #3 and #4. Application data channel for app#4 is originated by the UE with endpoint of remote UE, application data channel for app#3 is originated by the UE with endpoint of DC AS, and application data channel for app#2 is originated by the remote UE. The application data channel for app#4 uses the same stream ID of an P2A application data channel for app#1 originated by the remote UE, which is multiplexed with the application data channel for app#2 in the remote IMS network. In order to avoid multiplexing application data channels with same stream ID, the remote IMS network eventually de-multiplexes the application data channel for app#4. Since the originating network and terminating network both support data channel multiplexing, application data channels are multiplexed between the originating network and terminating network.

Figure AC.7.10.4.2.x-1: ADC de-multiplexing when same stream ID used by both UEs

0.	The UE#1 has multiplexed its application data channels for app#2 and #3 associated with stream ID-2 and stream ID-3 respectively, the endpoint of application data channel for app#3 is local DC AS#1 and the endpoint of application data channel for app#2 is UE#2. The UE#2 has multiplexed its application data channels for app#1 and #2 associated with stream ID-1 and stream ID-2 respectively, the endpoint of application data channel for app#1 is local DC AS#2 and the endpoint of application data channel for app#2 is UE#1.

1.	When UE#1 wants to establish a new application data channel for app#4 with UE#2 and multiplex it with data channels for app#2 and #3, the UE#1 includes multiplexed SDP media description when sending SIP reINVITE request. The UE#1 associates stream ID-1 with the data channel for app#4. The media feature tag in the Contact header field indicating its capability of supporting data channel multiplexing is included in SIP reINVITE request.

2.	The IMS AS#1 reports event to DCSF#1.

3.	The DCSF#1 determines that data channel for app#3 needs to be de-multiplexed because its target endpoint is local DC AS#1. The DCSF also determine that the multiplexing of data channels for app#2 and #4 is kept unchanged towards their target network as described in clause AC.7.10.3.

4.	The DCSF#1 generates data channel media information for local side and remote side.

5.	The DCSF#1 instructs the IMS AS#1 to reserve resources on MF.

6.	The IMS AS#1 instructs MF to reserve media resources for multiplexed data channel streams and other media resources based on instructions from DCSF#1.

7-8. The session continues after the media resource is reserved successfully.

9-10. The IMS AS#1 generates SIP reINVITE request to remote network. The SIP reINVITE request includes capability of supporting data channel multiplexing and the SDP offer for multiplexed data channels for app#2 and #4.

11.	The remote network supports data channel multiplexing. The IMS AS#2 reports event to DCSF#2.

12.	The DCSF#2 finds out that data channel for app#2 is already multiplexed with data channel for app#1 by UE#2. The DCSF#2 determines that data channel for app#4 needs to be de-multiplexed as described in clause AC.7.10.3 because it uses the same stream ID-1 as data channel for app#1.

NOTE:	Other de-multiplexing ways can be applied to avoid multiplexing application data channels with same stream ID, e.g., multiplexing data channels for app#2 and #4 as well as de-multiplexing data channel for app#1. It is preferable to not impact already multiplexed data channels by serving UE where possible.

13.	The DCSF#2 generates data channel media information for local side and remote side.

14.	The DCSF#2 instructs the IMS AS#2 to de-multiplex the data channel for app#4 and multiplex data channels for app#1 and #2.

15.	The IMS AS#2 instructs MF to reserve media resources for multiplexed data channel streams and other media resources based on instructions from DCSF#2.

16-17. The session continues after the media resource is reserved successfully.

18-19. The IMS AS#2 generates SIP reINVITE request to UE#2 with an SDP offer for multiplexed data channels for app#1 and #2 (one m line for data channels for app#1 and #2) as well as data channel for app#4 (a new m line for data channel for app#4) based on the instruction from DCSF#2.

20-21. The UE#2 replies 18X response with an SDP answer for multiplexed data channels for app#1 and #2 and de-multiplexed date channel for app#4.

22-23. The IMS AS#2 replies 18X response with capability indication of supporting data channel multiplexing and an SDP answer for multiplexed data channels for app#2 and #4 to remote network.

24.	The IMS AS#1 replies 18X response with the SDP answer for multiplexed data channels for app#2, #3 and #4 to UE#1.

25-26.	The IMS AS#1 reports event to DCSF#1.

27.	The subsequent procedure continues. The streams of data channels for app#1 and #2 are transported in the same SCTP connection and the streams of data channel for app#4 are transported in another SCTP connection between UE#2 and local MF. The streams of data channels for app#2, #3 and #4 are transported in the same SCTP connection between UE#1 and local MF. The streams of data channels for app#2 and #4 are transported in the same SCTP connection between MFs of local and remote network.

## ---End of the Change---