3GPP TSG-WG SA2 Meeting #166 Ad Hoc-e	   S2-2500333r15

20th – 24th January, 2025, Online

Source:	China Mobile

Title:	Removing one EN related to BS reader selection in KI#1

Document for:	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes methods for BS reader selection.

# Discussion

These conclusions related to AIOT RAN architecture are documented In clause 6.4.1 in RAN TR38.769 v2.0.1:

- Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

- For topology 1, the XXAP is terminated at an A-IoT RAN node.

- In Topology 1, an A-IoT RAN node may serve one or more readers.

ATOT RAN node and BS reader terms are suggested to keep consistent with AIOT RAN.  BS reader is a list of the common reader functions. It is equal to the reader in AIOT RAN. In SA2, we suggest using the BS reader in T1 to distinguish it from the UE reader in T2. And NGAP is terminated at an AIOT RAN node, not BS readers. At the same time, an AIOT RAN Node may service one or more BS readers.

Proposal 1:  Adding one note: For topology 1, the NGAP is terminated at an AIOT RAN node, and an AIOT RAN Node may service one or more BS readers.

Whether and how to expose BS reader info to the AIOTF is not decided in AIOT RAN.  If the BS reader info is not exposed to AIOTF, in that case, RAN has to expose some selection information so the A-IoT RAN node can select readers (such as something like an area to use for inventory and the AIOT RAN knows readers belong to that area).

AIOTF always selects an A-IoT RAN node. Furthermore, the AIOTF provides to AIoT RAN, for a given request, ether:

- a list of BS Readers (one or more), or

- some selection information so the A-IoT RAN node can select readers (such as something like an area to use for inventory and the AIOT RAN knows readers belong to that area).

For example,   one Cosco store is covered by an AIOT RAN with 10 BS readers.  It is served by BS reader ID 1 in location ID (cell 1), BS reader ID 2-3 in location ID (cell 2), BS reader ID 4-10 in location ID（cell 3）.

AF sends the AIOT service request with location info (Cell 2), then AIOTF can obtain AIOT RAN Node ID and smaller location Cell 2 info based on location match :

If RAN reports both BS reader ID and location (cell ID) mapping information to AIOT RAN,  AIOTF will choose BS reader 2 and BS reader 3 to perform the service.

If RAN reports only location ID (cell ID) information to AIOT RAN,  AIOT can’t choose BS reader ID directly.  It will put location ID (cell ID 2) in the request to AIOT RAN, and then AIOT RAN will choose BS reader 2 and BS reader 3 based on location ID (cell ID2) to perform the service.

Please keep in mind that if AIOTF doesn’t provide BS Reader ID information/location information to the AIOT RAN Node, It means the AIOT RAN node has to do the paging service in the whole area served by this AIOT RAN Node. If AIOTF does provide BS Reader ID or location information to RAN Node.   RAN Node just needs to do the paging service in the area served by the BS Reader ID or location ID (RAN will obtain reader ID based on location ID in RAN itself) , not the whole area served by AIOT RAN,  to improve the paging efficiency.

AIOT RAN Node has the BS reader ID info and its serving location info.  AIOT RAN can report it to AMF via NGAP setup (or AMF obtains it by OAM configuration) . then AMF can forward this info to AIOTF by subscriber/notification mechanism.

Proposal 2:   For topology 1, the AIOTF selects the AIOT RAN nodes, and the BS readers list or locations list information.

NOTE X:     If AIOTF obtains BS readers' information served by the AIoT RAN node and their serving location information from AIOT RAN,  AIOTF can inform AIOT RAN of BS readers directly. If AIOTF obtains only serving location information from AIOT RAN, AIOTF needs to inform AIOT RAN of location information to assist BS reader selection in AIOT RAN.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

## 8.1	Interim Conclusion on Key Issue #1

### 8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this clause.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor's note:	Final conclusions are assumed to be taken in coordination with RAN WGs.

1.	A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIoTF registers itself in the NRF with its NF profile.

Editor's note:	The details of the NF profile are FFS.

b.	For topology 1, the AIOTF selects the AIOT RAN node(s) and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g., BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g., their corresponding service area when the BS Readers are integrated in the network.

NOTE X:	Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA2 may include this feature in SA2 specifications.

NOTE X:	If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers.

NOTE X: The BS reader’s service area needs to coordinate with RAN WG(s).

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

e.	The AIOTF may provide the following assistance information to RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	Further information on AIoT service type may be determined later in cooperation with RAN WGs.

NOTE 2:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 3:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor's note:	How the aggregation can be done is FFS.

f.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves.

Editor's note:	Whether and how the AIoT Device ID privacy protection and ID authentication is done will be concluded by SA WG3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE 4:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIoTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Editor's note:	It is FFS whether to support any other messages besides Inventory Response, Command (e.g. Read and Write) Request and Response over AIoT Device NAS layer.

5.	The network may manage the AIOT device related information (e.g. device context information), includes the AIOT device permanent ID, and optionally the last known reader information.

Editor’s note:	How this AIOT device related information is used is FFS.

Editor’s note:	Where to store the AIOT device related information is FFS.

# * * * * End of changes * * * *