3GPP TSG-SA WG2 Meeting #166-Ad Hoc-e	S2-2500358

# E-meeting,  20 – 24 January 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5239 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating EIF service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following editor’s note is in the specification: Editor's note: More detailed input/output parameters of the service operations are FFS. This CR resolve the editor’s note for the following reasons: - Clause 5.2.28.2.2 refers to the Target of Event Reporting defined in 5.2.28.2.1 which is missing - Clause 5.2.28.2.4 specified the energy consumption as the required output of notification which is not aligned with the specification of the notify service operation of other NFs and it is not generic to consider other events (such as renewable energy) in the future - As defined in clause T.2 of TS 23.501, the energy consumption is composed of two main parts, i.e., energy consumption reported by gNB(s) and by UPF(s), which are not distinguishable if the consumer of EIF services is interested in. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Target of Event Reporting is clarified in clause 5.2.28.2.1 - Outputs of the EIF Notify service operation is generalized and aligned with the definition of the Notify service of other NFs - A new optional input parameter is introduced for distinguishing energy consumption reported by the gNB and by the UPF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent and incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.28.2.1, 5.2.28.2.2, 5.2.28.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

5.2.28.2	Neif_EventExposure service

5.2.28.2.1	General

Service description: The service provides the energy consumption information to the consumer NFs. There are several operations for this service:

-	Notifying the energy consumption information to other NFs.

-	Allow consumer NFs to subscribe and unsubscribe for energy consumption information.

The following events can be subscribed by a NF consumer as Target of Event Reporting (Event ID is defined in clause 4.15.1):

-	Energy Consumption: the event notification may contain energy consumption information for the required granularity as described in clause 5.51 of TS 23.501 [2]

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for EIF exposure.

Table 5.2.28.2.1-1: Example of Event Filters for EIF exposure events

| Event ID for EIF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Energy Consumption | <Parameter Type = UE level, Value = SUPI/GPSI> <Parameter Type = PDU session level, Value = SUPI/GPSI and DNN/S-NSSAI > <Parameter Type = QoS flow level, value = SUPI/GPSI, DNN/S-NSSAI and Flow description(s)> |

The following service operations are defined for the Neif_EventExposure service:

-	Neif_EventExposure_Subscribe.

-	Neif_EventExposure_Unsubscribe.

-	Neif_EventExposure_Notify.

********** Next Change **********

5.2.28.2.2	Neif_EventExposure_Subscribe service operation

Service operation name: Neif_EventExposure _Subscribe

Description: The consumer subscribes to receive the energy consumption information from the network, or the subscription is updated if the same subscription is already existing in EIF.

Inputs, Required: NF ID, Target of Event Reporting as defined in clause 5.2.28.2.1, Event ID defined in clause 5.2.28.2.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Inputs, Optional: Event Filter (s) associated with each Event ID; Event Filter(s) are defined in clause 5.2.28.2.1, time period, reporting period and reporting frequency as described in clause 5.51 of TS 23.501 [2], segment(s) (total, gNB, and/or UPF).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding energy related information report is included, if available.

NOTE:	If segment parameter is not present, the total energy consumption will be returned.

********** Next Change **********

5.2.28.2.4	Neif_EventExposure_Notify service operation

Service operation name: Neif_EventExposure_Notify

Description: EIF reports the energy consumption information to the consumer that has previously subscribed.

Inputs, Required: Event ID, Notification Correlation Information, time stamp.

Inputs, Optional: Event specific parameter list as described in clause 5.2.28.2.1.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

********** End of Changes **********