SA WG2 Meeting #166-Ad-Hoc-e	S2-2500390

20 - 24 January, 2025, Electronic meeting

Title:	[Draft] Reply LS on relay discovery announcement

Response to:	LS to SA2 on relay discovery announcement (S2-2500045/R2-2411184)

Release:	Rel-19

Work Item:	NR_SL_relay_multihop-Core, 5G_ProSe_Ph3

Source:	SA2

To:	RAN2

Cc:	-

Contact person:	LaeYoung Kim

laeyoung.kim@lge.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.304 CR#0532

Overall description

SA2 thanks RAN2 for the LS on relay discovery announcement.

Regarding the following question about Model A discovery from RAN2, SA2 would like to provide the answer as below.

- Question from RAN2: Whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

- Answer from SA2: Yes.

To clarify behavior of Intermediate UE-to-Network Relay related to Model A discovery including the above aspect, SA2 agreed the attached CR.

SA2 discussed behavior of Intermediate UE-to-Network Relay related to Model B discovery and found no issue based on e.g. the following operation:

-	if Layer-2 Intermediate UE-to Network Relay without connection to any UE-to-Network Relay receives multiple discovery response messages where itself is included in the path information, the Layer-2 Intermediate UE-to-Network Relay needs to select only one discovery response message (i.e. only one path which means only one UE-to-Network Relay) and forward it.

Actions:

To RAN2:

SA2 kindly asks RAN2 to take the above information into account in their future work.

Date of Next TSG-SA2 Meetings:

SA WG2#167	17 – 21 February 2025	Athens, Greece

SA WG2#168	07 – 11, April 2025	Goteborg, Sweden