3GPP TSG-WG2 Meeting #166-Ad Hoc-e	S2-2500398r09

# Electronic meeting, 20th – 24th January, 2025											(revision of TBD)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1338 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving ENs for VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Lenovo, InterDigital, OPPO, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Resolve ENs |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.2 resolve EN: Editor’s Note：Whether NWDAF as VFL server registers to NRF with VFL capability information is needed is FFS. by removing it, since in clause 6.2H.2.1.1 specifies that an NWDAF can detect a VFL server NWDAF Also adding that the existing parameters in NRF Serving Area information and/or NF set ID(s) of the data source(s) can be used also for VFL (not only for HFL). Having these parameters makes it possible for a Server to know the VFL training will cover all needed input data. If these are used, Feature ID can be a complement to further specify the input data, so both are optional and can be used at the same time. Clause 5.4 In this release no split of a VFL participant is done between training and inference, so follwing EN is removed, some text is added clarifying this point. Editor's note: For an NWDAF impacts of the split into AnLF and MTLF are FFS. Possibility to change samples is introduced in the procedures, so follwing EN is resolved Editor’s Note: Whether and how change of samples during a VFL training is supported is FFS. Resolving where to place the trusted AF as server by placing it in the same as NWDAF as server, and by this resolving following ENs Editor's note: This clause needs to be update to include the description of the clauses for NWDAF or AF as the server case. Editor´s note: It is FFS, whether to keep the case that trusted AF as VFL server along with the clause of NWDAF as server or with the clause of untrusted AF as server. Editor's note: Whether the case of trusted AF as VFL Server is included in the clause for NWDAF as VFL server or in the clause for untrusted AF as VFL server is FFS. Clause 6.2H.2.3.1 Training phase Follwing ENs are resolved and described in the procedures Editor's Note: It is FFS whether sample/feature information is required to be provided or updated in each training. by showing in procedure that Feature ID is only sent in first message at start. Delta samples may be sent in any step. Editor's Note: Whether and how to include interoperability information in the VFL training procedure is FFS. by only sending it at start of training Editor's Note: Whether and how to transfer the confirmation in VFL Preparation Phase at the beginning of VFL Training Phase is FFS. by showing no confirmation is needed in the end of the preparation phase Editor's Note: Whether the parameters negotiated in the preparation phase are provided at the end of the preparation phase or at the start of the training is FFS. this above is needed to be able to have the VFL correlation ID coupled to the selected parameters. The triggers for training are done in same way as for HFL, also discussed in the Discussion paper S2-2411640, it can also be internal or a request for inferencel. This solves the following EN: Editor's Note: Whether and how to define the trigger of VFL training is FFS. Adding same possibility as in HFL for client to leave training and also make it possible for server to skip clients during training, resolving part of the details of the services in the procedure. The EN is changed, since the later part “and whether VFL Training Start Flag is needed are FFS” is not resolved The same messages are used for multiple training rounds. This can lead to issues if messages are lost or repeated. For HFL, those issues were resolved via a training round ID. The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS. As a training round number initially set to 1 is suggested, an additional flag is not required.w Editor's Note: The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS. The follwing EN is removed since the EN in clause 6.2H.3 is the same EN Editor's note: Additional Parameters to be provided in the request are FFS. As stated as main motivation for VFL in the study document “A more flexible technique: In VFL vendor specific local models and features can be deployed in each participant, so that it is possible that each participant selects or configures the local model to be used, as such vendor or operator specific local models and features, including not standardized features, are simpler to implement comparing with HFL.” The following ENs are removed since in VFL the features are believed to be proprietary and therefore the local model shall not be shared from any entity specified in SA2. If anything is needed to be shared between participants it is e.g. the cut-layer in a NN. Editor's Note: It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process. Editor’s Note: Whether VFL server and VFL clients share feature information is FFS. The follwing EN is removed since the target is to be on par with HFL, and then the metrics is enough Editor’s Note: The content of the VFL status report is FFS. Since the Intermediate results/Intermediate training information are not specified the convergence reports are mentioned as an example that can be incorporated into these messages. Editor’s Note: Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS. It is deemed that same metrics can be used as for HFL, since VFL is just another way to achieve the same output, so following EN is removed. Editor's Note: Whether the ML model metric (e.g. ML model accuracy) defined for HFL can be re-applied to VFL is FFS. Since we use unsubscribe for termination following EN is removed: Editor’s Note: Whether VFL Training termination Flag in the termination message is required is determined after settling down the service operation It is understood that the server knows the impact each client has on the output in VFL training, and can judge what clients will contribute to the inference output. So therefore the following EN is removed: Editor's note: It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying. In cluase 6.2H.2.4.2 the follwing EN is solved in clause 6.2H.2.4.1, by letting the AF ID to be known Editor's note: When the AnLF determine the VFL server AF is FFS. For example, during the training phase. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Result in confusing and inconsistency regading Vertical Federated Learning; |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.4, 6.2H.1, 6.2H.2.3.1, 6.2H.2.3.2,6.2H.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

* * *Next Change * * *

5.4	Vertical Federated Learning (VFL)

Vertical Federated learning is a machine learning technique working without exchanging/sharing of local data set, while maintaining some level of coordination amongst VFL participants, when training and inference are performed on local ML Models, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). Vertical Federated Learning may involve multiple NWDAFs and AFs.

For Vertical Federated Learning, there may be one NWDAF or one AF acting as a VFL server and one or multiple NWDAF(s) and/or one or multiple AF(s) acting as VFL Client(s). Vertical Federated Learning is available among NWDAFs or between NWDAF(s) and AF(s) within a single PLMN or between an AF and NWDAF(s) in a single PLMN.

The main functionalities of VFL server and VFL client include:

VFL server:

-	An NWDAF or trusted AF acting as VFL server discovers and selects VFL client(s) (NWDAF(s) and/or AF(s)) to participate in a VFL procedure.

NOTE:	When an untrusted AF is acting as VFL server, NEF discovers and selects candidate VFL client NWDAFs, then the AF determines final set of VFL clients.

-	It requests VFL clients to do local ML model training for an Analytic ID, it assigns VFL correlation ID, and it requests to report intermediate results.

-	It optionally locally trains ML Model with the available local data set.

-	It combines intermediate results from VFL client(s) and VFL server and computes intermediate training results (e.g. gradient information, loss information) for updating its own local ML Model and the ML Models of VFL clients during the VFL training process and sends the intermediate training results towards VFL clients involved in the joint VFL training process. VFL server may send and receive separate message for each client.

NOTE:	NEF forwards the message from/to NWDAF to/from untrusted AF.

-	It determines to terminate the VFL training process.

-	It stores VFL correlation ID and locally trained ML Model after VFL training process.

-	It initiates the VFL inference process using VFL correlation ID.

-	It combines local inference result from VFL clients and generates the final VFL inference result.

-	It may send the final VFL inference result to the consumer.

-	It supports to monitor the accuracy of the VFL model.

VFL client:

-	It locally trains ML Model with the available local data set, which includes the data that may not be allowed to be shared with other VFL clients or VFL server due to e.g. data privacy, data security, data access rights.

-	It computes the intermediate results for their local ML Models involved in the VFL training and provide reports with the intermediate results to the AF or NWDAF acting as VFL server.

-	It stores VFL correlation ID and locally trained ML model after VFL training process.

-	It performs inference based on the local model and local data and provides inference results to VFL server.

Editor's note:	Details regarding Sample alignment and features alignment functionality or whether the functionality needs to be specified are FFS.

Editor's note:	Accuracy monitoring in VFL is FFS.

Editor's note:	For an NWDAF impacts of the split into AnLF and MTLF are FFS.

Vertical Federated Learning includes the following procedures:

-	Registration of the NF profile including a list of VFL related information to NRF. Registration of the NWDAF profile to NRF is described in clause 5.2. Registration of the AF profile to NRF is described in clause 5.5. The procedure for registration and discovery of VFL server and VFL client is described in clause 6.2H.2.1.

-	Preparation for VFL including sample alignment to ensure that all the VFL participants have common samples when training ML models as described in clause 6.2H.2.2.

-	Training for VFL as described in clause 6.2H.2.3.

-	Inference for VFL as described in clause 6.2H.2.4.

Editor's note:	If additional functionality is specified in clause 6.2H, the reference to the corresponding clause will be added.

In this release, the same NF associated with a VFL Server or VFL Client capability during the VFL training for a VFL correlation ID is also the same NF during the VFL inference.

Editor's note:	Whether and how change of samples during a VFL training is supported is FFS.

* * *Next Change * * *

6.2H	Vertical Federated Learning among NWDAFs and AFs

6.2H.1	General

This clause specifies procedures for Vertical Federated learning where AFs and NWDAF can can either act as VFL servers or VFL clients. Procedures for registration and discovery, for VFL training preparation, for VFL training, and for VFL inference are covered.

Both the VFL server and VFL client store the VFL model after finishing the VFL training process and use the same VFL local model to perform the VFL inference later based on the VFL correlation ID. The differences between the VFL training and inference are that for the inference there is no check of the labels, nor any server intermediate results are sent and as such only client intermediate results are sent to the server.

* * *Next Change * * *

6.2H.2.3	Training Procedure for Vertical Federated Learning

6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF or trusted AF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1: Training procedure for Vertical Federated Learning when NWDAF or trusted AF is acting as VFL server

Editor's note:	How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS.

Editor's note:	Whether VFL Training Start Flag is needed is FFS.

- [OPTIONAL] VFL training is triggered by the VFL server for the following cases:

-Triggered by ML model retrieving:

If the first NWDAF only containing AnLF that wants to perform inference and does not have a model, it discovers an NWDAF containing MTLF from NRF and sends a subscription request for a model to the NWDAF containing MTLF using Nnwdaf_MLModelProvision_Subscribe.

Step 0a-0b. If the discovered NWDAF containing MTLF does not have a model and realizes it cannot be VFL server but wants an AF to act as VFL server, the NWDAF containing MTLF discovers an AF as VFL server according to 6.2H.2.1, and may send a request  to the VFL server AF using Naf_VFLTraining_Subscribe including Analytics ID, optionally Notification target address, the AF as VFL server starts VFL Training according to step 2.

NOTE 1: The AF may have registered in NRF for the particular Analytics ID that it can perform Inference for it. In this case no training will be performed

Step 0c. If the discovered NWDAF containing MTLF does not have a model but it can be VFL server, the VFL server starts VFL Training according to step 2.

If decision to start VFL training is triggered on ML modelrequest from an NWDAF in step 0b or 0c, the VFL server sends in the service subscription response that no ML model will be available to download and an indication that no model will be provided as described in clause 6.2H.2.4.1. When VFL training is done, the first NWDAF only containing AnLF will be needing to perform VFL Inference using the requested Analytics ID to retrieve an VFL Inference output from the VFL Server (AF or NWDAF).

-Triggered by analytics retrieving:

Step 0d. NF consumer needs analytics output for a specific analytics ID, it discovers an NWDAF from NRF to provide the analytics ID. If the discovered NWDAF is a VFL server which supports the analytics ID, then to generate the analytics output via VFL inference as defined in 6.2H.2.4, the VFL server may trigger VFL training if no correponsing VFL training has been performed.

NOTE 2: In the case when AF is server the clients can only be NWDAFs.

NOTE 3: VFL server can also decide to initiate VFL training based on operator policy or internal configuration.

Editor's note:	Possible procedures to trigger an AF as VFL server to perform model training.are FFS.

Editor's note:	How to select the MTLF as VFL client when the AF is VFL Server is FFS.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

Steps 2-6 are repeated until the training termination condition is reached.

2.	To start the VFL training, the VFL server sends a request to start the training to all selected VFL clients The request includes VFL correlation ID, at least the parameters negotiated during the preparation phase and theVFL training iteration number set to 1, and a Notification Correlation ID., Optionally, the VFL Server includes the NF ID of the VFL Server and  parameters according to clause 6.2H.3.

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing an incremented a VFL training iteration number.  and intermediate training information to each of the VFL clients for next round of VFL training.

2a.	The VFL server sends a Nnwdaf_VFLTraining_Subscribe to the selected NWDAF VFL clients(s).

2b.	The VFL server sends a Naf_VFLTraining_Subscribe to the selected trusted AF VFL clients(s).

2c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Subscribe to the NEF handling that AF.

2d.	For each selected untrusted AF VFL clients, the NEF sends a Naf_VFLTraining_Subscribe to that AF. The NEF may also translate the analytic filter information if needed, e.g. TAIs into geographical area.

.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available. The data used by each VFL Client is collected as per alignment information.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and when applicable (e.g. after the first round of training) and possible intermediate training information distributed by the VFL server in the previous training iteration. Each VFL Client computes and reports the client intermediate training result of the local ML model to the VFL server.

NOTE 4:	The intermediate training information and intermediate training result are constructed in per sample granularity.

Editor's note:	It is FFS and may depend on the service design: When the clients report the client intermediate training result, it also includes the corresponding VFL correlation ID.

Editor's note:	Whether and how to exchange intermediate training result among VFL Clients is FFS.

5.	Each VFL client reports the computed client intermediate training result of the local ML model to the VFL server. the Notification Correlation ID, and a VFL training iteration number. A client may indicate in message for a request to leave the VFL.

NOTE 5:	If the VFL Server deems the training can continue, it responds back by informing the FL Client to cease the ML Model training by performing step 8 for this client.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	A trusted AF VFL client sends a Naf_VFLTraining_Notify to the VFL server.

5c.	An untrusted AF VFL client sends a Naf_VFLTraining_Notify to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external identifiers to internal identifiers and sends a Nnef_VFLTraining_AFClient_Notify to the VFL server.

6.	The VFL server may collect the local data and generate its own local intermediate training result. The VFL Server computes the intermediate model training information (e.g. gradient information or loss information) based on the VFL Client(s) intermediate training result(s) received in step 4, its own local intermediate results and the label. The intermediate model training information is used for updating the models of VFL clients and/or the model of the VFL Server. Different intermediate model training information may be computed for different VFL clients and for the VFL Server itself.

The VFL server may also compute the ML model metric (e.g. ML model accuracy) based on all the intermediate training result received from VFL clients and the label.

Editor's note:	Whether weight of the VFL Client is computed by VFL server is FFS.

7.	[Optional] The VFL server evaluates (e.g. based on the convergence of a loss function or loss value and/or if the pre-set iteration number is reached) whether VFL Training process converged. If the VFL Server evaluates the VFL Training process did not converge, the VFL Server determines another round of VFL training is required and repeats step 2 - 6. If the VFL Server evaluates the VFL training process converged, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Step 7b  Based on the consumer request, the VFL server notifies VFL status report to the consumer, to update the metric value to the consumer periodically (e.g. a certain number of training rounds or at fixed periods) or dynamically when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to either unsibscribe or continue the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

The VFL Server may also decide (e.g. dependent on contribution) to terminate the training for some VFL Clients by informing them to cease the ML Model training by performing step 8 for them and may request them to report back the current Intermediate results.

8.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process, the termination message contains VFL Correlation ID, the VFL Training termination Flag, and may contain the intermediate model training information to each of the VFL clients.

8a.	The VFL server sends a Nnwdaf_VFLTraining_Unsubscribe t to the selected NWDAF VFL clients(s).

8b.	The VFL server sends a Naf_VFLTraining_Unsubscribe to the selected trusted AF VFL clients(s).

8c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Unubscribe to the NEF handling that AF.

8d.	For each selected untrusted AF VFL clients, the NEF sends a Naf_VFLTraining_Unsubscribe to that AF.

9.	The VFL Server, stores VFL correlation ID, the local trained ML Model, the mapping information of the VFL correlation ID to the following parameters: Analytics ID related to the VFL training process, locally trained Model. Additionally, the VFL server stores the VFL client information(including the NF ID of the VFL Client), which may be used to determine associated VFL client in the VFL inference.

Each VFL client updates local ML model based on the intermediate model training information, if received, and stores VFL correlation ID, the locally trained ML Model, the mapping information of the VFL correlation ID to locally trained Model

NOTE 6:	The VFL correlation ID and the stored mapping information are used later for inference as described in Clause 6.2H.2.4.1.

NOTE 7:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

6.2H.2.3.1	Training Procedure for Vertical Federated Learning untrusted AF is acting as VFL server

Figure 6.2H.2.3.2-1: Training procedure for Vertical Federated Learning when untrusted AF is acting as VFL server

Editor's note:	The ENs listed in clause 6.2H.2.3.1 are also applied to this clause.

0.	[CONDITIONAL]

Same as in step 0 in clause 6.2H.2.3.1, but AF is replaced with untrusted AF, and Nnef_VFLTraining_Subscribe offered by NEF is used.

NEF forwards the subscription request to AF using Naf_ VFLTraining_Subscribe.

Either based on the information received or internal configuration, VFL server decides to initiate VFL training procedure.

1.	Same as step 1 in Figure 6.2H.2.3.1-1.

2.	To start VFL training, the VFL server do same as in step 1 in Figure 6.2H.2.3.1-1, using Nnef_VFLTraining_Subcribe.

Steps 3-7 are repeated until the training termination condition is reached.

3.	[Optional] Same as step 3 in Figure 6.2H.2.3.1-1.

4.	Same as step 4 in Figure 6.2H.2.3.1-1.

5.	Same as step 5 in Figure 6.2H.2.3.1-1.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	For an untrusted AF acting as VFL server, the NEF converts any internal identifiers to external identifiers, provides the external NWDAF ID and sends a Nnef_VFLTrainingNotify to the VFL server.

6.	[Optional] Same as step 6 in Figure 6.2H.2.3.1-1.

7.	Same as step 7 in Figure 6.2H.2.3.1-1.

8.	Same as step 8 of Figure 6.2H.2.3.1-1. However, sub steps in that figure are not applicable.

8a.	For each NWDAF VFL client, the untrusted AF as VFL server sends a Nnef_VFLTraining Unsubscribe to the NEF handling that AF. The untrusted AF identifies the VFL client using the external NWDAF ID assigned in the discovery procedure (see clause 6.2H.2.1.1).

8b.	The NEF sends an Nnwdaf_VFLTraining_Unsubscribe to the NWDAF VFL client indicated by the received external NWDAF ID.

9.	Same as step 9 of Figure 6.2H.2.3.1-1.

6.2H.3	Contents of ML Model Training service for Vertical Federated Learning

The consumers of the ML Model training services may provide the input parameters in Nnwdaf_VFLTraining service or Naf_VFLTraining service or in Nnefe_VFLTraining service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be trained.

-	VFL Correlation ID: identifies a VFL process (e.g., preparation, training) to be executed among the candidate VFL participants.

-	OPTIONAL VFL Interoperability Information that indicates the Intermediate training information that the VFL Server supports and ask the client to verify its support for (e.g. activations, gradients, type of loss), the content of the VFL Interoperability Information is not standardized in this release.

-	In preparation phase and in first step of training, the list of samples selected by Server. In later steps in training the VFL server can either indicate a delta from the list sent in first step of training procedure to indicate which samples that will be removed of the rest of training procedure.

- OPTIONAL In training phase, Maximum response time (i.e., the maximum time between VFL clients receive intermediate model training information and send back intermediate training result).

- [OPTIONAL] Training Filter Information: enables to select which data for the ML Model training is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the Training Filter Information are the same as or subset of parameter types in the ML Model Filter Information which are defined in clause 6.2A.2.

- In training, the Intermediate model training information includes what the Server sends to each client which contains what has been agreed in the preparation phase. The content is not standardized in this release.

The VFL client provides to the consumer of the ML Model training service operations the output information in as listed below:

-	VFL Interoperability information supported by each VFL Clients.

-	In preparation phase, the list of samples out of the samples selected by VFL server agreeable by VFL client. In training the VFL client can indicate a delta from the list received in first step of training procedure to indicate which samples that will not be part of the rest of training procedure.

-	OPTIONAL In preparation phase, the Feature ID(s) that the VFL client supports. A Feature ID indicates what features the VFL client can use for an Analytics ID, the values of the Feature ID are not standardized.

-	OPTIONAL VFL Interoperability Information that indicates the Intermediate training information that the client supports in response to what the server sent in the subscription request.

- In training, the Intermediate training result includes what the Client sends to the server which contains what has been agreed in the preparation phase. The content is not standardized in this release.

Editor's note:	Which parameters are optional, or mandatory is FFS, the list of parameters also needs to be extended.

* * * End of Changes * * *