3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500425r01

Online, January 20 - 24, 2025	(revision of S2-250xxxx)

Source:	Ericsson

Title:	KI#3 Conclusion Update of Enabling Temporarily Disabled AIoT devices

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This pCR discuss and update KI#3 interim conclusion to resolve the EN of enabling temporarily disabled devices.

1. Introduction

In SA2#166, there were some discussions about how to enable temporarily disabled AIoT devices. There was no consensus reached. And thus, an EN is left in the interim conclusions:

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled.

-	Enable: request to enable a temporarily disabled AIoT Device.

NOTE:	The security aspect of Enable and Disable is to be concluded by SA WG3.

Editor's note:	It is FFS whether and how to support enabling temporarily disabled AIoT devices.

To enable temporarily disabled devices, from SA2 point of view, AF can send a command towards CN (e.g., Enable command), and CN delivers the command towards devices via readers.

In TR 38.769, according to the agreements from RAN2, the command from upper layer can be delivered in “inventory and command” and “command-only” manners from readers, as “inventory-only” is expected to be used for inventory use case.

For “command-only” delivery manner, as the temporarily disabled devices are still receiving RF signals, they can determine whether they are the target devices. If they are, and if the command is an Enable command, the devices can enable themselves.

For “inventory and command” delivery manner, the temporarily disabled devices will receive an inventory request firstly (i.e., an A-IoT paging over the air). In inventory round, the devices cannot differentiate whether the inventory is caused by a normal inventory or Enable command delivery. They can determine themselves as target devices in A-IoT paging. But they are not expected to respond to the inventory request, as their capabilities of transmitting RF signals are disabled. And without the device ID response, CN has no chance to deliver the Enable command towards those devices.

[Proposal-1] To be able to enable temporarily disabled devices in “inventory and command” delivery, temporarily disabled devices need to respond to the inventory request when it is caused by an Enable command.

To be able to differentiate the inventory request caused by an Enable command, an indicator is required.

The figure above depicts the high-level flows of the interactions among AIoT device, Reader and CN for the inventory and command case, which is for illustrative purpose:

In step 2, when CN (the AIOTF) receives an Enable command, it can include an indicator which enables temporarily disabled devices to respond in the inventory request towards the reader.

In step 3, the reader includes the indicator together with the device ID information in A-IoT paging.

In step 4, only when the device ID information matches and the indicator is set, the temporarily disabled devices understand that they need to respond to the A-IoT paging. Otherwise, those devices can silently ignore the A-IoT paging.

[Proposal-2] For Enable command, the AIOTF includes an indicator which enables temporarily disabled devices to respond in the inventory request towards the reader. The reader includes the indicator in A-IoT paging, and the temporarily disabled devices can understand that they need to respond to the A-IoT paging.

In SA2#166, the explicit indicator was discussed but not agreed. Some companies believe there are no needs to introduce such explicit indicator. Instead, the device ID information itself would be sufficient. That is, in A-IoT paging, when the device ID information represents a group of AIoT devices (e.g., device ID + MASK), the temporarily disabled devices should not respond. Only when individual device ID is used in A-IoT paging, the temporarily disabled devices should respond. To avoid the temporarily disabled devices to respond inventory request or other command request, the CN (i.e., the AIOTF) should reject the request from the AF, if the request is targeting to this temporarily disabled device.

This proposal utilizes the individual device ID as an implicit indicator to enable the temporarily disabled AIoT devices to understand whether they need to respond to the A-IoT paging. Such implicit indicator could work when the AF sends an Enable command request targeting to this individual device. However, when the AF sends an Enable command request targeting to a group of devices, the temporarily disabled devices within the group will not respond, as the device ID information in A-IoT paging is not individual device ID.

[Observation-1] The proposal of using individual device ID as an implicit indicator which enables temporarily disabled device to respond, could work if the device ID information represents one or more individual devices, but has issues if the device ID information represents a group of devices.

According to clause 5.3.2 of TS 22.369, the management mechanism shall work for an AIoT device or a group of AIoT devices:

The 5G network shall support suitable management mechanisms for an Ambient IoT device or a group of Ambient IoT devices.

The 5G system shall support a mechanism to:

- 	disable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently able to transmit RF signals

- 	enable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently disabled to transmit RF signals

Based on operator policy, the 5G system shall provide a suitable mechanism to permanently disable the capability of an Ambient IoT device or a group of Ambient IoT devices to transmit RF signals.

Subject to operator policy and regulatory requirements, the 5G system shall support suitable mechanisms for the Ambient IoT device to move between one or more networks and countries.

[Observation-2] The 5G network shall support enable/disable a group of AIoT devices. AF may send request to enable/disable a group of devices.

When AF request to enable a group of devices, the solution of implicit indicator requires the AIOTF to split the request from the AF to quite many requests towards the reader, while each request targets to one or more individual devices. Depends on the request splitting, the AIOTF needs to trigger the A-IoT paging for quite many times, which makes the service request handling inefficient, especially when device MASK information reflects a large number of devices but not so many devices are deployed in the area.

The solution of using implicit indicator and the one using explicit indicator are not mutually exclusive. They can cooperate to make a complete solution:

- When the AF request to enable a group of temporarily disabled devices, the explicit indicator is included in A-IoT paging which enables temporarily disabled devices to respond.

- When the AF request to enable individual temporarily disabled devices only, the explicit indicator or the implicit indicator in A-IoT paging can enables temporarily disabled device to respond.

[Proposal-3] Resolve the EN by introducing an explicit indicator in A-IoT paging when AF is to enable individual or a group of temporarily disabled devices. The implicit indicator (individual device ID) in A-IoT paging could also work if AF is to enable individual device(s) only.

[Proposal-4] The explicit indicator impacts A-IoT radio which requires feedback from RAN2. Add an EN to highlight the dependency.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v1.2.1:

* * * Start of Change * * * *

### 8.3.2	AIoT services supported by the 5GC

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled.

-	Enable: request to enable a temporarily disabled AIoT Device. When "inventory and command" delivery applies, an explicit indicator is needed in A-IoT NAS layer in Inventory Request when the AF requests to enable individual device(s) or a group of devices. The explicit indicator enable temporarily disabled devices to respond the A-IoT paging.

NOTE X:	The protection of the explicit indicator depends on the conclusion in SA3 on protected device ID.

NOTE:	The security aspect of Enable and Disable is to be concluded by SA WG3.

* * * End of Change * * * *