3GPP TSG-SA2 Meeting #166-Ad-Hoc-e	S2-2500432r02

# 20 - 24 January, 2025, Electronic meeting

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1341 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sample ID intersection by NEF during the VFL preparation in untrusted AF VFL server scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, vivo?, Lenovo? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following exception has been agreed upon in SP-241513/S2-2413037: Vertical Federated Learning (VFL) For the case where AF is untrusted whether additional functionality in NEF is needed during the VFL preparation When untrusted AF is acting as the VFL server, as it has been documented in clause 6.2H.2.2.2 of TS 23.288, the NEF will forward the VFL preparation response from every VFL client individually to the server by mapping a list of internal sample IDs to external IDs. Considering the potential high number of VFL clients and the sample IDs in each VFL preparation response, NEF will have very high workload on sample ID mapping and VFL preparation response exposure. Furthermore, exposing the supported sample IDs of every single NWDAF VFL clients can potentially increase the risk of 5GC privacy leaking. Therefore, to reduce NEF workload and improve 5CG privacy, this CR proposes to allow the NEF to aggregate the VFL preparation responses from multiple VFL clients to determine the intersection of sample IDs. Then the NEF will only map the internal IDs of the intersected Samples to external IDs, which reduce NEF exposure workload and lower the risk of 5GC privacy leakage. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove EN on NEF work during VFL preparation in clause 6.2H.2.2.2. Introduce new steps into clause 6.2H.2.2.2 to support NEF to determine the sample ID intersection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NEF is with high workload caused by sample ID mapping and VFL preparation response exposure. 5GC privacy cannot be secured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

##### 6.2H.2.2.2	Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL server

This clause specifies the preparation (including sample alignment) procedure for untrusted AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

Editor´s note:	It is FFS whether trusted AF can act as VFL clients when an untrusted AF acts as VFL Server.

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL Server

1-	The untrusted AF as VFL server sends VFL preparation requests to each of the candidate VFL client(s), using Nnef_VFLTrainingRequest_Request to check if the VFL client(s) can meet the ML Model training requirement (which includes the Analytics ID, list of sample IDs, and optionally Feature ID, etc.). The external NWDAF IDs obtained in the discovery procedure (see clause 6.2H.2.1.1) is included to indicate the target NWDAFs. The suggested VFL Interoperability Information to negotiate the intermediate resuls that will be used in training.

Alternatively, for option 2, the untrusted AF as VFL server sends one VFL preparation request the NEF for multiple NWDAF VFL clients. The NEF translates the external NWDAF IDs into internal IDs and forwards the VFL preparation request to the NWDAF VFL clients.

2-	The NEF maps the external NWDAF and GPSI(s) to the internal NWDAF and SUPI(s). The NEF send VFL preparation request to the candidate internal NWDAF ID using Nnwdaf_VFLTraining_Request service with the same information as provided in step 1 in clause 6.2H.2.2.1.

3-	Same as step 2 in clause 6.2H.2.2-1.

4-	Same as step 3 in clause 6.2H.2.2-1.

5a-5c	The NEF maps the internal NWDAF and SUPI(s) to the external NWDAF and GPSI(s). The NEF sends the Nnef_VFLTraining_Request Response to the VFL Server with the same information as provided in step 4.

5d-5e.	Alternatively, for option 2, the NEF aggregates the response from multiple NWDAF VFL clients and determines the sample ID intersection among those VFL clients (if responses from more than one VFL clients are received). The NEF maps the internal NWDAF IDs and the intersected SUPI(s) only to the external NWDAF and GPSI(s), respectively. Then the NEF sends one Nnef_VFLTraining_Request Response to the VFL Server with the external NWDAF IDs and one list of intersected GPSI(s).

6-	Same as step 4 in clause 6.2H.2.2-1.

* * *End of Changes * * *