3GPP TSG-SA2 Meeting #166-Ad-Hoc-e	S2-2500432r02

# 20 - 24 January, 2025, Electronic meeting

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1341 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sample ID intersection by NEF during the VFL preparation in untrusted AF VFL server scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, vivo?, Lenovo?, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following exception has been agreed upon in SP-241513/S2-2413037: Vertical Federated Learning (VFL) For the case where AF is untrusted whether additional functionality in NEF is needed during the VFL preparation Unregistered UEs are not suitable for samples as no data related to them can be collected. An untrusted AF can use the Nnef_MemberSelection service to check whether UEs are registered. This avoids interactions with each VFL client. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove EN on NEF work during VFL preparation in clause 6.2H.2.2.2. The VFL server checks whether UEs are registered at the UDM. An untrusted AF can use the Nnef_MemberSelection service . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved issue about sample selection remains and there is a risk of unnnecessary signalling as each client might perform checks at the UDM for each UE. . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

##### 6.2H.2.2.2	Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL server

This clause specifies the preparation (including sample alignment) procedure for untrusted AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

Editor´s note:	It is FFS whether trusted AF can act as VFL clients when an untrusted AF acts as VFL Server.

Figure 6.2H.2.2-1: Preparation procedure for Vertical Federated Learning when untrusted AF is the VFL Server

0.	If UEs are to be used as samples, an untrusted AF may use the Nnef_MemberSelection service to ask the NEF to reduce the list of candidate UEs based on filter criteria it provides.

1-	The untrusted AF as VFL server sends VFL preparation requests to each of the candidate VFL client(s), using Nnef_VFLTrainingRequest_Request to check if the VFL client(s) can meet the ML Model training requirement (which includes the Analytics ID, list of sample IDs, and optionally Feature ID, etc.). The external NWDAF IDs obtained in the discovery procedure (see clause 6.2H.2.1.1) is included to indicate the target NWDAFs. The suggested VFL Interoperability Information to negotiate the intermediate resuls that will be used in training.

2-	The NEF maps the external NWDAF and GPSI(s) to the internal NWDAF and SUPI(s). The NEF send VFL preparation request to the candidate internal NWDAF ID using Nnwdaf_VFLTraining_Request service with the same information as provided in step 1 in clause 6.2H.2.2.1.

3-	Same as step 2 in clause 6.2H.2.2-1.

4-	Same as step 3 in clause 6.2H.2.2-1.

5a-5c	The NEF maps the internal NWDAF and SUPI(s) to the external NWDAF and GPSI(s). The NEF sends the Nnef_VFLTraining_Request Response to the VFL Server with the same information as provided in step 4.

6-	Same as step 4 in clause 6.2H.2.2-1.

* * *End of Changes * * *