3GPP TSG-SA2 Meeting #166-AdHoc-e	S2-2500446r02

# E-meeting, 20 – 24 January 2025	   		       		          					(was S2-2410595,12317)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3831 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE determination for S&F mode operation or power saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Store and Forward feature and corresponding architecture enhancements has been introduced in TS 23.401 clause 4.13.9. Based on CR3776, the feature of discontinous network coverage for satellite access has been supported to provide the coverage availability information of both the service link and feeder link. However in Rel-19, for network in S&F mode of operation, the service link and feeder link may not be simultaneously available. The enhancement for the coverage availability information is needed. In addition, the UE capable of both Enhanced Discontinuous Coverage Support and S&F mode operation may determine to apply power saving optimization functionalities rather than enable S&F mode. The clarification for triggering power saving optimization is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR introduces the following aspects: - Enhancement for the UE to determine whether to trigger power saving mechanism when RAN and core network entities perform Store and Forward Satellite operation. - Enhancement for the Coverage Availability Information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE is unable to determine whether to apply power saving mechanism based on Coverage availability information RAN and core network entities perform Store and Forward Satellite operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.8.2, 4.13.8.3, 4.13.8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# FIRST CHANGE

#### 4.13.8.2	Mobility Management and UE Power Saving Optimization

For satellite access that provides discontinuous network coverage, and in case both the UE and the network support Enhanced Discontinuous Coverage, then the Mobility Management and UE Power Saving Optimization functionality may be used.

If both the UE and the network indicate support for "Enhanced Discontinuous Coverage Support" and if the UE determines it will lose coverage and will become unavailable, and decides to remain in no service during that time, then:

-	The UE may be able to determine, including considering current and expected future locations of the UE, a Start of Unavailability Period and/or Unavailability Period Duration for when it expects to be out of coverage.

NOTE 1:	A UE, based on implementation, can combine successive periods of no satellite coverage into a single continuous period that is notified to the network if the UE does not require network access during this period.

NOTE 2:	UE informing the network of coverage gaps would increase signalling and UE power consumption if coverage gaps are more frequent than UE's communication period.

-	The UE triggers a TAU procedure and includes an indication of upcoming loss of coverage in the TAU Request message to the MME. If the UE is able to determine a Start of Unavailability Period and/or Unavailability Period Duration it also includes them in the TAU Request message.

-	The UE should trigger the TAU procedure early enough such that the procedure, under normal conditions, is able to complete before the start of the unavailability period.

-	The UE and the MME re-negotiate unavailability at every TAU procedure, if it is required. If Start of Unavailability Period and/or Unavailability Period Duration is not included in a TAU Request message any pending loss of coverage configuration stored in the UE context at MME is discarded.

-	If the UE determines an upcoming loss of coverage to the network no longer applies or determines a new Start of Unavailability Period or Unavailability Period Duration related to the upcoming loss of coverage, the UE sends a new TAU Request to the MME to update the Start of Unavailabililty Period and/or Unavailability Period Duration.

-	The UE may determine to trigger Power Saving Optimization functionality during feeder link unavailability period when camping on a cell broadcasting an indication of operating in S&F mode (as described in clause 4.13.9.1 and TS 36.300), or according to coverage availability information provisioned to the UE (as described in clause 4.13.8.3).

Upon receiving a TAU Request message from the UE including an indication of upcoming loss of coverage or Attach request message:

-	If the UE did not include a Start of Unavailability Period in the TAU Request message, the MME considers implicitly the Start of Unavailability Period to be the time at which it has received the TAU Request message from the UE. If the UE included a Start of Unavailability Period, the Start of Unavailability Period indicates the time at which the UE determines it expects to lose coverage, i.e. time until which the UE determines it is available.

-	The MME may determine, if not provided by the UE, or update the Unavailability Period Duration and/or the Start of Unavailability Period.

If the MME knows an Unavailability Period Duration and/or the Start of Unavailability Period (e.g. based on information available to the MME as described in clause 4.13.8.4) for the UE, and the UE not include an Unavailability Period Duration and/or the Start of Unavailability Period or included an Unavailability Period Duration and/or the Start of Unavailability Period different to the Unavailability Period Duration and/or the Start of Unavailability Period known to the MME, the MME should include the Unavailability Period Duration and/or the Start of Unavailability Period known to the MME in the TAU Accept or Attach Accept message and use those values in subsequent steps of this procedure. How the UE treats the MME provided Unavailability Period Duration and/or the Start of Unavailability Period is up to UE implementation e.g. to help to determine when to return to coverage after a discontinuous coverage period, whether to listen to paging in eDRX, not to initiate any NAS signalling (including Service Request for MO data) within the discontinuous coverage period in case of any UL signalling/data request or the UE may deactivate its Access Stratum functions for satellite access in order to optimise power consumption until coverage returns, etc.

The MME indicates to the UE in the Attach Accept or TAU Accept whether the UE is not required to perform a TAU procedure when the unavailability period has ended.

-	The MME stores the information that the UE is unavailable at the Start of Unavailability Period in the UE context, and considers the UE is unreachable from then until the UE enters ECM_CONNECTED state.

NOTE 3:	The UE does not include indication of upcoming loss of coverage, Unavailability Period Duration and the Start of Unavailability Period in Attach Request message.

If the UE requests power saving features the MME uses procedures defined in other clauses to provide the UE with timers (e.g. periodic TAU timer, extended idle mode DRX (see clause 5.13a), and PSM mode configuration (see clause 4.3.22)), and may also consider the Unavailability Period Duration (if available) and Start of Unavailability Period (if available).

Unless the MME indicated that the UE is not required to perform a TAU procedure when the unavailability period has ended, then once the event which makes the UE unavailable is completed in the UE, the UE triggers a TAU procedure. If the event which makes the UE unavailable is delayed to a future time or cancelled or unavailability period deviates from negotiated value in the UE, the UE triggers TAU procedure.

The MME should adjust the mobile reachable timer or Implicit Detach timer or both such that the MME does not implicitly detach the UE while the UE is out of coverage, see clause 4.3.5.2. Features described for High latency communication in clause 4.3.17.7 may be used to handle mobile terminated (MT) communication with UEs being unreachable due to satellite access with discontinuous coverage and the Unavailability Period Duration (if available) and Start of Unavailability Period (if available) may be used when determining the Downlink Buffering Duration time.

The UE may send Tracking Area Update request message to inform the network of its UE unavailability period even if Mobility Management back-off timer is running.

# 2nd CHANGE

#### 4.13.8.3	Coverage availability information provisioning to the UE

A UE may use satellite coverage availability information for satellite access to support discontinuous coverage operations. Satellite coverage availability information can be provided to a UE by an external server via a PDN Connection or SMS. The protocol and format of satellite coverage availability information via PDU Connection or SMS is not defined in this release of the specification, but some examples on the information that constitutes the satellite coverage availability information is defined in Annex N.

NOTE:	The satellite coverage availability information provisioned to the UE describes when and where satellite coverage with both the respective service link connectivity and feeder link connectivity is expected or not expected to be available in an area.

# THIRD CHANGE

#### 4.13.8.4	Coverage availability information provisioning to the MME

The MME may use satellite coverage availability information to support satellite access by UEs with discontinuous coverage operation. Satellite coverage availability information may be provisioned to the MME by O&M.

NOTE 1:	In this release of the specification there is no support for provisioning of satellite coverage availability information to an MME from an AF.

NOTE 2:	The satellite coverage availability information provisioned to the MME describes when and where satellite coverage with both the respective service link connectivity and feeder link connectivity is expected or not expected to be available in an area.

The satellite coverage availability information is not UE specific and can be applied by the MME for any UE in the affected area.

# End CHANGE

## 4.13	Introduction of satellite support for Cellular IoT

### 4.13.x	Support of Store and Forward Satellite Operation

#### 4.13.x.1	General

The Store and Forward Satellite operation in an E-UTRAN system with satellite access provides communication service for UEs in periods of time and/or geographical areas in which the satellite serving a UE does not have a simultaneous service link connection and feeder link connection. The Store and Forward Satellite operation is suitable for delay-tolerant communication services (e.g. CIoT/MTC, SMS, etc).

In Store and Forward Satellite Operation, the end-to-end exchange of signalling/data traffic is handled as a combination of two or more steps that are not concurrent in time. In the first step, signalling/data exchange between the UE and the satellite occurs without the satellite being simultaneously connected to the UE via the service link and ground network via the feeder link. The satellite moves from being connected to the UE in the first step to being connected to the ground network in the second step. In the second step, the connectivity between the satellite and the ground network is established so that communication between the satellite and the ground network can take place to complete the end-to-end signaling/data exchange.

When a satellite is operated in S&F mode which Feeder link is not available, the onboard eNB broadcasts an indication of operating in S&F mode over Service link to the UEs as described in TS 36.300 [5]. A UE determines when an eNodeB is operating in S&F mode of operation, as described in TS 36.300 [5]. The UE supporting S&F mode shall include the S&F capability during initial network access.

NOTE 1:	From system perspective if the network does not support S&F Satellite operation and the Feeder link is not available then RAN switches off and does not broadcast any signal.

When a satellite is operated in S&F mode which Feeder link is not available, the UE in S&F mode accesses S&F-based services from E-UTRAN satellite access in S&F mode. This network attachment and access to S&F-based services may use one or more satellites, depending on the deployment and implementation options.

The MME in the S&F mode shall reject the NAS procedure defined in TS 24.301[46] if it cannot be completed with the information currently available in the satellite e.g. the MME does not have UE security context or, the integrity check fails, and/or the MME fails to retrieve UE-specific authentication vectors or subscription information. In the response message of the related NAS procedure as defined in TS 24.301[46], MME shall include a reject cause code. The MME may also include a S&F wait timer and a list of satellites (i.e. S&F Monitoring list) from the same (UE selected) PLMN with which UE can attempt to retry the related NAS procedure as defined in TS 24.301[46], in the future.

NOTE 2: The UE can select a different PLMN to get services after getting rejection cause code due to S&F.

NOTE 3:	How the MME determines the S&F Monitoring List and S&F wait timer is up to MME implementation and how the UE treats the S&F Monitoring List and S&F wait timer is up to UE implementation.

Editor’s Note: It is FFS whether the S&F wait timer can be the same as the Unavailability Period Duration.

Upon or after expiry of the S&F wait timer, the UE may reattempt the related NAS procedure as defined in TS 24.301[46], towards any of the satellites among the list of satellites in the S&F Monitoring List provided during rejection. During the reattempt of related NAS procedure as defined in TS 24.301[46], if the MME has the UE context, authentication vectors and subscription data, the MME accepts the request and may indicate the estimated UL delivery time to UE. Additionally, the MME may include a S&F wait timer to indicate to the UE the time. UE shall wait before triggering any subsequent NAS procedure with one of the satellite of the same PLMN within S&F Monitoring list from the same (UE selected) PLMN, to indicate to the UE which are the satellites for triggering subsequent NAS procedures.

Editor’s Note: It is FFS whether the S&F monitoring list and S&F wait timer is needed to be sent to UE during Attach, TAU or service accept.

NOTE 4: How the MME determines the estimated UL delivery time is up to MME implementation.

NOTE 5: How UE uses estimated UL delivery time information is left for UE implementation.

Editor’s Note: It is FFS whether the estimated UL delivery time can be the same as the Start of Unavailability Period Duration such that the UE can use S&F services until the Service Link is unavailable.

If the UE does not include the S&F capability, the MME shall not include in the response message, the S&F rejection cause, any S&F Monitoring List or S&F wait timer to the UE. This rejection cause code can be an existing cause code other than S&F reject cause code.

The MME may indicate the timestamp information to HSS during the Location update procedure. This timestamp information shall be used by the HSS to ensure that newer location records for that UE are not cancelled.. The MME may trigger an Update location with the HSS along with an authentication procedure to fetch subscription information. If MME makes an Update Location Request before the completion of the authentication procedure, it shall include an indication that this Location Update is provisional, i.e. the HSS shall not consider the UE as registered until it receives the final Update Location Request after the completion of the authentication procedure. The timestamp information is the time when the MME has received the NAS procedure from the UE.

Editor’s Note: It is FFS whether the Authentication request can be used to inform the above timestamp information to HSS.

NOTE 6:	The timestamp is used by the HSS to ensure that interactions with MME for a UE in S&F mode are handled in the correct order.

Some example deployments are described in Annex X.

# End of CHANGEs