3GPP TSG-WG SA2 Meeting #166-AdHoc-e	S2-2500455r01

# 20 - 24 January, 2025, E-meeting

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5956 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for S-NSSAI granularity energy consumption exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Proposes support for S-NSSAI per UE granularity energy consumption exposure to address the below EN: Editor's note: Whether there are other required parameters for energy consumption information exposure are FFS. Having already collected energy consumption information per UE and per PDU Session granularities, it takes minimal processing time to provide energy consumption exposure per S-NSSAI per UE granularity. R02 – Clarified that the proposed S-NSSAI garanularity energy consumption is per UE S-NSSAI granularity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for S-NSSAI per UE granularity energy consumption exposure plus some cleanup. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | S-NSSAI per UE granurarity energy consumption exposure not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.1; 5.51.2.2; 5.51.2.3; 5.51.2.4; Annex T2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.502 CR 5244 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CRs revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 5.51.2.1	General

The Energy Information Function (EIF) is defined to collect the UE related Energy Consumption information, calculate the Energy Consumption information at UE, S-NSSAI per UE, PDU Session and/or QoS flow granularity and expose the Energy Consumption information to the authorized consumer NF(s) (AF/NEF or 5GC NF) subject to operator's policy.

Node level energy consumption is reported PLMN-wide at a configurable period T.

# * * * * end of 1st change * * * *

# * * * * 2nd change * * * *

#### 5.51.2.2	Energy Consumption information collection

The Energy Information Function (EIF) collects the UE related Energy Consumption information include Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e. S-NSSAI per UE, PDU Session and/or QoS flow) from UPF (via SMF).

The serving SMFs are retrieved from the UDM of the UE based on the provided input parameters including the UE ID and (S-NSSAI, DNN).

EIF provides UE ID, DNN/S-NSSAI, IP 5-tuple to retrieve the information from SMF. And the information collected from SMF by EIF, is shown in Table 5.51.2.2-1.

The data volume reporting period from the SMFs is the period T with PLMN-wide configuration.

Table 5.51.2.2-1: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI. |
| S-NSSAI per UE | Network Slice applicable to a UE |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| IP 5-Tuple | IP-5-tuple. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time period. |
| > UL/DL Data Volume of UPF | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > (I-)UPF ID(s) | Identifier of any (I-)UPF(s) associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a QoS flow (UE ID, S-NSSAI, DNN, IP 5-Tuple). |
| > gNB serving the UE | The gNB which serving the UE. |
| Time period | Time period of the collected information. |

Table 5.51.2.2-2 provides the list of information received from OAM. EIF requests the OAM information with providing the serving gNB ID(s) and (I-)UPF ID(s), which provided by SMFs. The reporting period from the OAM is the period T which is the same as data volume reporting period from SMFs.

Table 5.51.2.2-2: Information from OAM for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| gNB energy consumption | The Energy consumed by a gNB over the configured time period T based on clause 6.7.3.4.2 of TS 28.554 [209]. |
| gNB data volume | The UL/DL data volume handled by a gNB over the configured time period T based on clause 6.7.1.1 of TS 28.554 [209]. |
| UPF energy consumption | The Energy consumed by a UPF over the configure time period T based on clause 6.7.3.1 of TS 28.554 [209]. |
| UPF data volume | Data volume consumed at a UPF. |

When the gNB and/or the (I-)UPF(s) which serving the UE are changed, the serving gNB ID and UPF ID will be updated to the EIF through SMF(s).

NOTE 1:	The Node-level Energy Consumption information received from OAM could be used by EIF for all the UEs serving by the NF Node.

NOTE 2:	In this Release, only the energy related information of user plane communication is supported and that of control plane signalling is not supported.

# * * * * end of 2nd change * * * *

# * * * * 3rd change * * * *

#### 5.51.2.3	Energy Consumption information calculation

EIF calculates the Energy Consumption information of the required granularities (UE, S-NSSAI per UE, PDU Session and/or QoS flow), based on input parameters from Tables 5.51.2.2-1 and 5.51.2.2-2 and gets the results. Some example formulas to support the above calculation are described in Annex T.

# * * * * end of 3rd change * * * *

# * * * * 4th change * * * *

#### 5.51.2.4	Energy Consumption information exposure

AF/NEF or 5GC NF subscribes the EIF for Energy consumption information of required granularities (UE, PDU session and/or QoS flow).

-	For UE level energy exposure, the consumer NF provides UE ID (SUPI/GPSI).

-	For S-NSSAI per UE level exposure, the consumer NF provides UE ID (SUPI/GPSI), S-NSSAI.

-	For PDU session level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI.

-	For QoS flow level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and Flow description(s).

-	The consumer NF may also subscribe the above information exposure with providing reporting period, reporting frequency.

Editor's note:	Whether there are other required parameters for energy consumption information exposure are FFS.

Editor's note:	Whether and how the renewable energy is introduced is FFS and depending on SA WG5.

Editor's note:	Whether and how the per application energy consumption information can be exposed are FFS.

# * * * * end of 4th change * * * *

# * * * * 5th change * * * *

# T.2	Calculation of Energy Consumption parameters

The energy consumed at a UPF (EUPF) or a gNB (EgNB) is known over a time window T. And the EIF is aware of the data volume (DV) consumed at a UPF by a UE (DVUE,UPF), UE (DVS-NSSAI,UPF), UE PDU session (DVSession,UPF) and UE QoS flow (DVFlow,UPF) over the time interval T. The EIF is aware of the overall data volume at a gNB (DVgNB), as well as at a UPF  (DVUPF) which serving the UE. Then based on the below example formulas, the required granularities Energy consumption information can be obtained.

To derive the of energy consumed at a gNB over a period of time T by a UE, S-NSSAI per UE, PDU session, QoS flow, the formulas are:

EUE,gNB=EgNBDVUE,gNBDVgNB

ES-NSSAI,gNB=EgNBDVS-NSSAI,gNBDVgNB

ESession,gNB=EgNBDVSession,gNBDVgNB

EFlow,gNB=EgNBDVFlow,gNBDVgNB

To derive the energy consumed at a UPF over a time period T by a UE, S-NSSAI per UE, PDU session, QoS flow the formulas are:

EUE,UPF=EUPFDVUE,UPFDVUPF

ES-NSSAI,UPF=EUPFDVS-NSSAI,UPFDVUPF

ESession, UPF=EUPFDVSessionUE,UPFDVUPF

EFlow,UPF=EUPFDVFlow,UPFDVUPF

To derive the energy consumed in the network in the time interval T by a UE (EUE), S-NSSAI (ES-NSSAI), Session (ESession), QoS flow (EFlow), the formula is:

EUE,gNB=gNBi∈gNBUETEUE, gNBi+ UPFi∈UPFUETEUE, UPFi=ESession

where gNBUET are all gNBs used by the UE in an interval T, UPFUET are all UPFs used by the UE in an interval T.

ES-NSSAI=gNBi∈gNBS-NSSAITES-NSSAI, gNBi+ UPFi∈UPFS-NSSAITES-NSSAI, UPFi

where gNBS-NSSAIT are all gNBs used by the S-NSSAI per UE in an interval T, UPFS-NSSAIT are all UPFs used by the S-NSSAI per UE in an interval T.

ESession=gNBi∈gNBSessionTESession, gNBi+ UPFi∈UPFSessionTESession, UPFi

where gNBSessionT are all gNBs used by the PDU Session in an interval T, UPFSessionT are all UPFs used by the PDU Session in an interval T.

EFlow=gNBi∈gNBFlowTEFlow, gNBi+ UPFi∈UPFFlowTEFlow, UPFi

where gNBFlowT are all gNBs used by the QoS Flow in an interval T, UPFFlowT are all UPFs used by the QoS Flow in an interval T.

# * * * * end of 5th change * * * *