3GPP TSG-WG SA2 #166-Ad Hoc-e	S2-2500458

# Jan. 20 – 24, 2025, eMeeting

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5958 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: Correct the RTCP ‘MID’ for IP packet filter set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For KI#4: To support the differentiated QoS handling for multiplexed media flows, (S)RTP Multiplexed Media Identification Information has been included to provide extension to the IP Packet Filter Set in the clause 5.7.6.2. According to the SA4 LS-in (S4-242065, also being S2-2412586), the item ‘Media Identification (MID)’ is included for (S)RTP in the IP Packet Filter Set. However, this is inaccurate. As quoted from the IETF RFC-9143 (Negotiating Media Multiplexing Using the Session Description Protocol): … defines a new RTCP SDES item [RFC3550], 'MID', which is used to carry identification-tags within RTCP SDES packets. … also defines a new RTP SDES header extension, which is used to carry the 'MID' RTCP SDES item in RTP packets… Here, ‘MID’ is ‘Media Identification’. The above quoted paragraph (from the RFC-9143) contains quite some information: - The RFC-3550 states explicitly that the SDES, or Source Description item, is a type of RTCP control packet. - The ‘MID’ or Media Identification is defined as a new RTCP SDES item that is used to carry identification-tgas within RTCP SDES packets. That is, it is *NOT* an (S)RTP item. - Corrrespondingly, a new RTP SDES header extension is defined to carry the ‘MID’ RTCP SDES item in RTP packets. From the above explanations, the newly included (S)RTP Multiplexed Media Identification Information for IP Packet Filter Set needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the inaccurate part of the (S)RTP Multiplexed Media Identification Information for IP Packet Filter Set. - ‘Protocol Description’ (in 5.37.5.1) to include the reference to RFC 9143 (for new RTCP SDES MID and corresponding RTP header extension for MID). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Part of the (S)RTP Multiplexed Media Identification Information for IP Packet Filter Set is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.6.2, 5.37.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes* * * *

#### 5.7.6.2	IP Packet Filter Set

For IP PDU Session Type, the Packet Filter Set shall support Packet Filters based on at least any combination of:

-	Source/destination IP address or IPv6 prefix.

-	Source / destination port number.

-	Protocol ID of the protocol above IP/Next header type.

-	Type of Service (TOS) (IPv4) / Traffic class (IPv6) and Mask.

-	Flow Label (IPv6).

-	Security parameter index.

-	Packet Filter direction.

NOTE 1:	A value left unspecified in a Packet Filter matches any value of the corresponding information in a packet.

NOTE 2:	An IP address or Prefix can be combined with a prefix mask.

NOTE 3:	Port numbers can be specified as port ranges.

NOTE 4:	Type of Service (IPv4)/Traffic class (IPv6) can be used to define packet filters for DSCP and ECN as described in RFC 3168 [193].

-	(S)RTP Multiplexed Media Identification Information including a combination of at least one of the following:

-	Synchronization Source (SSRC), as defined by IETF RFC 3550 [185];

-	Payload Type (PT), as defined by IETF RFC 3550 [185];

-	RTCP SDES item Media Identification (MID) and RTP SDES header extension for MID, as defined by IETF RFC 9143 [207]

-	RTCP packet type.

# * * * * Next changes* * * *

#### 5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as a video frame or video slice. The PDU Set based QoS handling by the 5G-AN is determined by PDU Set QoS Parameters in the QoS profile of the QoS Flow (specified in clause 5.7.7) and PDU Set Information provided by the PSA UPF via N3/N9 interface as described in clause 5.37.5.2. The PDU Set based Handling can be applied for GBR and non-GBR QoS Flows. The AF should provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS Parameters as described in clause 5.7.7

-	Protocol Description: Indicates the transport protocol used by the service data flow (e.g. RTP, SRTP) and information, e.g. the following:

-	RTP [185] or SRTP [186];

-	RTP or SRTP with RTP Header Extensions, including:

-	RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179];

-	Other RTP Header Extensions as defined RFC 8285 [189], RFC 9143 [207];

-	RTP or SRTP without RTP Header Extensions, but together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179] and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with other RTP Header Extensions following RFC 8285 [189] and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]).

NOTE 1:	With the Protocol Description options combining SRTP together with RTP Payload Format the UPF can still obtain some of the PDU Set information from the RTP Header (refer to Annex A of TS 26.522 [179]).

-	Media over QUIC (MoQ) Transport, IETF draft-ietf-moq-transport [201] (as further described in clause 5.37.9.2).

When RTP Header Extensions for PDU Set Marking (as defined in TS 26.522 [179] or other RTP header extensions as defined in RFC 8285 [189] is included, the differentiation between different RTP Header Extension Types should be supported.

When RTP Payload Format is included, the differentiation between different RTP Payload Formats should be supported.

NOTE 2:	Multiplexing of different transport protocols and different media traffic for differentiated PDU Set based handling is not supported in the current Release.

The Protocol Description can be UL only, DL only or UL and DL. The Protocol Description for UL and DL traffic may be different.

For end-to-end encrypted traffic, PDU Set Information is received as media related information from the Application Server, see clause 5.37.9.

NOTE 3:	Identification of PDU Set information for end-to-end encrypted traffic is supported in the DL direction only.

AF provided PDU Set QoS Parameters and UL and/or DL Protocol Description may be used in determining the PCC Rule by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45] and the DL Protocol Description may be used for identifying the PDU Set Information and PDU Set Information marking by the PSA UPF.

When the SMF receives the PCC rule, the SMF performs binding of the PCC rule to one QoS Flow as described in clause 6.1.3.2.4 of TS 23.503 [45]. At least one of the following shall be included in the PCC rule to enable PDU Set based handling: 1) a PSIHI and/or 2) both PSDB and PSER. Based on the PCC rule, the SMF adds the PDU Set QoS Parameters to the QoS Profile of the QoS Flow as described in clause 6.2.2.4 of TS 23.503 [45]. Alternatively, the SMF may be configured to support PDU Set based Handling without receiving PCC rules from a PCF.

For the downlink direction, the PSA UPF identifies PDUs that belong to PDU Sets and marks them accordingly as described in clause 5.37.5.2. If the PSA UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the PSA UPF still maps it to a PDU Set and determines the PDU Set Information as described in clause 5.37.5.2.

NOTE 4:	If the PSA UPF receives a PDU that does not belong to a PDU Set, then it is assumed that the UPF determines the PDU Set Importance value based on pre-configuration.

For the uplink direction, the UE may identify PDU Sets and how this is done is left up to UE implementation. The SMF may send the UL Protocol Description associated with the QoS rule to UE.

NOTE 5:	Using the Protocol Description or not is left to UE implementation. The use of Protocol Description does not impact QoS Flow Mapping in the UE.

In this Release, the PDU Set based handling is supported in 5GS for a UE registered in 3GPP access for single access PDU Session with IP PDU Session Type, for a UE registered in untrusted or trusted non-3GPP accesses for single access PDU Session with IP PDU Session Type, and for a 5G-RG registered in W-5GAN for single access PDU Session with IP PDU Session Type. The support of PDU Set based handling in 5G-RG is specified in TS 23.316 [84].

# * * * * End of changes * * * *