3GPP TSG-SA2 Meeting #166-AdHoc-e	S2-2500475r01

# E-meeting, 20 – 24 January, 2025                  			             			(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0167 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service operation update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To supporting the altitude reporting, the following service operations are impacted: - Nnef_RetrieveInfoUAVFlight_Get. - Nnef_UAVFlightAssistance_Notify. - Nnef_UAVFlightAssistance_Update. It is proposed to update the service operations above to support the altitude reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update service operations to support altitude reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Altitude reporting is not supported by service operations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.3.1, 4.4.1.1.3.4, 4.4.1.1.3.5, 4.4.1.1.4.1, 4.4.1.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

4.4.1.1.3.1	General

Service Description: This service enables the consumer to request assistance for USS changeover if/when a UAV/UE moves from a geographical area served by one USS to an area served by another USS. For this purpose, a USS requests a core network to provide the assistance with the USS changeover, to inform the core network about the flight trajectory assigned and time schedule for the UAV/UE and to receive notifications when the UAV/UE moves out/in the geographical area served by a USS triggering and completing the changeover. This service also enables the UAS NF/NEF to trigger the UAV’s altitude reporting and receive the UAV’s altitude from USS.

* * * Second Change * * *

4.4.1.1.3.4	Nnef_UAVFlightAssistance_Update service operation

Service operation name: Nnef_UAVFlightAssistance_Update

Description: Used by the service consumer to update the core network about changes in the previously provided UAV flight path(s) and/or time schedule, or to inform the UAS NF/NEF about UAV’s altitude.

Input, Required: Flight plan configuration ID.

Input, Optional: Flight path information, which may include e.g. a list of TA, UAV's speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells), Notification Target Address, UAV’s altitude information.

Output, Required: None.

Output, Optional: None.

* * * Third Change * * *

4.4.1.1.3.5	Nnef_UAVFlightAssistance_Notify service operation

Service operation name: Nnef_UAVFlightAssistance_Notify

Description: Notifying a USS about UAV leaving its serviced area and which border crossing point will be used for this purpose, informing a target USS about the UAV entering its serviced area and confirming the completion of the changeover, notifying the USS about the information related to altitude reporting (e.g. the altitude thresholds and information about the required periodicit of the reporting).

Input, Required: Flight plan configuration ID, status of the UAV according to the assigned flight path (leaving, entering, deviating).

Input, Optional: Information about border-crossing point that will be used for USS changeover, altitude thresholds, indication about the required periodicity of the reporting.

Output, Required: Indication whether the USS changeover is completed successfully when used by the target USS when responding to the notification.

Output, Optional: None.

* * * Fourth Change * * *

4.4.1.1.4.1	General

Service Description: This service enables the consumer (i.e. USS) to request the 5GC to collect data from NFs to be used by the USS for UAV's flight planning and assistance purposes, for instance, USS changeover and/or pre-flight planning. This service also enables the UAS NF/NEF to trigger the UAV’s altitude reporting.

* * * Fifth Change * * *

4.4.1.1.4.2	Nnef_RetrieveInfoUAVFlight_Get service operation

Service operation name: Nnef_RetrieveInfoUAVFlight_Get

Description: Requesting from a NEF/UAS NF to retrieve data from 5GC NFs that can be used by USS for UAV's flight purposes, notifying the USS about the altitude reporting.

Input, Required: UAV's identifier (e.g. GPSI), indication about the purpose of the retrieved information (i.e. USS changeover, pre-flight planning).

Input, Optional: List of suitable target USSs, a list of candidate border-crossing point(s), acceptable deviations for flight plan/route, information about candidate flight path(s), requirements on the flight path as described in clause 5.12.

Output, Required: Report containing a Report ID (e.g. Transaction Reference ID assigned by the NEF) and the retrieved information.

Output, Optional: Altitude thresholds, reporting periodicity indication.

* * * End of Changes * * *