3GPP TSG-WG SA2 Meeting #166AHe	S2-2500486

Electronic, Jan. 20th – 24th, 2025	(revision of S2-250xxxx)

Source:	OPPO, vivo

Title:	AIoT T1 Protocol Stack

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT/ Rel-19

Abstract: This paper proposes protocol stack for AIoT Topology 1 in TS 23.xyz.

1. Discussion

1.1	Overview

Based on the conclusion in clause 8.1.2.2 in TR 23.700-13, following protocol stack is documented for Direct Path:

Figure 8.1.2.2-3: Example Protocol Stack between AIOTF and AIoT Device for Topology 1 (Direct Path)

Based on the conclusion in clause 8.1.2.3 in TR 23.700-13, following protocol stack is documented for Indirect Path via AMF:

Figure 8.1.2.3-3: Example Protocol Between AIOTF and AIoT Device for Topology 1 (indirect Path via AMF)

These protocol stacks should be documented in AioT TS 23.xyz.

2. Text Proposal

It’s proposed to agree the following changes into TS 23.xyz.

# * * * * First change * * * *

4.6	Protocol Stacks

4.6.1	General

This clause describes the protocol stack between AIoT Device and AF.

Figure 4.6.1-1: Protocol Stack for AIoT Device

Legend:

- AIoT NAS: The NAS protocol between AIoT Device and AIOTF for AIoT related data transmission. The AIOT NAS doesn’t support Registration Management and Connection Management (RM/CM) and Session Management (SM).

- AIoT AS: It is between the AIoT Device and the Reader is specified in TS 38.300 [Z].

- AIoT Data: It is used to exchange the necessary information between the AIoT Device and AF, e.g. the write data or read date, which is out of 3GPP scope.

As shown in figure 4.6.1-1, the AIoT Device is agnostic to any kind of functionality deployed between the Reader and AIoTF. The AIoT Device is also agnostic to whether the interface between the AIOTF and the Reader is direct or indirect via AMF.

When the reader is RAN reader, the AIoT RAN may communicate to AIOTF via AMF or not. The AIOT RAN is agnostic to the difference between the AMF and AIOTF and setup NGAP connection. The NGAP supports the delivery of AIoT operation (inventory and command) related messages.

- When the AIoT RAN communicates with AIOTF directly, the protocol stack is as specificed in clause 4.6.2.1.

- When the AIoT RAN communicates with AIOTF via AMF, the protocol stack is as specificed in clause 4.6.2.2.

4.6.2	Protocol Stack between AIoT Device and AF

4.6.2.1	Protocol Stack between AF and AIoT Device for Direct Path option

Figure 4.6.2.1-1: Protocol Stack between AF and AIoT Device for Direct Path option

4.6.2.2	Protocol Stack between AF and AIoT Device for Indirect Path via AMF option

Figure 4.6.2.1-2: Protocol Stack between AF and AIoT Device for Indirect Path via AMF option

In this Protocol Stack, AIoT NAS messages are transferred via the AMF transparently.

Editor’s note: Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

Editor’s note: The reference to AIoT AS Layer protocol will be updated based on RAN1/2 decision.

# * * * * End of changes * * * *