3GPP TSG-WG SA2 Meeting #166AHe	S2-2500486r03

Electronic, Jan. 20th – 24th, 2025	(revision of S2-250xxxx)

Source:	OPPO, vivo

Title:	AIoT T1 Protocol Stack

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT/ Rel-19

Abstract: This paper proposes protocol stack for AIoT Topology 1 in TS 23.xyz.

1. Discussion

1.1	Overview

Based on the conclusion in clause 8.1.2.2 in TR 23.700-13, following protocol stack is documented for Direct Path:

Figure 8.1.2.2-3: Example Protocol Stack between AIOTF and AIoT Device for Topology 1 (Direct Path)

Based on the conclusion in clause 8.1.2.3 in TR 23.700-13, following protocol stack is documented for Indirect Path via AMF:

Figure 8.1.2.3-3: Example Protocol Between AIOTF and AIoT Device for Topology 1 (indirect Path via AMF)

These protocol stacks should be documented in AioT TS 23.xyz.

2. Text Proposal

It’s proposed to agree the following changes into TS 23.xyz.

# * * * * First change * * * *

4.6	Protocol Stacks

4.6.1	General

This clause specifies the protocol stacks between entities used for AIoT. The protocol stacks include the following:

-	Protocol stacks between A IoT Device and AFs.

4.6.2	Protocol Stack between AIoT Device and AF

4.6.2.1	General

The general protocol stack between an AIoT Device, AIoT RAN, AIOTF and AF is shown in Figure 4.6.2.1-1.

The protocol stacks and routing of information between an AIOT Reader and the AIOTF protocol is defined in other clauses depending upon the architecture used, e.g. directly between AIoT RAN containing an AIoT Reader and the AIOTF, indirectly between AIoT RAN containing an AIoT Reader and the AIOTF via an AMF.

Legend:

-	AIoT NAS: The NAS protocol between AIoT Device and AIOTF.

-	AIoT AS: It is between the AIoT Device and the AIoT RAN is specified in TS 38.300 [Z].

-	AIoT Data: It is the application data exchanged between the AIoT Device and AF.

Figure 4.6.1-1: Protocol Stack for AIoT Device

The AIoT Device remains the same and is agnostic to the protocol stacks and routing of information between an AIOT RAN and the AIOTF.

The AIoT RAN may communicate to AIOTF via different paths, see clause 4.2.2.1. The AIoT RAN protocol stack remains the same and is agnostic to the difference paths. The NGAP supports the delivery of AIoT operation (inventory and command) related messages.

- Direct Path: When the AIoT RAN communicates with AIOTF via the direct path, the protocol stack is specified in clause 4.6.2.2.

- Indirect Path: When the AIoT RAN communicates with AIOTF indirectly via an AMF, the protocol stack is specified in clause 4.6.2.3.

4.6.2.2	Protocol Stack between AF and AIoT Device for AIoT RAN Direct Path

Legend:

-	AIoT Reader Control: It is between the AIOTF and AIoT RAN.

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Path option

4.6.2.3	Protocol Stack between AF and AIoT Device for AIoT RAN Indirect Path

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Path via AMF option

In this Protocol Stack, AIoT NAS messages are transferred via the AMF transparently.

Editor’s note: Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

Editor’s note: The reference to AIoT AS Layer protocol will be updated based on RAN1/2 decision.

# * * * * End of changes * * * *