3GPP TSG-SA2 Meeting #166-AdHoc-e	S2-2500513r02

# E-meeting, 20 – 24 January, 2025                  			             			(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0171 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Altitude reporting update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR updates the altitude reporting from the following aspects: - Clarification on the description “the UAS NF/NEF may also include an indication to perform UE's altitude reporting for all aerial UEs in a specific TA(s) or NG-RAN node(s).” Reason: based on description in step 2 in clause 5.16.2, the UAS NF/NEF determines the altitude thresholds based on the flight path of a specifc UAV UE, so the altitude thresholds only apply to one UAV UE. It is unclear how to apply the thresholds determined in step 2 to all aerial UEs in a specifc TA(s) or NG-RAN node(s) considering different aerial UEs have different flight paths. To make the descripton more clear, it is proposed to clarify the UAS NF/NEF determines the altitude thresholds apply to all aerial UEs based on requirements in TS 22.125. Furthermore, when the UAS NF/NEF requests all aerial UEs in a specifc TA(s) or NG-RAN node(s) to report the altitude, it is possible that some of the aerial UEs are in CM-IDLE mode. To support the altitude reporting, it is clarified that the AMF needs to trigger network triggered Service Request procedure for aerial UEs in CM-IDLE mode. - Add description on how does the aerial UE decide the UE's flight altitude meets the provided altitude reporting conditions. Reason: based on the description in setp 3 in clause 5.16.2, 3. Delivering the UAS NF/NEF-derived altitude thresholds and, optionally, the indication about reporting periodicity (i.e. threshold-based, event-based, or periodic) for UE's altitude reporting to the aerial UE(s). the aerial UE will receive the altitude thresholds and, optionally, the indication about reporting periodicity (i.e. threshold-based, event-based or periodic). But it is not clear that how the aerial UE uses the received information to decide that the conditions are met. To make clear the behavior of the aerial UE, the related description is added in clause 5.16.3, as follows: If the altitude thresholds or the altitude thresholds and the reporting periodicity (i.e. threshold based) are received in step 0, the altitude reporting conditions are met when the aerial UE altitude becomes higher or lower than the thresholds. If the altitude thresholds and the reporting periodicity (i.e. event based) are received in step 0, the altitude reporting conditions are met when the aerial UE altitude becomes higher or lower than the thresholds, and the event happens. If the altitude thresholds and reporting periodicity (i.e. periodic based) are received in step 0, the altitude reporting conditions are met when the aerial UE altitude becomes higher or lower than the thresholds, and each time the periodic period is over. - Add a new procedure on altitude reporting cancellation to support the UAS NF/NEF cancels the ongoing altitude reporting. Reason: the UAS NF/NEF can trigger the altitude reporting, but it is not clear how the UAS NF/NEF cancels the reporting, especially for the aerial UE(s) which has reached the destination of the flight path or the aerial UE(s) flying out of the service area of the UAS NF/NEF. To solve the issue, a new procedure on altitude reporting cancellation is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add description on how to request all aerial UEs in a specifc TA(s) or NG-RAN node(s) to perform altitude reporting. - Add description on how does the aerial UE decide the UE's flight altitude meets the provided altitude reporting conditions. - Add a new procedure on altitude reporting cancellation to support the UAS NF/NEF cancels the ongoing altitude reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.1, 5.16.2, 5.16.3, 5.16.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

5.16	Altitude reporting for aerial UEs

5.16.1	General

In order to assist a USS/UTM with aerial UE's flight planning and monitoring, the 5GS may enable configuration of aerial UEs (UAVs) to perform altitude reporting. This may include providing aerial UEs with threshold values for altitude reporting and/or request reporting periodicity (e.g. threshold-based, event-based, periodic). For that, the 5GS may determine altitude thresholds and periodicity with which an aerial UE should report its altitude. Altitude thresholds can be derived, for instance, based on the flight path information provided by the serving USS/UTM during a pre-flight planning request or a USS changeover request. Delivery of the altitude thresholds and reporting periodicity can be done via application layer or via node-level signalling as specified in clause 5.16.2. The 5GS should forward the altitude measurements results to the AMF and NEF/UAS NF in the case of node-level signalling. When the aerial UE's flight altitude meets the altitude reporting conditions, the aerial UE's altitude information is reported as specified in clause 5.16.3.

* * * Second Change * * *

5.16.2	Instructing aerial UEs to perform altitude reporting

Procedure shown in Figure 5.16.2-1 specifies how the 5GS instructs the aerial UEs (UAVs) to perform altitude reporting, considering requests from a USS/UTM for assistance with UAV's flight. This procedure can be used in conjunction with the NEF-assisted pre-flight planning, in-flight monitoring and USS changeover as specified in clauses 5.12.2, 5.12.3 and 5.13, respectively.

Figure 5.16.2-1: Procedure for instructing aerial UEs for altitude reporting

1.	UAS NF/NEF assists the USS/UTM with pre-flight planning, in-flight monitoring and/or USS changeover based on the received request, which is either a Nnef_RetrieveInfoUAVFlight_Get or a Nnef_UAVFlightAssistance_Create, as specified in clauses 5.12 or 5.13.

2.	Based on the flight path information included in step 1 and other information obtained as a result of either of the procedures (i.e. NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2), the UAS NF/NEF determines minimum and maximum values of altitudes (i.e. altitude range) for UAV's flight across each of the determined tracking areas (TAs) from the starting point to the destination point.

The determined minimum and maximum altitude values for each of the UAV UE is used by the UAS NF/NEF to derive altitude thresholds for altitude reporting events.

NOTE 1:	Event H1 and H2 for altitude-based reporting specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [23] can be used for altitude thresholds and event-based reporting.

Additionally, the UAS NF/NEF determines whether to request the aerial UE to report its altitude information periodically, threshold-based (e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided) or event-based (e.g. change of a tracking area/NG-RAN node).

3.	Delivering the UAS NF/NEF-derived altitude thresholds and, optionally, the indication about reporting periodicity (i.e. threshold-based, event-based, or periodic) for UE's altitude reporting to the aerial UE(s).

Option A: Application layer.

3a.	The UAS NF/NEF sends the derived altitude threshold values and provides indication about the required periodicity of the reporting (on demand, event-triggered, periodic, or per change of the altitude, or change of a specific geographical area) to the serving USS/UTM either in a Nnef_RetrieveInfoUAVFlight_Get response message or in a Nnef_UAVFlightAssistance_Notify message depending on the procedure that has triggered the UAS NF/NEF to derive the altitude thresholds for the aerial UE.

4a.	The USS/UTM sends the altitude reporting thresholds and, optionally, indication about reporting periodicity (and any additional information required to perform the reporting, e.g. a list of TA(s)/NG-RAN node identifier(s) for event-based reporting, periodicity value for periodic reporting) to the UAV over the application layer for the UAV's consumption.

NOTE 2:	Application layer reporting itself is outside of 3GPP scope.

Option B: Node-level signalling.

Editor's note:	This requires RAN to provide further enhancement of the procedure under RAN responsibility.

3b.	The UAS NF/NEF may send an Namf_EventExposure_Subscribe request to the AMF to get notified about events of UAV's deviation from the assigned trajectory (i.e. Event ID = "Assigned Trajectory") as specified in clause 5.2.2.3.1 of TS 23.502 [3]. Inside the subscription request, the UAS NF/NEF may include the derived altitude thresholds and indication about reporting periodicity for the UE's altitude reporting.

Additionally, the UAS NF/NEF may also include an indication to perform UE's altitude reporting for all aerial UEs in a specific TA(s) or NG-RAN node(s). In this case, the UAS NF/NEF determines the altitude threshold values applied to all aerial UEs in a specific TA(s) or NG-RAN node(s) basesd on the altitude requirements specified in clause 7.1 in TS 22.125 [5] instead of the flight path and other information described in step 2.

4b.	The AMF sends the corresponding N2 messages to all applicable NG-RAN node(s).

5b.	The NG-RAN node(s) instructs the relevant UE(s) to perform altitude reporting as requested by the UAS NF/NEF (via AMF).

NOTE 3:	Aspects related to AMF informing NG-RAN regarding instructing UEs to perform altitude reporting are specified in TS 38.413 [24].

* * * Third Change * * *

5.16.3	Reporting UE's altitude information

A procedure for reporting aerial UE's altitude information to the core network is shown in Figure 5.16.3-1. The reporting can be done by the aerial UE based on the received altitude reporting thresholds and indication about reporting periodicity as specified in clause 5.16.2.

Figure 5.16.3-1: Procedure for reporting of UE's altitude information

0.	An aerial UE receives request to perform its altitude information reporting as specified in clause 5.16.2. The request may include altitude reporting thresholds and, optionally, indication about reporting periodicity.

1.	UE's flight altitude meets the previously provided altitude reporting conditions (i.e. altitude thresholds and, optionally, reporting periodicity).

Option A: Application layer.

2a.	The UE may report the altitude information over the application layer to the serving USS/UTM in case the USS/UTM has provided altitude reporting threshold and, optionally, indication about reporting periodicity in step 4a in clause 5.16.2.

NOTE 1:	Application layer reporting itself is outside of 3GPP scope.

3a.	The USS may invoke the Nnef_UAVFlightAssistance_Update to inform the UAS NF/NEF about UE's altitude and time stamp of the altitude.

Option B: Node-level signalling.

1a-2b.	The UE may provide a report with the altitude information to the NG-RAN. The report may contain the altitude information and, for instance, a time stamp or a report ID indicating when the UE has measured the altitude information that is being reported.

NOTE 2:	Aspects related to a UE providing altitude information to an NG-RAN node are specified by RAN in TS 38.331 [23].

The NG-RAN node includes the received UE's altitude information in an N2 message to the AMF.

3b.	The AMF provides the UAS NF/NEF the altitude information report inside a Namf_EventExposure_Notify request.

4.	The UAS NF/NEF together with the USS determines the impact on UAV's flight path based on the received altitude information report; this aspect is outside the scope of the 3GPP.

* * * Fourth Change * * *

5.16.x	Altitude reporting cancellation

Procedure shown in Figure 5.16.x-1 specifies how the 5GS cancels altitude reporting. When the aerial UE receives the request, it stops altitude reporting.

Figure 5.16.x-1: Procedure for altitude reporting cancellation

1.	The aerial UE reports the altitude as specified in clauses 5.16.3.

2.	The UAS NF/NEF determines to cancel the altitude reporting based on the flight path information.

Option A: Application layer.

3a.	The UAS NF/NEF invokes Nnef_UAVFlightAssistance_Notify to inform the USS/UTM to cancel the altitude reporting.

4a.	The USS/UTM sends the altitude reporting cancellation indication to the UAV over the application layer.

NOTE X:	Application layer reporting itself is outside of 3GPP scope.

Option B: Node-level signalling.

Editor's note:	This requires RAN to provide further enhancement of the procedure under RAN responsibility.

3b.	The UAS NF/NEF sends an Namf_EventExposure_UnSubscribe request to the AMF to unsubscribe the altitude reporting.

4b.	The AMF sends the corresponding N2 messages to NG-RAN node.

5b.	The NG-RAN node instructs the UE to cancel altitude reporting as requested by the UAS NF/NEF (via AMF).

NOTE Y:	Aspects related to AMF informing NG-RAN regarding instructing UEs to perform altitude reporting are specified in TS 38.413 [24].

* * * End of Changes * * *