3GPP TSG-SA2 Meeting #166-AdHoc-e	S2-2500513r03

# E-meeting, 20 – 24 January, 2025                  			             			(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0171 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Altitude reporting update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR updates the altitude reporting from the following aspects: - Add the introduction of the UE altitude reporting procedure in clause 5.16.1 - Clarify that the USS provides the altitude and the time stamp of altitude to UAS NF/NEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add introduction of UE altitude reporting procedure in clause 5.16.3 in clause 5.16.1. - Clarify that the USS provides altitude and time stamp of altitude to UAS NF/NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.1, 5.16.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

5.16	Altitude reporting for aerial UEs

5.16.1	General

In order to assist a USS/UTM with aerial UE's flight planning and monitoring, the 5GS may enable configuration of aerial UEs (UAVs) to perform altitude reporting. This may include providing aerial UEs with threshold values for altitude reporting and/or request reporting periodicity (e.g. threshold-based, event-based, periodic). For that, the 5GS may determine altitude thresholds and periodicity with which an aerial UE should report its altitude. Altitude thresholds can be derived, for instance, based on the flight path information provided by the serving USS/UTM during a pre-flight planning request or a USS changeover request. Delivery of the altitude thresholds and reporting periodicity can be done via application layer or via node-level signalling as specified in clause 5.16.2. The 5GS should forward the altitude measurements results to the AMF and NEF/UAS NF in the case of node-level signalling. When the aerial UE's flight altitude meets the altitude reporting conditions, the aerial UE's altitude information is reported as specified in clause 5.16.3.

* * * Second Change * * *

5.16.3	Reporting UE's altitude information

A procedure for reporting aerial UE's altitude information to the core network is shown in Figure 5.16.3-1. The reporting can be done by the aerial UE based on the received altitude reporting thresholds and indication about reporting periodicity as specified in clause 5.16.2.

Figure 5.16.3-1: Procedure for reporting of UE's altitude information

0.	An aerial UE receives request to perform its altitude information reporting as specified in clause 5.16.2. The request may include altitude reporting thresholds and, optionally, indication about reporting periodicity.

1.	UE's flight altitude meets the previously provided altitude reporting conditions (i.e. altitude thresholds and, optionally, reporting periodicity).

Option A: Application layer.

2a.	The UE may report the altitude information over the application layer to the serving USS/UTM in case the USS/UTM has provided altitude reporting threshold and, optionally, indication about reporting periodicity in step 4a in clause 5.16.2.

NOTE 1:	Application layer reporting itself is outside of 3GPP scope.

3a.	The USS may invoke the Nnef_UAVFlightAssistance_Update to inform the UAS NF/NEF about UE's altitude and time stamp of the altitude.

Option B: Node-level signalling.

1a-2b.	The UE may provide a report with the altitude information to the NG-RAN. The report may contain the altitude information and, for instance, a time stamp or a report ID indicating when the UE has measured the altitude information that is being reported.

NOTE 2:	Aspects related to a UE providing altitude information to an NG-RAN node are specified by RAN in TS 38.331 [23].

The NG-RAN node includes the received UE's altitude information in an N2 message to the AMF.

3b.	The AMF provides the UAS NF/NEF the altitude information report inside a Namf_EventExposure_Notify request.

4.	The UAS NF/NEF together with the USS determines the impact on UAV's flight path based on the received altitude information report; this aspect is outside the scope of the 3GPP.

* * * End of Changes * * *