3GPP TSG-SA2 Meeting #166-AdHoc-e	S2-2500517

# E-meeting, 20 – 24 January, 2025                  			             			(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5968 | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solve the Editor’s Note related to the Additional ULI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on RAN3 LS reply (S2-2500052/R3-247909), RAN3 conclusion related to the additional ULI is as follows, RAN3 discussed this issue and the following WA was achieved: Additional ULI for WAB consists of TAI and NR CGI, which are determined by the WAB-node, reflecting the WAB-node’s physical location. This solution is compliant with Option 1 and Option 3. It is up to SA2 to support one of Option 1 and Option 3 or both. For option 3, if the MWAB-UE uses the satellite access in the BH PLMN, the cell size is very large (e.g. larger than 100 km). When the AMF in the Broadcasted PLMN receives the cell ID, such large cell size is useless for the AMF to determine UE location information or manages UE location related functions. So option 1 is preferred. Based on the discussion above, this CR is proposed to solve the following Editor’s Note by using option 1. Editor's note: For the case that the BH PLMN and the MWAB broadcasted PLMN are different the exact content and handling of the Additional ULI will be defined based on RAN feedback. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Solve the Editor’s Note related to Additional ULI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsolved Editor’s Note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

### 5.49.4	Support of Additional ULI

The TAC and cell ID broadcasted by the MWAB-gNB are configured and reconfigured e.g. upon MWAB mobility as specified in TS 38.401 [42]. The MWAB-gNB provides these in the User Location Information (ULI) to the AMF serving the UE accessing the MWAB.

The MWAB-gNB shall provide the Additional User Location Information (Additional ULI) that reflects the location of the MWAB-UE to the AMF when it sends the User Location Information (ULI) over N2 messages. In the Additional ULI, the MWAB-gNB provides the TAI/NR CGI of the PLMN/SNPN which the UE is registered as specified in TS 38.401 [42].

NOTE:	For the case of MWAB-UE and MWAB-gNB in different PLMN/SNPN, MWAB-gNB formulates the Additional ULI as specified in TS 38.401 [42]

If serving cell change report is required by AMF as specified in clause 4.10 of TS 23.502 [3], the MWAB-gNB shall report Additional ULI (and ULI) for the UE even when only serving cell of the MWAB-UE changes.

The AMF may consider the 'Additional ULI' when it determines UE location and manages the UE location related functions (e.g. Mobility Restrictions).

Based on operator policy, the AMF may send the Additional ULI when the AMF provides user location information it has received over the N2 interface to a NF (e.g. the LMF as specified in clause 5.19.1 of TS 23.273 [87]).

* * * End of Change * * *