3GPP TSG-WG SA2 Meeting #166 Ad Hoc	S2-2500554r01

# Online, 20th Nov – 24nd Jan, 2025		(revision of S2-241xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1480 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCC enhancements for support of identified non-3GPP devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following scenarios are not currently specified: - The UE changes the Non-3GPP device associated to a local information., e.g. the local IP address is assigned to a different Non-3GPP device - The Non-3GPP devices change the IP address (e.g. due an IP address renewable) or change the ports (e.g. due to the change in NAT when new application starts) - The Non-3GPP device disconnects from the UE (e.g the Non-3GPP device left the LAN or is turned off) - The UE disconnects a Non-3GPP device (e.g. the user decide to disconnect a specific device) or all Non-3GPP devices (e.g. the user turn off the Wifi interface) - The Non-3GPP device identifier is removed from the UE subscription information - The UE sends a Non-3GPP identifier which is not included in UE subscripton (e.g. the user has not added it in the subscription via AF interaction or a misuses in UE) All the above needs to be addressed properly as follow: - In case 1 and 2 the UE shall start a PDU session modification in order to update the PCC rules associated to the binding between the Non-3GPP Device Identifier and the User plane information - In case 3 and case 4 the UE may start a PDU session modification to remove the associated PCC rule. This is optional since the UE may wait some time in case that the disconnection is temporary. For example in case 4 based on implementation the UE may wait for a period of time before starting the PDU session modification to take into account possible temporary loss of connection or the UE (e.g. the application) allows to consider that some Non-3GPP device can always connect? with the same configuration (e.g. due to some static local IP address assignment to a specific device for enabling remote connection or for specific application such as survelliance camera, TV, etc). In case 4 probably the UE will perform immediately the PDU session modification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following specifications are added: - PDU session modification in order to update the PCC rules associated to the binding between the Non-3GPP Device Identifier and the User plane information when the user plane information changes or the Non-3GPP device is disconnected. - a PDU session modification shall be triggered by PCF or SMF for removing the related QoS rules when the Non-3GPP Device Identifier is removed from UE subscription. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong implementation of feature where QoS is not correctly managed for the correct Non-3GPP device due to misuses by UE and impredictable results. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

#### 6.1.3.31	Policy control for non-3GPP devices connecting behind a UE

The support of QoS differentiation for non-3GPP devices connecting behind a UE using a Non-3GPP Device Identifier is specified in clause 5.52 of TS 23.501 [2].

The SMF may receive from the UE the Non-3GPP Device Identifier and, optionally, the corresponding user plane address as specified in clause 4.3.3.2 of TS 23.502 [3]. When a Policy Control Request Trigger is met, the SMF sends the Non-3GPP Device Identifier and the user plane address of the non-3GPP device to the PCF. At reception of the Non-3GPP Device Identifier and user plane address, the PCF creates PCC rule(s) considering the Non-3GPP Device Identifier. The PCF determines QoS parameters in the PCC rule(s) based on the Non-3GPP Device Identifier Information which is retrieved from the UDR and/or based on operator policy. The PCF installs the PCC Rules at the SMF to enable QoS handling of the traffic to/from the non-3GPP device behind the UE.

If the Non-3GPP Device Identifier is not included within the Non-3GPP Device Identifier Information as specified in clause 5.2.12.2.1 of TS 23.502 [3] for the UE, then the PCF indicates to the SMF that the corresponding Non-3GPP Device Identifier is not available for the UE. In case of PDU Session Modification, the PCF shall reject PDU Session Modification. When PDU Session Modification is rejected, a cause code is used to notify the UE that the Non-3GPP Device Identifier is not available for the UE.

The SMF may receive from the UE that a Non-3GPP device does not require QoS differentiation any longer, thenthe SMF provides this information to PCF which removes the PCC Rules associated to that Non-3GPP Device Identifier.

If the Non-3GPP Device Identifier is removed from the UE subscription, the UDR notifies to the PCF via Nudr_DM_Notify service of removal of N3GPP device (as defined in TS 23.502 clauses 4.15.6.7.2 and 4.15.6.15). If there is PCC rules associated to the traffic associated to the removed Non-3GPP Device Identifier in a PDU session, the PCF shall remove the PCC rules and start a PCF intitiated PDU session modification including a cause indicating that the Non-3GPP Device Identifier is not available.

* * * End of Changes * * * *