3GPP TSG-SA2 Meeting #166AH-e	S2-2500559r02

# Electronic meeting, January 20 – 24, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3855 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transitioning between operation in S&F mode and normal mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For 5GGSAT_PH3-ARC, following work task is included in Work Item Exception (2b) – Functional specification of expected UE and network behaviour when transitioning between operation in S&F mode and operation in normal mode As described in TS 23.401, when both the service link and feeder link are available, all services can be provided. However, user plane establishment and user plane data transfer cannot be supported, when operating in S&F Mode in the split MME deployment. The supported features of satellites may change when transition between the S&F and normal operation modes. For the split MME deployment, only Control Plane CIoT EPS Optimisation applies for data transfer when operationg in S&F Mode. However, when both the service link and feeder link are available, user plane data transfer can be applied. The preferred and supported network behaviour is exchanged between the UE and MME during the attach or TAU procedure. If the satellite feeder link availability changes, the supported network behaviour of the MME may need to be updated to the UE. However, there is no triggering of the UE for TAU when switching between the S&F and normal operation modes. We propose that when switching between the S&F and normal operation modes the MME-onboard may decide to trigger the UE to initiate TAU procedure by considering, e.g. the UE's preferred Network Behaviour, UE's usage setting and/or subscription data, to update the supported network behaviour information to the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MME-onboard may trigger S1 release procedure with the existing cause "load balancing TAU required" when transition between the S&F mode and normal mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE and network expected behaviour when transitioning between operation in S&F mode and non-S&F remains undefined and may lead to mis-operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.13.9.X	Transition between S&F and normal operation modes

When switching between the S&F and normal operation modes (from S&F mode to normal mode or from S&F mode to normal mode), the MME may decide to initiate the S1 Release procedure with release cause "load balancing TAU required" (clause 5.3.5), e.g. determined by considering the Network Behaviour the UE can support and/or subscription data. The S1 and RRC connections are released and the UE initiates a TAU.

NOTE X:	The MME can be aware of changes of feeder link availability by implementation.

NOTE Y:	Some deployments may not need this procedure, for example they can support both user plane and control plane data transfer, when operating in S&F Mode in the Full CN deployment described in Annex O.

NOTE Z: Based on the operator policy or regulation, the satellite may not change the mode (e.g. still operate in S&F after feeder link become available).

* * * * End of Changes * * * *