3GPP TSG-WG2 Meeting #166-Ad Hoc-e	S2-2500588r03

# 20 - 24 January, 2025, Online		(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0650 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of input data collection from LMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, ZTE, Apple, China Telecom?, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. Considering that the procedure in clause 6.22.4 can be reused by the NWDAF to collect input data for performing ML model performance monitoring, as described in clause 6.2E.4 of TS 23.288, so the following EN can be removed. Editor's note: It’s FFS how to adapt this procedure to ML model performance monitoring. 2. As described in clause 6.2E.4 of TS 23.288, to perform ML model performance monitoring, the NWDAF may need to collect the input data of PRU(s), so the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF. Therefore, the following EN is removed, and the PRU related descriptions are added in the procedure of input data collection from LMF and Nlmf_DataExposure service operations. Editor's note: Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. 3. There are different names for the two parameters, e.g., “requested number for data samples”, “number of samples”, “time window for data collection”, “time window when to collect data”, and the meaning of “time window for data collection” is unclear. To make alignment for the name of these two parameters, we change it to “requested number of data samples” and “Data time window of data samples”, and refer to the definition of Data time window in TS 23.288, we add the description for “Data time window of data samples”, i.e., indicating that only data samples that have been collected in the specified time interval are required. 4. When the “requested number of data samples” or “Data time window of data samples” was provided by the NWDAF, the LMF may not be able to provide data samples that can meet the requirements indicated by these two parameters. For example, the LMF cannot collect enough data samples, or the LMF cannot collect data samples within the Data time window (e.g., due to the NG-RAN is overloaded and cannot provide the data samples to the LMF during the Data time window). Therefore, the LMF may provide a cause code to the NWDAF when the requested number of data samples and/or Data time window of data samples cannot be met. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Remove the following EN. Editor's note: It’s FFS how to adapt this procedure to ML model performance monitoring. 2. Remove the following EN, and the PRU related descriptions are added in the procedure of input data collection from LMF and Nlmf_DataExposure service operations. Editor's note: Whether the NWDAF needs to be aware of PRU information and indicate to obtain the PRU information from LMF is FFS. 3. Make alignment for the name of “requested number of data samples” and “Data time window of data samples”, and clarify the meaning of “Data time window of data samples”. 4. Add a cause code to the step 5 of clause 6.22.4 and Nlmf_DataExposure_Notify service operation. 5. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ENs regarding data collection from LMF still exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.4, 8.3.4.2, 8.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (new) * * * *

### 6.22.4	Input data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring

The NWDAF containing MTLF may subscribe to input data (i.e. location measurement related data) from LMF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Figure 6.22.4-1: Procedure of input data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning based on the request from LMF or internal trigger, or the NWDAF containing MTLF determines to perform ML model performance monitoring for LMF-based AI/ML Positioning.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover an LMF, the service operation includes an AoI and the Nlmf_DataExposure service as discovery parameters. If the NWDAF wants to collect the input data of PRUs, the NWDAF may also include a PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI, the Nlmf_DataExposure service, and the PRU existence indication (if available), and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

Editor's note:	The input data used for LMF-based AI/ML Positioning will be decided by RAN WGs.

3.	The NWDAF subscribes to or cancels subscription to input data from LMF by invoking Nlmf_DataExposure_Subscribe / Nlmf_DataExposure_UnSubscribe service operation. The NWDAF includes an AoI and a notification target address to request the input data from LMF. The NWDAF may also include requested number of data samples,  a highest label quality indication (indicating to provide the requested f data samples with highest label quality), and Data time window of data samples., Data source type (UE or RAN measurements)  The NWDAF may also include an PRU indication to indicate obtaining the PRU information if the NWDAF wants to collect the Location Measurement related data of PRUs.

Editor's note:	Whether the NWDAF needs to indicate and/or data type(s) is also FFS.

4.	The LMF performs steps 1 to 5 in clause 6.22.2, or steps 1 to 7 in clause 6.22.3.

5.	The LMF sends the collected data samples to the NWDAF by invoking Nlmf_DataExposure_Notify service operation, the data samples include the PRU location measurement(s) and associated PRU known location if the PRU indication is indicated in step 3, then the NWDAF trains the ML model or performs ML model performance monitoring based on the data samples. The LMF may send a cause code to the NWDAF when the requested number of data samples and/or Data time window of data samples cannot be met.

# * * * * Next change * * * *

#### 8.3.4.2	Nlmf_DataExposure_Subscribe service operation

Service operation name: Nlmf_DataExposure_Subscribe

Description: Allow the consumer NF to subscribe the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Target Address, Notification Correlation ID, AoI.

Input, Optional: Data time window of data samples (indicating that only data samples that have been collected in the specified time interval are required), requested number of data samples, highest label quality indication, PRU indication, Subscription Correlation ID (in the case of modification of the subscription). Data source type (UE or RAN measurements)

Editor's note:	Whether the consumer NF (i.e. NWDAF) needs to indicate the and/or data type(s) is FFS, which needs to be coordinated with RAN WGs.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

When the requested number of data samples has been provided or the Data time window of data samples has elapsed, the subscription is automatically terminated.

# * * * * Next change * * * *

#### 8.3.4.3	Nlmf_DataExposure_Notify service operation

Service operation name: Nlmf_DataExposure_Notify

Description: Allow the consumer NF to get notification about the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Input, Required: Notification Correlation ID.

Input, Optional: Data samples (i.e. location measurement related data), Subscription Correlation ID (this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation ID by the LMF), cause code (e.g., insufficient UEs/PRUs in the AoI, revocation of user consent, NG-RAN overload during the requested Data time window, etc.).

Editor's note:	The data samples used for LMF-based AI/ML Positioning will be decided by RAN WGs.

Output, Required: Operation execution result indication.

Output, Optional: None.

# * * * * End of changes * * * *