3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500614r01

# E-meeting, 20 – 24 January 2025		(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5982 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the UE registration in the INS case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_NetShare |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the UE serves by the same PLMN for 3GPP and non-3GPP access, the same AMF is expected to be registered. In the INS case, the UE indeed is registered to the AMF in the PLMN of hosting operator. It is not expected that N3IWF/TNGF/W-AGF of participating operator(HPLMN or EHPLMN) can connect to AMF in hosting operator as this is strong deployment restriction to operator. Hence if UE registers to AMF1 in the INS area via 3GPP access(hosting operator) but it can not register to the same AMF1 via non-3GPP access as it is unreachable for N3IWF/TNGF/W-AGF in participating operator. To solve this issue: - Option 1: support UE registers to AMF2(non-3GPP access) and still one AMF serves two access, i.e. 3GPP access registration also kept at the AMF2. The UDM need cancel the UE registration to AMF1(3GPP access). But it is impossible for AMF2 to connect to NG-RAN in hosting operator. So this means all ongoing data transferred at the 3GPP access need be stopped. And if UE triggers Service Request again in 3GPP access, another registration via 3GPP access need be triggered. This triggers the ping-pong registration among different AMFs. - Option 2: support UE registers to AMF2(non-3GPP access) and not require that in the same PLMN same AMF is to selected, i.e. 3GPP access registration and non-3GPP access registration are managed at different AMF separately. As UE is not aware INS, it use the same GUTI(allocated by AMF2) in 3GPP access. So if UE trigger Service Request in 3GPP access again, it will always trigger re-registration and UE context movement/cancel registration among different AMFs, i.e. same ping-poing registration among different AMFs as option 1. Above two options also nees consider how to solve the MT issue as the AMF2(non-3GPP access) can not connect to the NG-RAN in the INS area and the new GUTI has been allocated by AMF2. Hence the paging in 3GPP access is impossible(for option 1) or responsed(for option 2) by UE. - Option 3: not allow UE to register to non-3GPP access. This give some usage restriction. However this is the minimum impact to solve this issue and solve the current operator’s urgent requirement(focus on the 3GPP access), it is a acceptable approach. Per above consideration, if the UE registers to 5GS via 3GPP access in the INS area first and trigger registration via non-3GPP access, it is suggested to go with option 3. If the UE registers to 5GS via non-3GPP access first, then it registers to 5GS in the INS area via 3GPP access, the registration is allowed. Also the AMF2 can register itself as serving AMF for non-3GPP access. However as the AMF in the hosting operator is impossible to update the N3IWF/TNGF/W-AGF UE context. It is suggested that the AMF1(non-3GPP access) delete the UE context in the N3IWF/TNGF/W-AGF per the deregistration notification from UDM. In the updated paper, this paper focus on the UE register to non-3GPP first, then move to 3GPP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If UE are served by the 3GPP access and the non-3GPP access via the same HPLMN or EHPLMN, the registration is handed as following: - If UE registers in non-3GPP access first and move to 3GPP access in the INS area, the AMF in 3GPP access can registere to UDM as serving AMF for non-3GPP access. The old AMF in the non-3GPP access remove the UE contex at N3IWF/TNGF/W-AGF per the deregistration notification from UDM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how to handle UE registration to the same PLMN via 3GPP access and non-3GPP access if the INS is supported in the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.18.1	General concepts

A network sharing architecture shall allow multiple participating operators to share resources of a single shared network according to agreed allocation schemes. The shared network includes a radio access network. The shared resources include radio resources.

The shared network operator allocates shared resources to the participating operators based on their planned and current needs and according to service level agreements.

In this Release of the specification, the 5G Multi-Operator Core Network (5G MOCN) network sharing architecture, in which only the RAN is shared in 5G System, is supported. The 5G System may also support Indirect Network Sharing deployment between hosting operator (i.e. shared network operator) and participating operator (see clause 6.21 of TS 22.261 [2], Figure 5.18.1-2 and Annex R), in which the RAN is shared. The communication between the shared RAN and the core network of the participating operator is routed through the core network of the hosting operator that connects to the shared RAN.

5G MOCN for 5G System, including UE, RAN and AMF, shall support operators' ability to use more than one PLMN ID (i.e. with same or different country code (MCC) some of which is specified in TS 23.122 [17] and different network codes (MNC)) or combinations of PLMN ID and NID. 5G MOCN supports NG-RAN Sharing with or without multiple Cell Identity broadcast as described in TS 38.300 [27]. Indirect Network Sharing for 5G system, including UE, shared RAN and CP NFs of hosting operator, shall support each participating operator to use more than one PLMN ID.

For Indirect Network Sharing, the shared RAN broadcasts multiple PLMN IDs, including the PLMN ID which represents the hosting operator and the PLMN IDs which represent participating operators. Multiple PLMN IDs are supported by the serving AMF (i.e. the AMF in the core network of the hosting operator). A UE from a participating operator can select the PLMN ID representing the participating operator in the shared RAN area based on existing procedures specified in TS 23.122 [17]. The serving AMF selects core network functions in the PLMN of the participating operator for the UE, based on home routed roaming architecture principle as specified in clause 4.2.4. In addition, the serving AMF selects the SMF of participating operator possibly considering UE location information and also selects a V-SMF in its own network during the PDU session establishment procedure. The serving PLMN ID used in the procedures is determined as follows:

-	For interaction between the Network Functions in the hosting operator network and the participating operator network (e.g. AUSF, UDM), the NF of hosting operator network sets the serving PLMN ID to the selected PLMN ID. This includes the cases when the interaction is via another NF (e.g. V-PCF for UE policy, V-SMF) in the hosting operator network.

-	For interactions between the Network Functions only in the hosting operator network (e.g. V-PCF for AM policy), the NF of hosting operator network sets the serving PLMN ID to the PLMN ID of the hosting operator.

NOTE 1:	In this Release of specification, the Indirect Network Sharing is only applicable for NR with 5GC of hosting operator and 5GC of participating operator. There are maximum of two SMFs (i.e. V-SMF of the hosting operator and H-SMF of the participating operator) controlling a PDU session.

If the UE is served by the 3GPP access via Indirect Network Sharing, where the shared RAN broadcasts the HPLMN, and by the non-3GPP access with the same HPLMN, the registration is handed as following:

- When the new AMF in the hosting operator receives the UE registration via 3GPP access, the new AMF checks whether the UE is registered in the same PLMN via non-3GPP access before and N3IWF/TNGF/W-AGF information is included in the retrieval UE context. If yes, the new AMF registers itself to the UDM for non-3GPP access but not update the UE context at the N3IWF/TNGF/W-AGF. The UE context at the N3IWF/TNGF/W-AGF is released by the old AMF per the deregistration notification from UDM.

5G MOCN also supports the following sharing scenarios involving non-public networks, i.e.NG-RAN can be shared by any combination of PLMNs, PNI-NPNs (with CAG) and SNPNs (each identified by PLMN ID and NID).

NOTE 2:	PNI-NPNs (without CAG) are not explicitly listed above as it does not require additional NG-RAN sharing functionality compared to sharing by one or multiple PLMNs.

In all non-public network sharing scenarios, each Cell Identity as specified in TS 38.331 [28] is associated with one of the following configuration options:

-	one or multiple SNPNs;

-	one or multiple PNI-NPNs (with CAG); or

-	one or multiple PLMNs only.

NOTE 3:	This allows the assignment of multiple cell identities to a cell and also allows the cell identities to be independently assigned, i.e. without need for coordination, by the network sharing partners, between PLMNs and/or non-public networks.

NOTE 4:	Different PLMN IDs (or combinations of PLMN ID and NID) can also point to the same 5GC. When same 5GC supports multiple SNPNs (identified by PLMN ID and NID), it is up to the operator's policy whether they are used as equivalent SNPNs for a UE.

NOTE 5:	There is no standardized mechanism to avoid paging collisions if the same 5G-S-TMSI is allocated to different UEs by different PLMNs or SNPNs of the shared network, as the risk of paging collision is assumed to be very low. If such risk is to be eliminated then PLMNs and SNPNs of the shared network needs to coordinate the value space of the 5G-S-TMSI to differentiate the PLMNs and SNPNs of the shared network.

Figure 5.18.1-1: A 5G Multi-Operator Core Network (5G MOCN) in which multiple CNs are 
connected to the same NG-RAN

Figure 5.18.1-2: Indirect Network Sharing in which multiple participating operators' CNs connect to hosting operator's CN to share NR

NOTE 6:	Not all interfaces between the hosting operator and the participating operator are depicted in the Figure 5.18.1-2 for simplicity.

NOTE 7:	For the sake of clarity, SEPPs are not depicted in the Figure 5.18.1-2.

# * * * * End of changes * * * *