3GPP TSG-SA2 Meeting #166AH-e	S2-2500615r01

# 20th– 24th January, 2025, E-meeting

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5983 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on steering functionalities description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current specification of Clause 5.32.6.1 describes steering functionalities in combination of the followings: 1) ATSSS-LL functionality with any steering mode. 2) MPTCP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. 3) MPQUICUDP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. 4) MPQUIC IP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. 5) MPQUIC E functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. 6) MPQUIC IP functionality with any steering mode without any ATSSS-LL functionality. “MPQUIC-E functionality with any steering mode without any ATSSS-LL functionality” is a necessary option to establish Ethernet type MA PDU Session for a UE that doesn’t support ATSSS-LL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following to UE’s ATSSS capability. 7) MPQUIC-E functionality with any steering mode without any ATSSS-LL functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GS cannot support an Ethernet type MA PDU Session for a UE that doesn’t support ATSSS-LL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.2, 5.32.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Add 5.32.2 modification. |  |  |  |  |  |  |  |  |

*** Start of Changes ***

5.32.2	Multi Access PDU Sessions

A Multi-Access PDU (MA PDU) Session is managed by using the session management functionality specified in clause 5.6, with the following additions and modifications:

-	When the UE wants to request a new MA PDU Session:

-	If the UE is registered to the same PLMN over 3GPP and non-3GPP accesses, then the UE shall send a PDU Session Establishment Request over any of the two accesses. The UE also provides Request Type as "MA PDU Request" in the UL NAS Transport message. The AMF informs the SMF that the UE is registered over both accesses and this triggers the establishment of user-plane resources on both accesses and two N3/N9 tunnels between PSA and the RAN/AN.

-	If the UE is registered to different PLMNs over 3GPP and non-3GPP accesses, then the UE shall send a PDU Session Establishment Request over one access. The UE also provides Request Type as "MA PDU Request" in the UL NAS Transport message. After this PDU Session is established with one N3/N9 tunnel between the PSA and (R)AN established, the UE shall send another PDU Session Establishment Request over the other access. The UE also provides the same PDU Session ID and Request Type as "MA PDU Request" in the UL NAS Transport message. Two N3/N9 tunnels and User-plane resources on both accesses are established.

-	If the UE is registered over one access only, then the UE shall send a PDU Session Establishment Request over this access. The UE also provides Request Type as "MA PDU Request" in the UL NAS Transport message. One N3/N9 tunnel between the PSA and (R)AN and User-plane resources on this access only are established. After the UE is registered over the second access, the UE shall establish user-plane resources on the second access.

-	In the PDU Session Establishment Request that is sent to request a new MA PDU Session, the UE shall provide also its ATSSS capabilities, which indicate the steering functionalities and the steering modes supported in the UE. These functionalities are defined in clause 5.32.6.

-	If the UE indicates it is capable of supporting:

-	the "ATSSS-LL functionality with any steering mode" (as specified in clause 5.32.6.1);

and the network accepts to activate this functionality, then the network may provide to UE Measurement Assistance Information (see details in clause 5.32.5) and shall provide to UE one or more ATSSS rules.

NOTE 1:	As specified in Table 5.32.8-1 and in Table 5.8.5.8-1, the ATSSS-LL functionality cannot be used together with the Redundant steering mode. When the UE indicates it is capable of supporting the ATSSS-LL functionality with any steering mode, it is implied that the UE can support the ATSSS-LL functionality with any steering mode except the Redundant steering mode.

-	If the UE indicates it is capable of supporting:

-	the "MPTCP functionality with any steering mode and the ATSSS-LL functionality with only the Active-Standby steering mode (as specified in clause 5.32.6.1);

and the network accepts to activate these functionalities, then the network provides MPTCP proxy information to UE and allocates to UE (a) one IP address/prefix for the MA PDU session (as defined in clause 5.8.2.2) and (b) two additional IP addresses/prefixes, called "MPTCP link-specific multipath" addresses. Further details are provided in clause 5.32.6.2.1. In addition, the network may provide to UE Measurement Assistance Information and shall provide to UE one or more ATSSS rules. If the UE supports the ATSSS-LL functionality with only the Active-Standby steering mode, the network shall provide to UE an ATSSS rule for non-MPTCP traffic. The ATSSS rule for non-MPTCP traffic shall use the ATSSS-LL functionality and the Active-Standby Steering Mode to indicate how the non-MPTCP traffic shall be transferred across the 3GPP access and the non-3GPP access in the uplink direction.

If the network accepts to activate only ATSSS-LL functionality with only the Active-Standby steering mode, the network shall provide to the UE an ATSSS rule that uses the ATSSS-LL functionality and the Active-Standby Steering Mode, to indicate how the traffic shall be transferred across the 3GPP access and the non-3GPP access in the uplink direction.

-	If the UE indicates it is capable of supporting

-	the "MPQUIC-UDP functionality with any steering mode and the ATSSS-LL functionality with only the Active-Standby steering mode" (as specified in clause 5.32.6.1);

and the network accepts to activate these functionalities, then the network provides MPQUIC-UDP proxy information to UE and allocates to UE (a) one IP address/prefix for the MA PDU session (as defined in clause 5.8.2.2) and (b) two additional IP addresses/prefixes, called "MPQUIC link-specific multipath" addresses. Further details are provided in clause 5.32.6.2.2. In addition, the network may provide to UE Measurement Assistance Information and shall provide to UE one or more ATSSS rules. If the UE supports the ATSSS-LL functionality with only the Active-Standby steering mode, the network shall provide to UE an ATSSS rule for non-MPQUIC traffic. The ATSSS rule for non-MPQUIC traffic shall use the ATSSS-LL functionality and the Active-Standby Steering Mode to indicate how the non-MPQUIC traffic shall be transferred across the 3GPP access and the non-3GPP access in the uplink direction.

If the network accepts to activate only ATSSS-LL functionality with only the Active-Standby steering mode, the network shall provide to the UE an ATSSS rule that uses the ATSSS-LL functionality and the Active-Standby Steering Mode, to indicate how the traffic shall be transferred across the 3GPP access and the non-3GPP access in the uplink direction.

-	If the UE indicates it is capable of supporting:

-	the "MPQUIC-IP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" (as specified in clause 5.32.6.1);

and the network accepts to activate these functionalities, then the network provides MPQUIC-IP proxy information to UE and allocates to UE (a) one IP address/prefix for the MA PDU session (as defined in clause 5.8.2.2) and (b) two additional IP addresses/prefixes, called "MPQUIC link-specific multipath" addresses. Further details are provided in clause 5.32.6.2.2. In addition, the network may provide to UE Measurement Assistance Information and shall provide to UE one or more ATSSS rules.

If the network accepts to activate only ATSSS-LL functionality with only the Active-Standby steering mode, the network shall provide to the UE an ATSSS rule that uses the ATSSS-LL functionality and the Active-Standby Steering Mode, to indicate how the traffic shall be transferred across the 3GPP access and the non-3GPP access in the uplink direction.

-	If the UE indicates it is capable of supporting:

-	the "MPQUIC-E functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" (as specified in clause 5.32.6.1);

and the network accepts to activate MPQUIC-E functionality, then the network provides MPQUIC-E proxy information to UE and allocates to UE two IP addresses/prefixes, called "MPQUIC link-specific multipath" addresses. Further details are provided in clause 5.32.6.2.2. In addition, the network shall provide to UE one or more ATSSS rules.

If the network accepts to activate only ATSSS-LL functionality with only the Active-Standby steering mode, the network shall provide to the UE an ATSSS rule that uses the ATSSS-LL functionality and the Active-Standby Steering Mode, to indicate how the traffic shall be transferred across the 3GPP access and the non-3GPP access in the uplink direction. In addition, the network may provide to UE Measurement Assistance Information.

-	If the UE indicates it is capable of supporting:

-	the "MPQUIC IP functionality with any steering mode without any ATSSS-LL functionality" (as specified in clause 5.32.6.1);

and the network accepts to activate MPQUIC-IP functionality, then the network provides MPQUIC-IP proxy information to UE and allocates to UE (a) one IP address/prefix for the MA PDU session (as defined in clause 5.8.2.2) and (b) two additional IP addresses/prefixes, called "MPQUIC link-specific multipath" addresses. In addition, the network shall provide to UE one or more ATSSS rules.

-	If the UE indicates it is capable of supporting:

-	the "MPQUIC-E functionality with any steering mode without any ATSSS-LL functionality" (as specified in clause 5.32.6.1);

and the network accepts to activate MPQUIC-E functionality, then the network provides MPQUIC proxy information to UE and allocates to UE two IP addresses/prefixes, called "MPQUIC link-specific multipath" addresses. Further details are provided in clause 5.32.6.2.2. In addition, the network shall provide to UE one or more ATSSS rules.

NOTE 2:	The "MPTCP link-specific multipath" addresses and the "MPQUIC link-specific multipath" addresses can be the same.

-	If the UE requests an S-NSSAI, this S-NSSAI should be allowed on both accesses. Otherwise, the MA PDU Session shall not be established.

-	The SMF determines the ATSSS capabilities supported for the MA PDU Session taking into consideration one or more of the following aspects based on the ATSSS capabilities provided by the UE and per DNN configuration on SMF:

a)	If the UE includes in its ATSSS capabilities "MPTCP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" (as specified in clause 5.32.6.1), then:

i)	If the DNN configuration allows MPTCP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL), including RTT measurement without using PMF protocol, the MA PDU Session is capable of (1) MPTCP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL) in the downlink and (2) MPTCP and ATSSS-LL with Active-Standby mode in the uplink.

NOTE 3:	In this case, it is assumed that ATSSS-LL with "Smallest Delay" steering mode is selected for the downlink only when the UPF can measure RTT without using the PMF protocol, e.g. by using other means not defined by 3GPP such as using the RTT measurements of MPTCP.

ii)	If the DNN configuration allows MPTCP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL), but not RTT measurement without using PMF protocol, the MA PDU Session is capable of (1) MPTCP in the downlink (2) ATSSS-LL with any steering mode except Smallest Delay steering mode (i.e. any Steering Mode allowed for ATSSS-LL except Smallest Delay steering mode) in the downlink and (3) MPTCP and ATSSS-LL with Active-Standby mode in the uplink.

iii)	If the DNN configuration allows MPTCP with any steering mode and ATSSS-LL with only Active-Standby steering mode, the MA PDU Session is capable of MPTCP and ATSSS-LL with Active-Standby mode in the uplink and in the downlink.

iv)	If the DNN configuration does not allow MPTCP steering functionality and the DNN configuration allows ATSSS-LL functionality with at least the Active-Standby steering mode, then the MA PDU Session is capable of ATSSS-LL with only Active-Standby steering mode in the uplink and in the downlink.

b)	If the UE includes in its ATSSS capabilities "MPQUIC-UDP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" (as specified in clause 5.32.6.1), then:

i)	If the DNN configuration allows MPQUIC-UDP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL), including RTT measurement without using PMF protocol, the MA PDU Session is capable of (1) MPQUIC-UDP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL) in the downlink and (2) MPQUIC-UDP and ATSSS-LL with Active-Standby mode in the uplink.

NOTE 4:	In this case, it is assumed that ATSSS-LL with "Smallest Delay" steering mode is selected for the downlink only when the UPF can measure RTT without using the PMF protocol, e.g. by using other means not defined by 3GPP such as using the RTT measurements of MPQUIC.

ii)	If the DNN configuration allows MPQUIC-UDP and ATSSS-LL with any steering mode (i.e. any Steering Mode allowed for ATSSS-LL), but not RTT measurement without using PMF protocol, the MA PDU Session is capable of (1) MPQUIC-UDP in the downlink (2) ATSSS-LL with any steering mode except Smallest Delay steering mode (i.e. any Steering Mode allowed for ATSSS-LL except Smallest Delay steering mode) in the downlink and (3) MPQUIC-UDP and ATSSS-LL with Active-Standby mode in the uplink.

iii)	If the DNN configuration allows MPQUIC-UDP with any steering mode and ATSSS-LL with only Active-Standby steering mode, the MA PDU Session is capable of MPQUIC-UDP and ATSSS-LL with Active-Standby mode in the uplink and in the downlink.

iv)	If the DNN configuration does not allow MPQUIC-UDP steering functionality and the DNN configuration allows ATSSS-LL functionality with at least the Active-Standby steering mode, then the MA PDU Session is capable of ATSSS-LL with only Active-Standby steering mode in the uplink and in the downlink.

c)	If the UE includes in its ATSSS capabilities "ATSSS-LL functionality with any steering mode" (as specified in clause 5.32.6.1) and the DNN configuration allows ATSSS-LL with any steering mode allowed for ATSSS-LL, the MA PDU Session is capable of ATSSS-LL with any steering mode allowed for ATSSS-LL in the uplink and in the downlink.

d)	If the UE includes in its ATSSS capabilities "MPQUIC IP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" (as specified in clause 5.32.6.1) and the DNN configuration allows MPQUIC IP functionality with any steering mode, the MA PDU Session is capable of MPQUIC IP with any steering mode in the uplink and in the downlink. If the DNN configuration does not allow MPQUIC-IP steering functionality and the DNN configuration allows ATSSS-LL functionality with at least the Active-Standby steering mode, then the MA PDU Session is capable of ATSSS-LL with only Active-Standby steering mode in the uplink and in the downlink.

e)	If the UE includes in its ATSSS capabilities "MPQUIC E functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" (as specified in clause 5.32.6.1) and the DNN configuration allows MPQUIC E functionality with any steering mode, the MA PDU Session is capable of MPQUIC E with any steering mode in the uplink and in the downlink. If the DNN configuration does not allow MPQUIC-E steering functionality and the DNN configuration allows ATSSS-LL functionality with at least the Active-Standby steering mode, then the MA PDU Session is capable of ATSSS-LL with only Active-Standby steering mode in the uplink and in the downlink.

f)	If the UE includes in its ATSSS capabilities "MPQUIC IP functionality with any steering mode without any ATSSS-LL functionality" (as specified in clause 5.32.6.1) and the DNN configuration allows MPQUIC IP functionality with any steering mode, the MA PDU Session is capable of MPQUIC IP with any steering mode in the uplink and in the downlink. If the DNN configuration does not allow MPQUIC-IP steering functionality, then the SMF rejects the MA PDU Session request.

NOTE 5:	As described above, an example of how SMF determines the ATSSS capabilities of a MA PDU Session is as follows:

If the UE indicates that it supports "MPTCP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" and "MPQUIC-UDP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" and "MPQUIC IP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode" and the DNN configuration allows only MPTCP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode, then the MA PDU Session is capable of MPTCP and ATSSS-LL with Active-Standby mode in the uplink and in the downlink.

g)	If the UE includes in its ATSSS capabilities "MPQUIC-E functionality with any steering mode without any ATSSS-LL functionality" (as specified in clause 5.32.6.1) and the DNN configuration allows MPQUIC-E functionality with any steering mode, the MA PDU Session is capable of MPQUIC-E with any steering mode in the uplink and in the downlink. If the DNN configuration does not allow MPQUIC-E steering functionality, then the MA PDU Session is not capable of MPQUIC-E.

NOTE 6:	MPQUIC E functionality is enabled only when the type of the MA PDU Session is Ethernet. The MPTCP, MPQUIC UDP and MPQUIC IP functionalities are not enabled when the type of the MA PDU Session is Ethernet.

For ATSSS-LL with only Active-Standby steering mode, the UE and the UPF needs to support Access Availability/Unavailability Report as specified in clause 5.32.5.3.

The SMF provides the ATSSS capabilities of the MA PDU Session to the PCF during PDU Session Establishment.

-	The PCC rules provided by PCF include MA PDU Session Control information (see TS 23.503 [45]). They are used by SMF to derive ATSSS rules for the UE and N4 rules for the UPF. When dynamic PCC is not used for the MA PDU Session, the SMF shall provide ATSSS rules and N4 rules based on local configuration (e.g. based on DNN or S-NSSAI).

-	The UE receives ATSSS rules from SMF, which indicate how the uplink traffic should be routed across 3GPP access and non-3GPP access. Similarly, the UPF receives N4 rules from SMF, which indicate how the downlink traffic should be routed across 3GPP access and non-3GPP access.

-	When the SMF receives a PDU Session Establishment Request and a "MA PDU Request" indication and determines that UP security protection (see clause 5.10.3) is required for the PDU Session, the SMF shall only confirm the establishment of the MA PDU session if the 3GPP access network can enforce the required UP security protection. The SMF needs not confirm whether the non-3GPP access can enforce the required UP security protection.

-	The UE indicates during MA PDU Session Establishment to the AMF whether it supports non-3GPP access path switching, i.e. whether the UE can transfer the non-3GPP access path of the MA PDU Session from a source non-3GPP access (N3IWF/TNGF) to a target non-3GPP access (a different N3IWF/TNGF). If the UE has indicated support for non-3GPP access path switching and the AMF supports non-3GPP access path switching, the AMF selects an SMF that supports non-3GPP access path switching, if such an SMF is available. If the AMF supports to maintain two N2 connections for non-3GPP access during the Registration procedure and the selected SMF supports non-3GPP path switching, the AMF indicates whether the UE supports non-3GPP path switching to the SMF. The SMF indicates support for non-3GPP path switching to the UE in the PDU Session Establishment Accept message.

NOTE 7:	If the AMF selects an SMF not supporting non-3GPP access path switching, the non-3GPP access path switching can still be performed with the AMF triggering release of the old user plane resources before new user plane resources are established.

-	After the MA PDU Session establishment:

-	At any given time, the MA PDU session may have user-plane resources on both 3GPP and non-3GPP accesses, or on one access only, or may have no user-plane resources on any access.

-	The AMF, SMF, PCF and UPF maintain their MA PDU Session contexts, even when the UE deregisters from one access (but remains registered on the other access).

-	When the UE deregisters from one access (but remains registered on the other access), the AMF informs the SMF to release the resource of this access type in the UPF for the MA PDU Session. Subsequently, the SMF notifies the UPF that the access type has become unavailable and the N3/N9 tunnel for the access type are released.

-	If the UE wants to add user-plane resources on one access of the MA PDU Session, e.g. based on access network performance measurement and/or ATSSS rules, then the UE shall send a PDU Session Establishment Request over this access containing PDU Session ID of the MA PDU Session. The UE also provides Request Type as "MA PDU Request" and the same PDU Session ID in the UL NAS Transport message. If there is no N3/N9 tunnel for this access, the N3/N9 tunnel for this access is established.

-	If the UE wants to re-activate user-plane resources on one access of the MA PDU Session, e.g. based on access network performance measurement and/or ATSSS rules, then the UE shall initiate the UE Triggered Service Request procedure over this access.

-	If the network wants to re-activate the user-plane resources over 3GPP access or non-3GPP access of the MA PDU Session, the network shall initiate the Network Triggered Service Request procedure, as specified in clause 4.22.7 of TS 23.502 [3].

-	If the UE wants to move the non-3GPP user-plane resources of the MA PDU Session from a source non-3GPP access (e.g. source N3IWF or TNGF) to a target non-3GPP access (e.g. target N3IWF or TNGF), the UE initiates a Mobility Registration Update via the target non-3GPP access as described in TS 23.502 [3], clause 4.22.9.5. This procedure may also be used to move the non-3GPP user-plane resources of single access PDU Session(s).

NOTE 8:	The UE can request activation of single access PDU Session(s) over the target non-3GPP access while performing Mobility Registration Update procedure according to the existing procedure.

-	The SMF may add, remove or update one or more individual ATSSS rules of the UE by sending new or updated ATSSS rules with the corresponding Rule IDs to the UE.

A MA PDU Session may be established either:

a)	when it is explicitly requested by an ATSSS-capable UE; or

b)	when an ATSSS-capable UE requests a single-access PDU Session but the network decides to establish a MA PDU Session instead. This is an optional scenario specified in clause 4.22.3 of TS 23.502 [3], which may occur when the UE requests a single-access PDU Session but no policy (e.g. no URSP rule) and no local restrictions in the UE mandate a single access for the PDU Session.

A MA PDU Session may be established during a PDU Session modification procedure when the UE moves from EPS to 5GS, as specified in clause 4.22.6.3 of TS 23.502 [3].

The AMF indicates as part of the Registration procedure whether ATSSS is supported or not. When ATSSS is not supported, the UE shall not

-	request establishment of a MA PDU Session (as described in clause 4.22.2 of TS 23.502 [3]); or

-	request addition of User Plane resources for an existing MA PDU Session (as described in clause 4.22.7 of TS 23.502 [3]); or

-	request establishment of a PDU Session with "MA PDU Network-Upgrade Allowed" indication (as described in clause 4.22.3 of TS 23.502 [3]); or

-	request PDU Session Modification with Request Type of "MA PDU request" or with "MA PDU Network-Upgrade Allowed" indication after moving from EPC to 5GC (as described in clause 4.22.6.3 of TS 23.502 [3]).

The AMF indicates as part of the Registration procedure whether it supports non-3GPP access path switching. When the AMF does not indicate support of non-3GPP access path switching, the UE shall not perform the Mobility Registration Update procedure for non-3GPP access path switching, i.e. to switch traffic from a source non-3GPP access to a target non-3GPP access. The SMF indicates as part of the PDU Session Establishment procedure whether it supports non-3GPP access path switching. If the UE has one or more PDU sessions and at least one serving SMF for the PDU Sessions supports non-3GPP access path switching, the UE may include ("Non-3GPP path switching while using old AN resources") indication when the UE performs the Mobility Registration Update procedure for non-3GPP access path switching. If the UE is registered to different PLMNs over 3GPP and non-3GPP accesses, the UE shall use the capability received over non-3GPP access to determine whether to perform the Mobility Registration Update procedure for non-3GPP path switching and whether to include ("Non-3GPP access path switching while using old AN resources") indication.

NOTE 9:	If the AMF receives ("Non-3GPP path switching while using old AN resources") indication from Mobility Registration Update procedure and the serving SMF(s) for PDU Session(s) is not supporting non-3GPP access path switching, the non-3GPP access path switching can still be performed with the AMF triggering for each PDU Session the release of the old user plane resources before new user plane resources are established.

An ATSSS-capable UE may decide to request a MA PDU Session based on the provisioned URSP rules. In particular, the UE should request a MA PDU Session when the UE applies a URSP rule, which triggers the UE to establish a new PDU Session and the Access Type Preference component of the URSP rule indicates "Multi-Access" (see TS 23.503 [45]).

*** Next Change ***

5.32.6.1	General

The functionality in an ATSSS-capable UE that can steer, switch and split the MA PDU Session traffic across 3GPP access and non-3GPP access, is called a "steering functionality". An ATSSS-capable UE may support one or more of the following types of steering functionalities:

-	Steering functionalities based on multi-path protocols:

-	Steering functionality that applies the MPTCP protocol (IETF RFC 8684 [81]) and is called "MPTCP functionality" (see clause 5.32.6.2.1). This steering functionality can be applied to steer, switch and split the TCP traffic flows identified in the ATSSS/N4 rules. The MPTCP functionality in the UE may communicate with an associated MPTCP Proxy functionality in the UPF, by using the MPTCP protocol over the 3GPP and/or the non-3GPP user plane.

-	Steering functionalities based on MPQUIC that apply the QUIC protocol (IETF RFC 9000 [166], IETF RFC 9001 [167], IETF RFC 9002 [168]) and its multipath extensions (draft-ietf-quic-multipath [174]). The MPQUIC functionality in the UE may communicate with an associated MPQUIC Proxy functionality in the UPF, by using the QUIC protocol and its multipath extensions over the 3GPP and/or the non-3GPP user plane. The following MPQUIC-based steering functionalities are supported:

-	"MPQUIC-UDP" steering functionality that can be applied to steer, switch and split UDP traffic flows identified in the ATSSS/N4 rules.

-	"MPQUIC-IP" steering functionality that can be applied to steer, switch and split IP traffic identified in the ATSSS/N4 rules.

-	"MPQUIC-E" steering functionality that can be applied to steer, switch and split Ethernet traffic identified in the ATSSS/N4 rules.

-	Steering functionalities based on data switching:

-	A steering functionality called "ATSSS Low-Layer functionality", or ATSSS-LL functionality (see clause 5.32.6.3.1): This steering functionality can be applied to steer, switch and split all types of traffic, including TCP traffic, UDP traffic, Ethernet traffic, etc.

NOTE:	Filters used in ATSSS rules related with a MA PDU Session of type Ethernet can refer to IP level parameters such as IP addresses and TCP/UDP ports.

The UE indicates to the network its supported steering functionalities and steering modes by including in the UE ATSSS Capability one or more of the following capabilities:

1)	ATSSS-LL functionality with any steering mode.

In this case, the UE indicates that it is capable to steer, switch and split all traffic of the MA PDU Session by using the ATSSS-LL functionality with any steering mode allowed for ATSSS-LL, as specified in clause 5.32.8.

2)	MPTCP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. The MPTCP functionality shall not be used when the type of the MA PDU Session is Ethernet.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split the MPTCP traffic of the MA PDU Session by using the MPTCP functionality with any steering mode specified in clause 5.32.8; and

b)	it is capable to steer and switch all other traffic (i.e. the non-MPTCP traffic) of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode specified in clause 5.32.8.

3)	MPQUICUDP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. The MPQUIC-UDP functionality shall not be used when the type of the MA PDU Session is Ethernet.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split the MPQUIC traffic of the MA PDU Session by using the MPQUIC functionality with any steering mode specified in clause 5.32.8; and

b)	it is capable to steer and switch all other traffic (i.e. the non-MPQUIC traffic) of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode specified in clause 5.32.8.

4)	MPQUIC IP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. The MPQUIC-IP functionality shall not be used when the type of the MA PDU Session is Ethernet.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split all traffic of the MA PDU Session, by using the MPQUIC-IP functionality with any steering mode as specified in clause 5.32.8. and

b)	it is capable to steer and switch all traffic of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode specified in clause 5.32.8.

5)	MPQUIC E functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode. MPQUIC E functionality shall be used only when the type of the MA PDU Session is Ethernet.

In this case, the UE indicates that:

a)	it is capable to steer, switch and split all traffic of the MA PDU Session of type Ethernet, by using the MPQUIC-E functionality with any steering mode as specified in clause 5.32.8. and

b)	it is capable to steer and switch all traffic of the MA PDU Session by using the ATSSS-LL functionality with the Active-Standby steering mode specified in clause 5.32.8.

6)	MPQUIC IP functionality with any steering mode without any ATSSS-LL functionality. The MPQUIC-IP functionality shall not be used when the type of the MA PDU Session is Ethernet. This capability shall be provided when the UE supports only MPQUIC-IP functionality.

In this case, the UE indicates that it is capable to steer, switch and split all traffic of the MA PDU Session by using the MPQUIC-IP functionality with any steering mode as specified in clause 5.32.8.

7)	MPQUIC-E functionality with any steering mode without any ATSSS-LL functionality. The MPQUIC-E functionality shall be used when the type of the MA PDU Session is Ethernet. This capability shall be provided when the UE supports only MPQUIC-E functionality.

In this case, the UE indicates that it is capable to steer, switch and split all traffic of the MA PDU Session by using the MPQUIC-E functionality with any steering mode as specified in clause 5.32.8.

The above steering functionalities are schematically illustrated in the Figure 5.32.6.1-1, which shows an example model for an ATSSS-capable UE supporting the MPTCP functionality, the MPQUIC-UDP functionality and the ATSSS-LL functionality. The MPTCP flows and the MPQUIC flows in this figure represent the traffic of the applications for which MPTCP can be applied and for which MPQUIC can be applied respectively. The five different IP addresses illustrated in the UE are further described in clause 5.32.6.2.1 and in clause 5.32.6.2.2. When the MPTCP functionality and the MPQUIC functionality are both applied, the addresses (IP@1, IP@2) used for MPTCP may be the same as the addresses (IP@4, IP@5) used for MPQUIC.

Figure 5.32.6.1-1: Steering functionalities in an example UE model

Within the same IP type MA PDU Session, it is possible to simultaneously steer different packet flows by different steering functionalities. For the same packet flow, only one steering functionality shall be used. For the same Ethernet type MA PDU Session, either ATSSS-LL steering functionality or MPQUIC-E steering functionality is enabled.

All steering functionalities in the UE shall take ATSSS decisions (i.e. decide how to steer, switch and split the traffic) by using the same set of ATSSS rules. Similarly, all ATSSS decisions in the UPF shall be taken by applying the same set of N4 rules, which support ATSSS. The ATSSS rules and the N4 rules supporting ATSSS are provisioned in the UE and in the UPF respectively, when the MA PDU Session is established.

If the UE supports multiple steering functionalities, it shall use the provisioned ATSSS rules (see TS 23.503 [45]) to decide which steering functionality to apply for a specific packet flow.

*** End of Changes ***