3GPP TSG-WG SA2 Meeting #166AH-E	S2-2500630

# E-meeting, January 20 – 26, 2025	 	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1352 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on VFL server discovery and VFL preparation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - If untrusted AF has capability of VFL server which allows the untrusted AF triggers VFL training or inference towards the whole network at any time, it is unsecure without registration information of the untrusted AF in network. In addition, in VFL inference, the AnLF needs to discover the untrusted AF as VFL server. Thus, the following Editor’s Note can be removed. Editor's note: Whether a discovery of an AF acting as VFL server is required and how it can be done is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the EN regarding to discovery of an AF acting as VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s Notes are not addressed and no support on NEF assisting VFL preparation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 5.5	AF Discovery and Selection for VFL

The AF discovery and selection is defined in clause 6.3.25 of TS 23.501 [2]. In addition to support VFL training and inference, the following factors may be considered for AF discovery and selection:

-	VFL capability information per supported AnalyticsID, which includes:

-	VFL capability type (i.e. VFL client)

-	optional Time interval supporting VFL.

-	VFL interoperability indicator of AF as VFL client per supported AnalyticsID: indicating if certain NF (i.e. NWDAF and/or AF) can perform VFL together with the AF as VFL client.

-	[Optional] supported feature ID(s): representing the different feature information supported by the AF as VFL client.

A trusted AF registers the above factor in NRF, for an untrusted AF the NEF is configured via OAM to register this factor into NRF.

For the discovery of a trusted AF or untrusted AF, the consumer may select an AF instance considering the above factor. The detailed procedure and parameters of AF registration and discovery for VFL are as defined in clause 6.2H.2.1.

# * * * * 2rd change * * * *

#### 6.2H.2.1	Registration and Discovery procedure for Vertical Federated Learning

##### 6.2H.2.1.1	Registration and Discovery procedure for Vertical Federated Learning when NWDAF is acting as the VFL server

Figure 6.2H.2.1.1-1: Registration and Discovery procedure for Vertical Federated Learning when NWDAF is acting as VFL server and NWDAF(s) and/or AF(s) are the VFL clients

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is NWDAF or a trusted AF.

1a.	VFL Server NWDAF/trusted AF registers to NRF with its NF profile, which includes NF Type (i.e. NWDAF type or AF type), Analytics ID(s), service area if available, VFL capability information per analytics ID and VFL interoperability indicator and optional supported feature IDs, as described in clause 5.2.

1b.	NWDAF as VFL client registers to NRF with its NF profile, which includes NF Type (i.e. NWDAF type), Analytics ID(s), service area if available, VFL capability information per analytics ID and VFL interoperability indicator and optional supported feature IDs, as defined in clause 5.2.

1c.	When untrusted AF as VFL client, it shall register to the NEF via OAM configuration: Analyics ID(s) and its VFL capability information per supported analytics ID and VFL interoperability indicator and optional supported feature IDs, as defined in clause 5.5. Then NEF updates NEF profile to NRF including associated AF ID and AF's VFL capability information and VFL interoperability indicator and optional supported feature IDs.

1d.	When trusted AF as VFL client, it registers to NRF with its NF profile, which includes NF Type (i.e. AF type), analytics ID(s), service area if available, VFL capability information per analytics ID and VFL interoperability indicator and optional supported feature IDs, as defined in clause 5.5.

2.	The NRF receives the registrations from VFL server and VFL client(s), and stores their NF profile.

3.	The NRF sends registration response to VFL server and VFL client(s).

Steps 4 to 6 are the NWDAF and AF Discovery procedures when the VFL server is NWDAF.

4-6.	NWDAF as the VFL server determines that the ML Model requires VFL based on e.g. operator policy, Analytics ID, and Service Area.

NOTE:	Step 4 in Figure 6.2H.2.1-1 may be triggered at the VFL Server by itself or a request from a consumer.

If the NWDAF can not perform as VFL Server, it first discovers and selects another VFL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, VFL capability type as VFL server, Time Period of Interest and optional Service Area.

Once the VFL Server is determined, the VFL Server discovers other NWDAF(s) and/or AF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: NF type (i.e. NWDAF type, AF type, or NEF type), Analytics ID, VFL capability type (i.e.VFL client), VFL interoperability indicator, Time Period of Interest, optional feature IDs and optional Service Area.

##### 6.2H.2.1.2	Registration and Discovery procedure for Vertical Federated Learning when untrusted AF is acting as the VFL server

The procedure below shows registration and discovery for VFL training and inference when the untrusted AF is the VFL server. There can be multiple NWDAFs as VFL clients.

NOTE 1:	For a deployment scenario that untrusted AF is the VFL server and only one NWDAF will be an VFL client, it may be assumed that VFL client information for an analytic ID is configured in NEF and step 6 and 7 may be skipped in Figure 6.2H.2.1.2-1.

Figure 6.2H.2.1.2-1: Registration and Discovery procedure for Vertical Federated Learning when AF is acting as VFL server and NWDAF(s) are the VFL clients

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is untrusted AF.

1a.	Same as the step 1b, in clause 6.2H.2.1, NWDAF as VFL client registers to NRF with its NF profile, it may include those analytics IDs on which it supports to do VFL with AF as VFL server.

1b. AF may register its capability as VFL server to NEF via OAM configuration.

NOTE 1:	The AF can use unstandardized values of the analytics ID. The unstandardized values can be used by the AF as the VFL server to initiate the VFL training VFL inference and need to be supported by NWDAFs acting as VFL clients. It is the operator´s responsibility to guarantee that unstandardized analytics ID values within a PLMN are unique.

2-3.	Same as the steps 2-3, in clause 6.2H.2.1.

Steps 4-10 are the NWDAF Discovery procedures when the VFL server is an untrusted AF.

4.	Untrusted AF acting as the VFL server determines that VFL operations are required and the NWDAF(s) as VFL client(s) are required. The AF sends a Nnef_NFDiscovery_Request for the VFL client(s) to the NEF and provides selection criteria: analytics ID, required NF type (i.e. NWDAF type), VFL capability type (i.e. VFL client), VFL interoperability indicator, Time Period of Interest, optional required feature IDs, and optional Service Area.

5.	The NEF checks based on configured policies whether the AF is entitled to request a VFL client for the analytics ID.

6-7.	The NEF discovers VFL client (i.e. NWDAF) on behalf of the AF from the NRF by invoking the Nnrf_NFDiscovery_Request using the selection criteria provided by the AF as defined in step 4.

Editor's note:	It is FFS, whether NEF should perform VFL NWDAF selection (or shortlisting) or it should only discover NWDAF client candidates without any further selection.

8.	The NEF selects an NWDAFs capable as acting as VFL clients and matching the received selection criteria. The NEF anonymizes NWDAF instances ID(s) and assigns external NWDAF ID(s) for each selected NWDAF intance as VFL client. The NEF stores the external NWDAF ID(s) together with information how to reach the NWDAFs.

NOTE 2:	The external NWDAF ID assigned by NEF is temporary and can be released if needed.

9.	The NEF sends Nnef_NFDiscovery_Request response including the external NWDAF ID(s) to the untrusted AF.

10.	The AF stores the received external NWDAF ID(s) and uses it subsequent interactions with the NEF to indicate the target VFL client.

NOTE 3:	If the VFL client NWDAFs are preconfigured to the NEF and untrusted AF, the discovery procedure can be skipped.

The AF may indicate to the NEF that the AF will no longer use the VFL process identified by VFL Correlation ID (external NWDAF IDs are no longer required). If the NEF receives this indication, it shall remove the stored external NWDAF ID(s) associated with the VFL correlation ID allocated to the AF. Then, the NEF responds to the AF.

# * * * * End of changes * * * *