3GPP TSG-WG SA2 Meeting #166AH-E	S2-2500631r01

# E-meeting, January 20 – 26, 2025	 	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1353 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on VFL training and VFL inference to remove ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In VFL, ML model metric could be one of elements to eveluate whether the ML model is converged. Furthermore, it is also important to monitor the accuracy of the ML model. Since the ground truth in each client cannot be shared in VFL, the ML model accuracy can be calculated locally in each client and then the server can determine the final accuracy, as it does in HFL. The algorithms of calculating accuracy between VFL and HFL may be different, but the definition of ML model metric can be reused. Thus, the following Editor’s Note can be removed. Editor's note: Whether the ML model metric (e.g. ML model accuracy) defined for HFL can be re-applied to VFL is FFS. - In VFL inference procedure, since analytics consumer is unable to be aware whether the Analytics output is generated by VFL method when it subscribes Analytics IDs, the consumer may also subscribe the Analytics accuracy information at the same time. In this case, the AnLF could provide the Analytics accuracy information based on ML model accuracy information (i.e. ML model metric) which is able to be calculated as the same logic as training process.Thus, the related accuracy monitoring procesures also need to be added in VFL inference. - Samples might be changeable during the VFL training. It does not make sense if the client could not hold the samples any more. Therefore, the sample maintanance is supposed to be performed at every round of VFL. Regarding to features, supported features are always based on the functionnalities of the NFs (i.e. NWDAFs or AFs), which are relatively fixed. Thus, the Feature IDs can only be provided in preparation phase. The following Editor’s Notes can be removed. Editor's note: It is FFS whether sample/feature information is required to be provided or updated in each training. Editor's note: Whether VFL server and VFL clients share feature information is FFS. - When untrusted AFs as clients, it might be a case that filter information provided by the server includes a list of TAIs, and TAI1 is in the AF1’s scope, TAI2 is in AF2’s scope… To avoid exposuring unnecessay network information to untrusted AFs (e.g. expose corresponding geographical area of TAI1 to untrusted AF2), the NEF is supposed be able to determine the specific filter information, target information, sample IDs, etc. for each client. In addtion, to reduce signallinga in VFL, the NEF may also gather the reports from clients and send them in one notification to the server. Thus, the following Editor’s Notes can be removed. Editor's note: How the NEF assists the VFL training process as well as whether the service operations going via NEF is using the existing or new service operation are FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add ML model accuracy monitoring related descriptions in VFL training and remove EN regarding to ML model metric. - Add Analytics accuracy monitoring in VFL inference. - Add sample maintanance during the VFL training and remove the ENs regarding to sample/feature information sharing issues. - Clarify on how NEF assists VFL training/inference process and remove the NEF related EN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s Notes are not addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.3.1, 6.2H.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 6.2H.2.3	Training Procedure for Vertical Federated Learning

##### 6.2H.2.3.1	Training Procedure for Vertical Federated Learning when NWDAF is acting as VFL server

The figure 6.2H.2.3.1-1 below shows the training procedure for Vertical Federated Learning when NWDAF is acting as VFL server.

Figure 6.2H.2.3.1-1: Training procedure for Vertical Federated Learning when NWDAF is acting as VFL server

Editor's note:	The details of the services in the procedure and whether VFL Training Start Flag is needed are FFS.

Editor's note:	Whether and how to include interoperability information in the VFL training procedure is FFS.

Editor's note:	Whether and how to define the trigger of VFL training is FFS.

Editor's note:	Whether and how to transfer the confirmation in VFL Preparation Phase at the beginning of VFL Training Phase is FFS.

1.	The NWDAF acting as VFL server determines the VFL clients that participate in VFL procedure in the VFL clients discovery and preparation phase as described in the clause 6.2H.2.1 and clause 6.2H.2.2.

NOTE 1:	VFL Server can determine to start the training based on local configuration and agreement among vendors and/or application providers participating in the same group for specific VFL task(s).

Steps 2-6 are repeated until the training termination condition is reached.

2.	To start the VFL training, the VFL server sends a request to start the training to all selected VFL clients The request includes VFL correlation ID, at least the parameters negotiated during the preparation phase, Optionally, the VFL Server includes: Analytic filter information, maximum response time (i.e. the maximum time between VFL clients receive intermediate model training information and send back intermediate training result), ML model accuracy check flag to request ML model accuracy information including the desired ML Model metric, suggested list of sample IDs if sample has change comparing with the sample in last round.

Editor's note:	Whether the parameters negotiated in the preparation phase are provided at the end of the preparation phase or at the start of the training is FFS.

If the VFL procedure continues in subsequent iterations, the VFL server sends a request for a new VFL training iteration containing the VFL correlation ID and intermediate model training information to each of the VFL clients for next round of VFL training.

2a.	The VFL server sends a Nnwdaf_VFLTraining_Subscribe to the selected NWDAF VFL clients(s).

2b.	The VFL server sends a Naf_VFLTraining_Subscribe to the selected trusted AF VFL clients(s).

2c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Subscribe to the NEF handling that AF.

2d.	For each selected untrusted AF VFL client, the NEF determines suggested list of sample IDs, analytic filter information corresponding to each untrusted AF based on the configuration in NEF and sends a Naf_VFLTraining_Subscribe to that AF. The NEF may also translate the analytic filter information if needed, e.g. TAIs into geographical area.

NOTE 2:	In this release, the same NF associated with a VFL Server or VFL Client capability during the VFL training for a VFL correlation ID is also the same NF during the VFL inference.

Editor's note:	Additional Parameters to be provided in the request are FFS.

Editor's note:	It is FFS whether and how the local ML model is obtained by VFL Client in VFL training process.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has no local data already available. The data used by each VFL Client is collected as per alignment information.

4.	During VFL training procedure, each VFL client further trains the local ML model associated with the same VFL Correlation ID based on their own collected or available data and when applicable (e.g. after the first round of training) and possible intermediate model training information distributed by the VFL server in the previous training iteration. Each VFL Client computes and reports the client intermediate training result of the local ML model to the VFL server. Each client checks the suggested list of sample IDs provided by the sever, and provides the new list of sample IDs that it can accept out of the suggested sample IDs if the accepted samples change.

NOTE 3:	The intermediate model training information and intermediate training result are constructed in per sample granularity.

Editor's note:	It is FFS and may depend on the service design: When the clients report the client intermediate training result, it also includes the corresponding VFL correlation ID.

Editor's note:	Whether and how to exchange intermediate training result among VFL Clients is FFS.

5.	Each VFL client reports the computed client intermediate training result of the local ML model, optionally, list of sample IDs to the VFL server. Each client may report local ML model metric (e.g. ML model accuracy, rating of the local data) if desired ML Model metric is contained in step 2.

5a.	A NWDAF VFL client sends a Nnwdaf_VFLTraining_Notify.

5b.	A trusted AF VFL client sends a Naf_VFLTraining_Notify to the VFL server.

5c.	An untrusted AF VFL client sends a Naf_VFLTraining_Notify to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external identifiers to internal identifiers and sends a Nnef_VFLTraining_AFClient_Notify to the VFL server. The NEF may gather the reports from untrusted AF VFL clients and send them together in one notification to the VFL server.

6.	The VFL server may collect the local data and generate its own local intermediate training result. The NWDAF acting as VFL Server computes the intermediate model training information (e.g. gradient information or loss information) based on the VFL Client(s) intermediate training result(s) received in step 4, its own local intermediate results and the label. The intermediate model training information is used for updating the models of VFL clients. Different intermediate model training information may be computed for different VFL clients and for the VFL Server itself. The client may further check the list of samples provides by each client and determine whether to update samples or reselect clients.

The VFL server may also compute the global ML model metric (e.g. ML model accuracy) based on all the intermediate training result and local ML model metric received from VFL clients and the label.

Editor's note:	Whether weight of the VFL Client is computed by VFL server is FFS.

7.	[Optional] The NWDAF acting as VFL server evaluates (e.g. based on the convergence of a loss function or loss value and/or if the pre-set iteration number is reached and/or global or local ML model metric is stable) whether VFL Training process converged. If the VFL Server evaluates the VFL Training process did not converge, the NWDAF acting as a VFL Server determines another round of VFL training is required and repeats step 2 - 6. If the VFL Server evaluates the VFL training process converged, it determines the VFL Training is completed. In this case, the VFL Server terminates the current VFL training process via step 7.

The VFL training termination decision may be also made as follows:

Based on the consumer request, the VFL server sends VFL status report to the consumer. The status report may include global model metric (e.g. ML model accuracy).

Editor's note:	The content of the VFL status report is FFS.

Editor's note:	Whether VFL server sending convergence report to the VFL client and what is convergence report are FFS.

The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

Based on the subscription request sent from the consumer, the VFL server updates or terminates the current VFL training process.

8.	The VFL server sends VFL training termination message to VFL Client if it decides to terminate the VFL training process, the termination message contains VFL Correlation ID.

8a.	The VFL server sends a Nnwdaf_VFLTraining_Unsubscribe t to the selected NWDAF VFL clients(s).

8b.	The VFL server sends a Naf_VFLTraining_Unsubscribe to the selected trusted AF VFL clients(s).

8c.	For each selected untrusted AF VFL clients, the VFL server sends a Nnef_VFLTraining_AFClient_Unubscribe to the NEF handling that AF.

8d.	For each selected untrusted AF VFL clients, the NEF sends a Naf_VFLTraining_Unsubscribe to that AF.

9.	The VFL Server, stores VFL correlation ID, the local trained ML Model, the mapping information of the VFL correlation ID to the following parameters: Analytics ID related to the VFL training process, locally trained Model. Additionally, the VFL server stores the VFL client information, which may be used to determine associated VFL client in the VFL inference.

Each VFL client stores VFL correlation ID, the locally trained ML Model, the mapping information of the VFL correlation ID to locally trained Model

NOTE 4:	The VFL correlation ID and the stored mapping information are used later for inference as described in Clause 6.2H.2.4.1.

Editor's note:	Whether VFL Training termination Flag in the termination message is required	is determined after settling down the service operation.

NOTE 5:	If untrusted AF is involved in VFL Clients, the message between NWDAF acting as VFL Server and the untrusted AF is via NEF.

# * * * * 2nd change * * * *

#### 6.2H.2.4	Inference procedure for vertical federated learning

##### 6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF is acting as VFL server

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period and Analytics Filter) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

If the analytics consumer NF requests Analytics Accuracy Monitoring, Analytics Accuracy Request information as defined in clause 6.1.3 will be contained in Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF determines the VFL Server for the requested analtyics, sends a subscription request to NWDAF VFL server using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period, Analytics Filter and Analytics Accuracy Request information.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. VFL Server selects clients(s) using information stored in the VFL training process. The server may select some or no clients, e.g. depending on the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients.

When no VFL Clients are selected, the VFL server may generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 - 6 (and 7 if step 1 was also skipped).

VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients including the Target of VFL inference = e.g. UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used and optionlly VFL inference filter, desired ML Model metric to request ML Model accuracy information for monitoring the ML model accuracy, if Analytics Accuracy Request information is contained in step 1.

Editor's note:	It is FFS whether additional parameters are needed to send from the VFL server to VFL client, e.g. parameters used in training phase and parameters from Analytics request.

Editor's note:	It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying.

2a.	For each NWDAF VFL client, the VFL Server NWDAF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference_Subscribe or Nnef_VFLInference_Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF determines Target of VFL inference, VFL inference filter corresponding to each untrusted AF based on the configuration in NEF and converts any internal identifiers to external identifiers. The NEF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results. Each VFL client will further collect ground truth data to calculate local ML Model metric (e.g. local ML model accuracy information), if the VFL client is requested to provide ML Model metric in step 2.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	VFL Client sends the client intermediate results to the VFL server.

The intermediate results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results. VFL client will send local ML model metric (e.g. local ML Model accuracy information) to VFL server if the VFL client is requested to provide ML Model metric in step 2.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF.

5b.	Each trusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference_Request response to the NWDAF VFL server. The NEF may gather the reports from untrusted AF VFL clients and send them together in one notification to the VFL server.

6.	The VFL server may collect its local data and generate the intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate results when generates the combined inference output.

The VFL server may also compute the global ML model metric (e.g. ML model accuracy) based on all the intermediate training result and local ML model metric received from VFL clients and the label.

7.	Depending on request, the NWDAF VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results, optionally, ML model metric (e.g. ML model accuracy).

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

The NWDAF containing AnLF may also provide Analytics Accuracy Information based on ML model metric (e.g. ML model accuracy) if the consumer NF requests Analytics Accuracy Monitoring in step 0.

# * * * * End of changes * * * *