SA WG2 Meeting #166-Ad-Hoc-e	S2-2500647r01

20 - 24 January, 2025, Electronic meeting	(revision of S2-25xxxxx)

Source:	ZTE

Title:	Address EN Regarding Command-only Service Operation

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: The paper is proposed to address Editor’s Note regarding Command-only service operation.

# 1. Discussion

In TR 23.700-13, the following Editor’s Note exists:

Editor’s note:	If the “command-only” case applies, pending SA WG3, if a command and paging can be performed in a single operation, then an AIoT specific NAS message may be included in the request.

In RAN TR 38.769, the following description regarding command-only service operation is given:

-	For the detailed use case of "command-only":

-	It can be also supported by the baseline procedure with Step A, Step B, Step C1 and Step C2.

-	In addition, another candidate to support this use case is following, whose feasibility still depends on the conclusion from [151] and [152]:

-	Step A’: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message including the command, indicating device(s) to process/respond the command.

-	Step C2: Possible D2R data transmission (e.g., the device ID and/or the corresponding response to command), via the A-IoT random access procedure or without using the A-IoT random access procedure.

The paper removes the EN and adds clarification on command-only service operation in TR 23.700-13.

# 2. Proposal

It is proposed to capture the following changes into TR 23.700-13.

* * * * Start of Changes * * * *

### 8.3.4	Principles on the procedures to support AIoT services

To support the services provided by 5GC and the NEF exposure of those AIoT services, the following procedures are supported:

-	Inventory Procedure.

-	Command Procedure, to e.g. transfer AF AIoT Data to/from AIoT Device(s) as AIoT specific NAS messages.

There are requests that are used from the AIOTF towards the Reader and responses from the Reader to the AIOTF. The routing of the request and response messages and their encoding depends on the topology and transport to the Reader (see KI#1).

All the procedures follow have the following steps:

1.	The AF makes a service request to the NEF, including parameters to identify the target AIoT Device(s), target Readers and service operation specific parameters.

2.	The NEF determines an AIOTF for the requested operation, and invokes a new service operation on the AIOTF.

3.	For the requested operation the AIOTF:

1).	Performs initial reader selection by either:

-	The AF providing information to identify readers to include in the initial reader selection network. The information can identify multiple or an individual Reader to use for the request.

NOTE 1:	Which readers are identified by the information from the AF is up to the network deployment, configuration or implementation.

- 	If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

- 	If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

If no readers can be selected then the request is rejected.

2).	Determines A-IoT Device Identification information  based on the information from the AF, to be included in the paging message on the AIoT radio interface to find the AIoT Devices. AIoT Devices compare the A-IoT Device Identification information  with their own AIoT Device Identifier (part of or full AIoT Device Identifier) to determine whether respond to the paging message.

Editor's note:	Whether and how the A-IoT Device Identification information will be security protected will be concluded by SA WG3.

3).	Determines Reader Assistance information required for the operation used to the Reader, taking into account assistance information from the AF.

4).	Constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request. The Inventory request is routed to the Readers determined by the initial reader selection.

See clause 8.1 for how to provide the request to a Reader.

5).	The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages. The AIOTF can determine which request the results are for using the correlation identifier.

6).	The AIOTF may, depending on the information within the AIoT Device identifier, obtain subscription-like information from either:

-	the serving network performing the operation (either as identified by a AIoT Device Identifier or operator policy to check it been provided with information for a specific AIoT Device), or

-	another network as identified by a AIoT Device Identifier, or

-	A third party as identified by a AIoT Device Identifier.

7).	Checks if the AIoT Device is subscribed.

NOTE 2:	Whether and how AIOT Device Identifiers are verified depends on SA WG3.

8).	If the operation is a command operation, the AIOT generates a request, including an AIoT specific NAS message for the command, along with any additional information required by the Reader to execute the command, a TASK ID etc. The request is then routed to the Reader. The Reader executes the command, passing the AIoTF specific NAS message to the AIoT Device and collecting any AIoT specific NAS responses. The AIoT specific NAS responses are then routed back to the AIOTF.

NOTE 3:	Whether and how security protection is applied to the AIoT specific NAS message send to the AIoT Device and the response AIoT specific NAS message from the AIoT Device depends on SA WG3.

9).	Provide the results of the operation to the NEF. Results from multiple AIoT Devices may be included/aggregated in the service response(s).

4.	Provide the results of the operation from the NEF to the AF.

* * * * End of Changes * * * *