SA WG2 Meeting #166-Ad-Hoc-e	   S2-2500648r05

20 - 24 January, 2025, Electronic meeting

Title:	[DRAFT] Reply LS on Assistance Information from CN to the Reader

Response to:	Further LS on Assistance information from CN to the reader (S2-2500078/R2-2411232)

Release:	Rel-19

Work Item:	FS_Ambient_IoT_solutions, FS_AmbientIoT

Source:	SA2

To:	RAN2

Cc: 	RAN3

Contact Person:

Name:	Hao Dong

Email:	dong dot hao1 at zte dot com dot cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks RAN2 for the LS (R2-2411232) on Assistance information from CN to the reader. SA2 would like to provide the answer to the questions in the LS as following:

Q1: Is it feasible for CN to provide the following information to the reader?

- the service type of A-IoT (e.g. inventory, command)

- whether the service is targeted for a single or multiple devices;

- the approximate number of target devices (if available).

SA2 answer: CN will provide the service type, and if provided by the AF, the approximated number of target devices, to the reader. And CN will not provide explicit indication whether the service is targeted for a single or multiple devices.

Q2: Can there be a case where a D2R response is not expected for a R2D message for command service type? If so, can the CN indicate to the reader whether a message is expected in D2R direction in response to an R2D message for command service type (e.g., either explicitly or implicitly by indicating the response message size if available)?

SA2 answer: No. A D2R response is always expected for a R2D message for command service type (e.g., the service data response for 'Read' service operation, and the operation result indication response for 'Write' or 'Disable' service operation).

Q3: Will there be any latency related requirements associated with completion time of a given A-IoT service, provided/visible to the reader?

SA2 answer: No.

2. Actions:

To RAN2:

ACTION: SA2 respectfully asks RAN2 to take the answer to the above questions into account.

3. Date of Next SA2 Meetings:

SA2 Meeting #167		17 - 21 February, 2025	Athens, Greece

SA2 Meeting #168		7 - 11 April, 2025	Goteborg, Sweden