3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e 	S2-2500676

# Elbonia, 20-24 January, 2025                                                 	            (revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1361 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to VFL Inference Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | a) Alignment of VFL Training definitions of their usage in the VFL Inference The Clause 6.2H.2.3.1 defined that the mapping information stored at the end of the training will be used in the VFL Inference. However, how this information is used in the VFL Inference was not defined yet. NOTE 4: The VFL correlation ID and the stored mapping information are used later for inference as described in Clause 6.2H.2.4.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Stored Mapping information (defined in the VFL Training procedure Clause 6.2H.2.4.1 to be used in the VFL Inference) is used for supporting the NWDAF contianing ANLF to determine: i) the VFL Correlation ID to the requested analtyics ID, ii) the ML model associated with the VFL Correlation ID; iii) whether the NWDAF containing AnLF that received the subscription is the VFL Server itself (i.e., is the same NF as the MTLF that performed the training) or a VFL Client and the VFL Server to perform the request (related to step 1 and 2 in procedure in Figure 6.2H.2.4.2-1); |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of consistency between VFL training procedure and VFL Inference Procedure, and lack of compatibility with accuracy information exposure to NWDAF Service Consumers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF is acting as VFL server

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Analytics Filter Information, Target of Analytics Reporting= e.g. UE IDs and Analytics Reporting Information) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	When the NWDAF containing AnLF receives the subscription for analytics ID from a Consumer NF, the NWDAF Containing AnLF verifies r VFL model indication, If there is stored VFL Server information, the NWDAF containing AnLF determines the VFL Server for such analytics ID..

NOTE 1:	In this release, the same NF associated with a VFL Client  capability during the VFL training is also the same NF during the VFL inference. This information is comprised in stored mapping information as per VFL Client information for a given VFL correlation ID.

When the NWDAF containing AnLF receives the subscription for analytics ID from a Consumer NF,if the NWDAF containing AnLF determines it is the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF determines it has no VFL Server information stored during VFL Training, the NWDAF containing AnLF sends a model provisioning request to a NWDAF containing MTLF (as defined in clause 6.2A.1). If the NWDAF containing MTLF that received the request determines that VFL is applicable and it is the VFL Server for the request, the NWDAF containing MTLF provides a Nnwdaf_MLModelProvision_Notify message to the NWDAF containing AnLF including the VFL Model indicationindicating VFL Model(s) have to be used for the request and no ML Model will be provided.

if  the NWDAF containing AnLF can not generate the analytics output by itself, the NWDAF containing AnLF determines the VFL Server by mapping the requested analytics ID to the correaponsding VFL server. The mapping information is stored in the AnLF after the VFL training process. Alternatively, if the NWDAF containing AnLF received VFL Model indication, the NWDAF containing AnLF ,the AnLF sends a request to NWDAF as VFL server using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Analytics Filter Information, Target of Analytics Reporting = e.g. UE IDs, the Subscription Correlation ID of previous Nnwdaf_MLModelProvision_Subscribe, and Analytics Reporting Information or other parameters as defined in Clause 6.1.3.

2.	Based on the information received in the step 0 or 1 and the information stored in the VFL Training process, the VFL server decides to initiate the VFL inference procedure. The VFL Server determines the applicable VFL correlation ID for the requested or subscribed analtyics and the its own ML model to used for its own generation of the VFL results based on the mapping information stored at the end of the VFL Training procedure or the information received in step 1.

The VFL server may select some or no VFL clients associated with the VFL correlation ID from the VFL Clients in stored information during VFL training procedure for such VFL Correlation ID. The selection of some or no VFL client(s) depends on at least one of the following criteria: the accuracy of the VFL inference, the contribution to the training result and the current status of the VFL clients.

When no VFL Clients are selected, the VFL server may generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 - 6 (and 7 if step 1 was also skipped). If the server does not select VFL clients for all features, it stores information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results, for instance in the ADRF, to anable to subsequently trace the origin of analytics results during analytics monitoring.

When the criteria for selecting the VFL clients is the contribution to the training results, the VFL Server verifies in the stored mapping information the VFL Clients with the contribution weight to participate in the VFL Inference.

When VFL Client(s) are selected, the VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients including the Target of VFL inference = e.g. UE IDs, VFL correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used, and optionlly VFL inference filter. The VFL Client, checks its local stored mapping information in order to determine the local ML model associated with the VFL Correlation ID to be used for the VFL inference process requested by the VFL Server.

Editor's note:	It is FFS whether additional parameters are needed to send from the VFL server to VFL client, e.g. parameters used in training phase and parameters from Analytics request.

Editor's note:	It is FFS how the origin of analytics results can be traced and explained if not all clients participate and results are not satisfying.

2a.	For each NWDAF VFL client, the VFL Server NWDAF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference_Subscribe or Nnef_VFLInference_Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

5.	VFL Client sends the intermediate local inference results to the VFL server.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

Editor's note:	It is FFS additional parameters are needed to send from the VFL client to VFL server.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF.

5b.	Each trusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends a Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference_Request response to the NWDAF VFL server.

6.	The VFL server may collect its local data and generate the intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate local inference results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate results when generates the VFL inference results.

7.	Depending on request, the NWDAF VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

# * * * * End of changes * * * *