3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500677

# Elbonia, 20-24 January, 2025		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1362 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VFL Accuracy Monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Accuracy Monitoring is a functionality from NWDAF and the introduction of VFL Inference requires changes in the way that accuracy monitoring procedures are executed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal to introduce 2 new procedures. - A new procedure for VFL Accuracy Monitoring based on the existing procedure for Analytics Accuracy Montioring in Clause 6.2D.2 with the extensions of Steps 1, 2, 5, to enable extending the procedure to allow the NWDAF acting as VFL Server to calculate the VFL accuracy information and use such information for the control of the inclusion of VFL Clients in the procedure. - Extensions of the procedure for MTLF-based ML Model Accuracy Monitoring in clause 6.2E.2 to enable the MTLF-base VFL training accuracy determination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No proper support for accuracy monitoring when VFL Inference is executed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.X (new), 6.2H.X.1 (new), 6.2H.X.2 (new), 6.2E.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change (ALL NEW TEXT)* * * *

6.2H.X	Accuracy Monitoring for Vertical Federated Learning

6.2H.X.1	Procedure for Analytics Acuracy Information for Inference with Vertical Federated Learning

The procedure for Analytics Accuracy Information Subscription define in clause 6.2D.2 is the baseline for the procedure for the VFL Accuracy Monitoring. The extensions to the original procedure for supporting VFL accuracy monitoring are described as follows:

- 	Step 1 in Figure 6.2D.2-1 describes a request from the NWDAF Service Consumer as a trigger for the analtyics accuracy generation. In VFL Accuracy monitoring, in addition to a request from the NWDAF Service Consumer, the NWDAF containing AnLF itself may trigger the VFL accuracy monitoring as described in clause 6.2H.2.4.1.

When the NWDAF containing AnLF is the VFL server in a VFL Inference execution, the NWDAF containing AnLF triggers the VFL accuracy monitoring in order to generate the VFL accuracy information. This information is used by the NWDAF containing AnLF acting as VFL Server to determine changes in the generation of VFL inference results, e.g., including selected VFL Clients if no VFL Client was selected before.

-	Step 2 in Figure 6.2D.2-1 describes the execution of the analytics accuracy monitoring and the calculation of the analtyics accuracy information.

When the generation of analytics output is based on VFL Inference, and the subscription request received from the NWDAF service consumer in Step 1 includes the Analytics Accuracy Request information and/or the VFL Server triggered the VFL Accuracy Monitoring in Step 1, the NWDAF containing AnLF generates the VFL Accuracy Information for a VFL correlation ID, and it comprises the calculated accuracy value, the VFL correlation ID, and the list of VFL Clients information related to the calcutated accuracy. The VFL accuracy information is generated based on the collected ground truth and the caculated VFL inference result (as performed in clause 6.2H.2.4.1).

If Analytics Accuracy Information has to be provided to the NWDAF Service Consumer, the NWDAF containing AnLF assigns the calculated accuracy value of the VFL Accuracy Information as the same calculate value of the Analytics Accuracy Information without exposing any further information of the VFL Accuracy Information to the NWDAF Service Consumer.

NOTE 1:	The association of the list of VFL Clients (e.g., none or some or all VFL Clients that participated in the VFL training) to the calculated accuracy for the VFL inference, allows NWDAF containing AnLF to trace and support the analysis of the contribution of selected VFL Clients to a VFL inference procedure as described in clause 6.2H.2.4.1.

- Step 5:	Same as Step 5 in clause 6.2D.2 with the following extensions. 	If VFL Inference procedure in clause 6.2H.2.4.1 is executed for the requested Analtyics ID, the NWDAF acting as VF Server may decide based on the determination of the low or insufficient accuracy to change the VFL Inference. If no VFL Clients were selected for the execution of the VFL Inference, the NWDAF acting as VFL server may decide to select and include some or all VFL Clients. The changes on the execution of the VFL procedure are transparent to the NWDAF Service Consumer.

NOTE 2:	The NWDAF VFL server keeps the mapping of the generated analytics output to the list of VFL Clients (or no VFL Clients) used for the generated analtyics output. When applicable, the VFL Server also assoicates the VFL accuracy information to such mapping.

# * * * * Next Change* * * *

6.2E.3.1	General

The procedures described in this clause enable the following functionality:

-	An NWDAF containing AnLF may register with an NWDAF containing MTLF when it starts using an ML Model and monitoring the accuracy of analytics generated by that ML Model for a given Analytics ID. It is assumed that the NWDAF containing AnLF obtained the ML Model in a previous interaction with the NWDAF containing MTLF, e.g. using the Nnwdaf_MLModelInfo_Request or Nnwdaf_MLModelProvision_Subscribe services. This registration enables the NWDAF containing MTLF to become aware of NWDAF containing AnLF that are using a given ML Model for certain Analytics ID and that the NWDAF containing AnLF supports the capability of monitoring the accuracy of the corresponding analytics.

-	An NWDAF containing MTLF may subscribe to an NWDAF containing AnLF where an existing Nnwdaf_MLModelMonitor service is established for receiving notifications of the accuracy of analytics generated by a given ML Model for a certain Analytics ID. NWDAF containing AnLF can generate the accuracy information in many ways: e.g. comparing predictions of ML Model and its corresponding ground truth data, comparing changes in internal configuration for the analytics ID generation, previous existent records of Analytics Accuracy Information, etc.

NOTE:	In this release of the specification, if VFL is supported by the NWDAF containing MTLF requiring AnLF-Assisted MTLF ML Model Accuracy monitoring, and this NWDAF is the VFL Server, then the NWDAF containing MTLF is the same as the NWDAF containing AnLF. Therefore, the procedures in Clause 6.2E.3.2 and 6.2E.3.3 are implementation dependent.

# * * * * End of changes * * * *