3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500685

# Elbonia, 20 – 24 January 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0294 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Further clarification on Local offloading traffic handling at I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As discussed at S2-2500610, it is suggested to clarify following issues: - AF traffic influence information is transferred to the I-SMF to avoid different DNAI selection for the same service flow. - How to use and update the Offload ID is not clear. - How N4 rules are generated by I-SMF need clarification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SMF transfers the AF traffic influence information to I-SMF and remove the corresponding EN. - How to use and update offload ID is added. - Option 1: Add description that the I-SMF sends the SDF(s) of offloaded traffic to SMF for generating N4 information for QoS handling. This option also supports the SMF to be aware of DNAIs for QoS Flow binding to differentiate dynamic value of CN PDB as described in clause 6.1.3.2.4 of TS 23.503 (i.e. which requires different QoS Flows for SDFs that have different dynamic values of CN PDB). Option 2: The packet filter set within the PDR for offloaded traffic handling is set by the I-SMF. This option cannot support dynamic CN PDB since the SMF does not know SDF(s) and DNAI(s) of the offload traffic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unclear whether the AF traffic influence information is sent to I-SMF. - Unclear how the Offload ID is used and updated. - Unclear how the SMF fill the packet filter set within the PDR for offloaded traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.2, 6.10.3.1, 6.10.3.3, Option 1: 6.10.1 Option 2: 6.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.502 CR 5272 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.10.3	Policy provisioning for Local Offloading Management

6.10.3.1	General

For Local Offloading Management, traffic routing related policy (e.g., local offloading management policy) need be provided to the I-SMF.

Based on the received AF traffic influence information, Local offloading management policy information and EAS Deployment information, the I-SMF may (re)discover and (re)select ULCL and local PSA UPF for routing the traffic locally.

NOTE:	The AF does not need to be aware of whether Local Offloading Management is applied or not.

6.10.3.2	Local offloading management policy handling for Local Offloading Management

The Local Offloading Management Policy is generated by PCF and retrieved by SMF for Local Offloading Management as described in clause 6.10.2. The Local offloading Management Policy may also be provided with a Offload Identifier. The Offload Identifier is assigned by PCF, and is unique within the PLMN. The SMF and I-SMF for Local Offloading Management may also be preconfigured with Local Offloading policy and tagged with related Offload Identifier(s). In this case, the PCF only provides the Offload Identifier to the SMF.

The detailed information of Local Offloading Management Policy is described in clause 6.4 of TS 23.503 [4].The SMF forwards the received Local Offloading Management Policy or Offload Identifier to the I-SMF transparently.

6.10.3.3	AF request for Local Offloading Management

The AF may send to NEF an AF request to influence traffic routing as described in caluse 4.3.6 of TS 23.502[3]. When the SMF receives AF traffic influence information in the PCC rule, it provides to I-SMF the related AF traffic influence information to assist I-SMF for traffic offloading. The I-SMF takes the received AF traffic influence informaion into account for local PSA UPF (re)selection.

NOTE X:	The AF does not need to to be aware of whether Local Offloading Management is applied or not.

# * * * * Option 1 * * * *

# * * * * Next change * * * *

6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects an I-SMF to support Local Offloading Management. This also includes the case when the UE is within the SMF service area. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 1:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 2:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF, as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

The I-SMF may insert a Local PSA and optional an UL CL into the data path due to traffic offloading, e.g., per the result of the DNS message handling. The I-SMF notifies the SDF (Service Data Flow Filter) of the offloaded traffic to the SMF. Per the received SDF, the SMF generates N4 information and sends to the I-SMF to configure N4 rules on UL CL and Local PSA.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

# * * * * Option 2 * * * *

# * * * * next change * * * *

6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects an I-SMF to support Local Offloading Management. This also includes the case when the UE is within the SMF service area. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 1:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 2:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF, as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

The I-SMF may insert a Local PSA and optional an UL CL into the data path due to traffic offloading, e.g., per the result of the DNS message handling. When the SMF generates the N4 information for offloaded traffic handling, it does not fill the offloaded traffic related packet filter set(s) within the PDR, which are filled by the I-SMF before the N4 rule(s) are configured at the Local PSA and UL CL.

NOTE X: 	Dynamic CN PDB is not supported for Local Offloading Management in this release of the specification.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

# * * * * End of changes * * * *