3GPP TSG-SA WG2 Meeting #166-Ad-Hoc-e	S2-2500718r01

# Electronic meeting, January 20-24, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1561 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of mobility functionality for Support of UE-Satellite-UE communication in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the approved 5GSAT_Ph3-ARC work item and the conclusion of FS_5GSAT_Ph3-ARC captured in clause 8.3 of TR 23.700-29, this CR adds description on mobility functionality for Support of UE-Satellite-UE communication in IMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of mobility functionality for Support of UE-Satellite-UE communication in IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An objective of the 5GSAT_Ph3-ARC work item is not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.2.2.1, AE.2.2.2 (new), AE.3.3 (new), AE.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

### AE.2.2.1		P-CSCF

The P-CSCF used for this feature is enhanced to support the following functionalities:

-	the P-CSCF determines the activation of optimized media routing and interacts with 5GS, as described in clause AE.3.

-	the P-CSCF interacts with IMS AGW, as described in clause AE.4.

-	the P-CSCF requests IMS AS used for this feature to initiate SIP re-INVITE to complete IMS AGW relocation, as described in clause AE.4.

## ---Start of the 2nd Change---

AE.2.2.2	IMS AS

The IMS AS used for this feature is enhanced to support the following functionalities:

-	the IMS AS, requested by P-CSCF used for this feature, initiates SIP re-INVITE to complete IMS AGW relocation, as described in clause AE.4.

## ---Start of the 3rd Change---

AE.3.3	At change of satellite

The P-CSCF determines whether optimized media routing continues to be possible between a satellite that is about to serve the local UE and the satellite serving the remote UE based on the identifiers of those satellites. The P-CSCF obtains the identifier of the new satellite from PCF as per clause AE.5.2.1. With the lack of the identifier of the new satellite in the notification, the P-CSCF determines that optimized media routing is no longer possible.

NOTE:	In this Release of the specification, how the P-CSCF uses the satellite identifier to derive whether optimized media routing can continue is not specified.

If the P-CSCF determines that optimized media routing can continue at change of satellite, the P-CSCF sends a message to the PCF as defined in TS 23.502 [94] and as per clause AE.5.2.1 to establish a path on the target satellite for optimized media routing.

If the P-CSCF determines that optimized media routing is no longer possible e.g. due to a change of satellite, the P-CSCF executes the ground fallback procedure as per clause AE.5.2.2 to reroute the media via the ground.

## ---Start of the 4th Change---

AE.4.3	At change of satellite

When P-CSCF is informed that the satellite serving UE changes as defined in TS 23.502 [94] and as per clause AE.5.2.1, the P-CSCF initiates the procedure to establish an IMS AGW on the target satellite as per clause AE.5.2.1 or on ground as per clause AE.5.2.2, while keeping the IMS AGW on the source satellite, depending on whether optimized media routing continues to be possible or not.

As the last step of establishment of the IMS AGW on the target satellite or on ground, the P-CSCF requests the IMS AS to initiate SIP re-INVITE to inform the remote network and then the local UE to use the newly established IMS AGW(s) as per clause AE.5.2.1 and AE.5.2.2.

## ---End of the Changes---