3GPP TSG-SA WG2#166AHE	S2-2500734

E-meeting, 20-24 January 2025	(was)

Source:	Samsung, vivo

Title:	pCR to TS 23.xyz: NEF Services

Document for:	Discussion/Approval

Agenda Item:	19.14.2

Work Item / Release:	Rel-19 / AmbientIoT

Abstract of the contribution: This paper proposes text for NEF Services.

1 Discussion

During SA2#166 meeting, interim conclusions on AIoT Architecture and Services. Considering possible agreements in SA2#166AHE, we may capture TS text for NEF Services.

2 Proposal

It is proposed to capture the following changes to new TS.

START OF CHANGES (All New)

7.4	NEF services

The following NF services are specified for NEF for Ambient IoT:

Table 7.4-1: NF Services provided by NEF for Ambient IoT

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

7.4.1	Nnef_ AIoT_Inventory service operation

Service operation name: Nnef_AIoT_Inventory.

Description: The consumer requests an inventory operation for AIoT Device(s).

Inputs, Required: AF ID, Filtering information for AIoT Devices or AIoT Device ID(s)

Inputs, Optional: target Reader ID list, target area information, the approximate number of target AIoT Devices, approximate D2R message size, report control information.

Outputs, Required: Correlation ID, operation accepted or rejected.

Outputs, Optional: None.

7.4.2	Nnef_ AIoT_Command service operation

Service operation name: Nnef_AIoT_Command.

Description: The consumer requests a command operation for AIoT Device(s).

Inputs, Required: AF ID, AIoT data, Filtering information for AIoT Devices or AIoT Device ID(s), service type.

Inputs, Optional: Target Reader ID list, Target area information, the approximate number of target AIoT Devices, report control information.

Outputs, Required: Correlation ID, operation accepted or rejected.

Outputs, Optional: None.

7.4.3	Nnef_ AIoT_Notify service operation

Service operation name: Nnef_AIoT_Notify

Description: The NEF uses this service operation to notify the results of the service operation towards the consumers.

Inputs, Required: Correlation ID.

Inputs, Optional: results of the service operation.

Outputs, Required: None.

Outputs, Optional: None.

END OF CHANGES