3GPP TSG WG Meeting 166-Adhoc	S2-2500748r05

# Emeeting, 20 – 24 January 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5996 | rev | - | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving ENs in Clause 5.51.5 and 5.51.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., vivo, Samsung (?), ZTE (?), Nokia (?), CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For resolving Editor’s notes listed below: - Editor's note: The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS. - Editor's note: Whether the Energy Saving indicator is also stored in the UDR is FFS. 3 Editor's note: A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS. - The energy saving indicator can be a single value which can be the ratio of the energy consumption for a UE as described in Annex T.2, or the weight value calculated based on the energy related information by the operator’s policy. Multiple value for the enegy saving indicator may trigger the inconsistent decision for network side. - Since the value is accessed by AMF, it should be stored in UDR for AMF to retrive - If the energy saving indicator is based on the energy consumption calculation as descriend in Annex T.2, the PCF can be notified by EIF with the latest value to check whether the UE’s energy comsuption meets the energy saving indicator and to see whether the PCF needs to re-make the policy for the UE. - For UP adjustment for a PDU Session, the PCF may also request SMF to trigger PDU Session modification and PDU Session release procedures as specified in TS 23.502 after making policy desion |  |  |  |  |  |  |  |  |
| Summary of change: |  | To resolve ENs based on below changes: - The energy saving indicator is a singe value and can be calculated based on the operator’s policy - The energy saving indicator can be stored in UDR - The PCF can be notifed by EIF with the latest value of the energy consumption calculation - It updates the PCF can trigger PDU Session modification and PDU Session release as specified in TS 23.502 after making policy decision |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not complete feature for EnegySys |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.5, 5.51.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.502 ... CRxxx ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.51.5	Subscription information aspects

The Subscription data for a UE may include an Energy Saving indicator that the UE is subject to network energy saving operation. The Energy Saving indicator is stored in UDM/UDR.The AMF retrieves the Energy Saving indicator in the UE Subscription data. The AMF forwards the Energy Saving indicator to the NG-RAN and to the PCF.

NOTE X:	The indicator is expected to be a scalar and is implemented by the operator based on its operator policy an energy saving strategy for the UE in the system.

Editor's note:	Whether the indicator is provided to the NG-RAN andhow the indicator it is used in the NG-RAN is FFS.

5.51.6	Policy control aspects

During AM policy association establishment procedure, the AMF sends to the PCF the Energy Saving indicator as described in clause 5.51.5. The PCF may also subscribe to the notifications from EIF for energy consumtion information. Based on the Energy Saving indicator and EIF notifications, the PCF may make policy decisions reusing the existing parameters based on the operator policy.

Editor's note:	A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS.