3GPP TSG-WG SA2 Meeting #166-Ad Hoc-e	S2-2500782r02

# Online, 20th Jan – 24th Jan, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3867 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of S&F Monitoring List in S&F Mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARC |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As described in the discussion paper (S2-2500780), this contribution proposes: 1. Remove EN An EN was added in last meeting as below: Editor’s Note: How to handle the specification of the “List of satelliteID(s)” and “S&F Monitoring List” as per conclusions in TR 23.700-29 is FFS. The satellite list (no matter the “list of satellite ID(s)” or “S&F Monitoring List”) is a tool which is used by the UE assist with saving power and determining which satellites to synchronise data with, and a useful tool for the network to guide the UEs allowing the network to save resources. While how UE behaves when receiving the satellite list from MME is UE implementation. It is difficult to define the UE behaviour to only follow the received satellite list to access the network. Thus there’s no need for the MME to send two different satellite to UE. This contribution proposes MME sends S&F Monitoring list to UE and UE behaviour is based on UE implementation. For example, if the UE determines a satellite is operating in S&F Mode, the UE should follow the S&F Monitoring List to ensure a successful access; if the UE determines a satellite is operating in normal Mode, the UE can access to this satellite even it is not in the S&F Monitoring List. 2. Mode transition As proposed in the DP S2-2500780, this contribution proposes that when UE transits between S&F Mode and normal mode in different cell, the UE initiates initial Attach procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Introduce S&F Monitoring List to assist UE to have a sucessful network access and realize power saving. 2. When UE transits between S&F Mode and normal mode in different cell, the UE initiates initial Attach procedure to avoid failed NAS procedure due to UE context synchronization issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. S&F Monitoring List to assist UE with power saving and allow network to save resource is not supported. 2. The procedure for UE transits between S&F Mode and normal mode may be failed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.13.9	Support of Store and Forward Satellite Operation

#### 4.13.9.1	General

The Store and Forward Satellite Operation applies in E-UTRAN with satellite access and is suitable for delay-tolerant communication services (e.g. CIoT/MTC, SMS, etc.). If the satellite does not support Store and Forward Satellite operation and there is no feeder link connection to the ground network, then the satellite cannot provide any service to any UEs. When both the service link and feeder link are available, all services can be provided. To support Store and Forward Satellite operation, some network functionality needs to be deployed on the satellite payload.

Example deployments are described in Annex O.

Figure 4.13.9.1-1: Store and Forward (S&F) Satellite operation

NOTE 1:	In the Figure 4.13.9.1-1, the NTN GW belongs to the satellite deployment and is not subject to 3GPP specifications.

In Store and Forward Satellite operation, the end-to-end exchange of signalling/data traffic is handled in a sequence of steps reflecting the intermittent availability of the service link when the satellite can exchange data with the UE and of the intermittent availability of the ground link when the satellite can exchange data with the core network. This is depicted in Figure 4.13.9.1-1.

For example, a simple sequence of events for the transmission of data from the UE to a server on the ground may involve two steps: firstly, when service link is available (e.g. location L1 in Figure 4.13.9.1-1) the UE sends the data to the satellite. Subsequently (e.g. location L2 in Figure 4.13.9.1-1), the satellite carrying the payload connects to the ground network and delivers the UE data to the network.

Downlink data can be stored onto the same or a different satellite and provided to the UE later using the first step and the process is repeated.

When a satellite supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in S&F Mode and if a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

NOTE 2:	Some deployments cannot support all system features, for example support user plane establishment and user plane data transfer cannot be supported, when a satellite is operating in S&F Mode with the split MME deployment described in Annex O.

The MME may adapt periodic update timer, mobile reachable timer and Implicit Detach timer to the fact that the UE is operating in S&F Mode.

For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode:

-	If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation.

-	The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure.

NOTE 3:	How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc.

When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause.

The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE can monitor to (re)attempt NAS procedures or receive MT data.

If the MME does not send a S&F Monitoring List or a S&F Wait Timer in a NAS procedure, the UE can continue to use the available S&F Monitoring List, S&F Wait Timer or both, if the UE determines the satellite is operating in S&F Mode until the MME sends a new S&F Monitoring List, or S&F Wait Timer or both.

NOTE Y:	How UE behaves when receiving the S&F Monitoring List is up to UE implementation. For example, if the UE determines a satellite is operating in S&F Mode, the UE can follow the S&F Monitoring List to ensure successful access..

-	The MME may indicate to the UE the Uplink S&F estimated delivery time in a NAS accept messages (i.e. Attach Accept, TAU Accept or Service Accept messages).

NOTE 4:	The Uplink S&F estimated Delivery time is the estimated to deliver the data to the ground from the time the data is sent by the UE. How UE uses this information is left for UE implementation.

If the UE did not indicate Store and Forward capability in NAS signalling, if the network determines to reject the UE, the network shall reject the UE request with a cause non-specific to S&F Mode.

The MME may trigger a Update Location procedure with the HSS along with the authentication procedure to fetch subscription information from the HSS. The MME may indicate the timestamp information to HSS during the Update Location procedure. This timestamp information shall be used by the HSS to ensure that newer location for that UE is not cancelled. If MME makes an Update Location Request before the completion of the authentication procedure, it shall include an indication that this Location Update is provisional, i.e. the HSS shall not consider the UE as registered until it receives the final Update Location Request (without indication that the Location Update is provisional). The timestamp information is the time when the on-board of satellite MME part has received the NAS procedure from the UE. The HSS compares the timestamp received in the Update Location Request with any stored timestamp of a previous Update Location Request and determines whether to accept or reject the request. If the received Update Location Request does not include a timestamp, the HSS assumes the present time as the timestamp of the received Update Location Request. The HSS shall reject the Update Location Request, if the timestamp associated with this request is older than the stored timestamp. If the HSS accepts the Update Location Request, the HSS shall store the timestamp associated with the latest Update Location Request. If the HSS does not support the timestamp, the MME shall reject a UE Attach request for S&F Mode.

NOTE 5:	The timestamp information is used by the HSS to ensure that interactions with MMEs for a UE are handled in the correct order.

NOTE 6:	The absence of timestamp in the Update Location Request can occur e.g. when the UE is connecting from a network that does not operate in S&F mode.

The EPS may expose whether a UE is in S&F Mode and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE. This is further described in clause 5.6.3.10 of TS 23.682 [74].

# * * * * End of changes * * * *