SA WG2 Meeting #S2-166-Ad-Hoc-e	S2-2500805

20 - 24 January, 2025, Electronic meeting	(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5286 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the AF provisioning procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the clause 4.14.6.15, it said “the procedure defined in clause 4.15.6.7 is performed with the following considerations”. But in the clause 4.15.6.7.2, the AM-PCF is involved which is not needed for this case. So this CR proposes to clarify this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying the steps performed for the AF provisioning of non-3GPP device indetifier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The step 5 to 8 of procedure in the 4.15.6.7 are not performed for AF provisioning of non-3GPP device indetifier information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.7.2, 4.15.6.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 4.15.6.7.2	Service specific parameter provisioning by AF to HPLMN

Figure 4.15.6.7.2-1 shows procedure for service specific parameter provisioning. The AF uses Nnef_ServiceParameter service to provide the service specific parameters to the HPLMN and the UE. In the roaming case, PCF is replaced by H-PCF, the AMF interacts with the V-PCF which interacts with H-PCF.

Figure 4.15.6.7.2-1: Service specific information provisioning by AF to HPLMN

0a.	The AMF establishes UE Policy Association as specified in clause 4.16.11.

0b.	PCF requests notifications from the UDR on changes in UE policy information.

1.	To create a new request, the AF invokes an Nnef_ServiceParameter_Create service operation. The request may include subscription information to the report of the outcome of UE Policy delivery.

To update or remove an existing request, the AF invokes an Nnef_ServiceParameter_Update or Nnef_ServiceParameter_Delete service operation together with the corresponding Transaction Reference ID which was provided to the AF in Nnef_ServiceParameter_Create response message.

The content of this service operation (AF request) includes the information described in clause 5.2.6.11.

2.	The AF sends its request to the NEF. The NEF authorizes the AF request. The NEF performs the following mappings:

-	Map the AF-Service-Identifier into DNN and S-NSSAI combination, determined by local configuration.

-	Map the External Application Identifier into the corresponding Application Identifier known in the core network.

2a.	The NEF may invoke Nudm_SDM_Get service operation to perform the following mappings:

-	Map the GPSI in Target UE Identifier into SUPI, according to information received from UDM.

-	Map the External Group Identifier in Target UE Identifier into Internal Group Identifier, according to information received from UDM.

If the AF subscribed to the outcome of UE Policy delivery, the AF indicates where the AF receives the corresponding notifications.

(in the case of Nnef_ServiceParameter_Create): The NEF assigns a Transaction Reference ID to the Nnef_ServiceParameter_Create request.

2b.		(in the case of Nnef_ServiceParameter_Create or Update): The NEF may need to authorize the service specific parameter provisioning request with the UDM by sending a Nudm_ServiceSpecificAuthorisation_Create service operation as defined in clause 4.15.6.7a.

NOTE 2:	The NEF skips the mapping of GPSI or External Group Identifier in step 2a if it needs to authorize the service specific parameter provisioning request with the UDM as the response of the authorization request from UDM includes the SUPI or Internal Group Identifier.

(in the case of Nnef_ServiceParameter_delete): The NEF requests the UDM to remove the authorization of the service specific parameters provisioned by sending a Nudm_ServiceSpecificAuthorisation_Remove service operation.

3.	(in the case of Nnef_ServiceParameter_Create or Update): The NEF stores the AF request information in the UDR as the "Application Data" (Data Subset setting to "Service specific information") together with the assigned Transaction Reference ID.

(in the case of Nnef_ServiceParameter_delete): The NEF deletes the AF request information from the UDR.

4.	The NEF responds to the AF. In the case of Nnef_ServiceParameter_Create response message, the response message includes the assigned Transaction Reference ID.

If the UE is registered to the network and the PCF performs the subscription to notification to the data modified in the UDR by invoking Nudr_DM_Subscribe (AF service parameter provisioning information, SUPI, Data Set setting to "Application Data", Data Subset setting to "Service specific information") at step 0, the following steps are performed:

If the AF provision the Non-3GPP Device Identifier(s) Information as defined in the clause 4.15.6.15, the step 5 to 8 are skipped.

5.	The PCF(s) receive(s) a Nudr_DM_Notify notification of data change from the UDR.

NOTE 3:	PCF does not have to subscribe for each UE the application specific information, e.g. if PCF has already received the application specific information for a group of UE or for a DNN by a subscription of other UE. The same application specific information is delivered to every UE in a group or a DNN.

For PIN service, PCF generates the URSP rules with PIN ID in the Traffic Descriptor as specified in Table 6.6.2.1-2 of TS 23.503 [20].

6.	The PCF initiates UE Policy delivery as specified in clause 4.2.4.3.

7.	If the AF subscribed to notifications about the outcome of UE Policies delivery due to Service specific parameter provisioning and the PCF is notified of UE Policy Container from the AMF, the PCF notifies the UE Policy delivery result contained in the UE Policy container as the outcome of the procedure to NEF by sending Npcf_EventExposure_Notify including the SUPI, the list of GPSI(s) if available, and if provided in the step 2a, the Internal-Group-Id.

If the PCF is notified about UE Policy delivery failure from the AMF due to UE not reachable, the PCF may determine to retry step 6 of this procedure when the UE becomes reachable. In such a case, the PCF may report the interim status i.e. UE is temporarily unreachable as the outcome of the procedure to NEF by sending Npcf_EventExposoure_Notify.

If the PCF determines the failure of the UE Policies delivery procedure, the PCF notifies the failure with an appropriate cause such as UE is unreachable as the outcome of the procedure to NEF by sending Npcf_EventExposure_Notify.

If the PCF determines that it cannot yet deliver URSP Rules that are based on the service parameters from the AF, then the PCF may report in the interim status that URSP Rules have not yet been delivered by the PCF as the outcome of the procedure to NEF by sending Npcf_EventExposure_Notify.

The content of event reporting in this step is described in clause 6.1.3.18 of TS 23.503 [20].

8.	When the NEF receives Npcf_EventExposure_Notify, the NEF performs information mapping (e.g. AF Transaction Internal ID provided in Notification Correlation ID to AF Transaction ID, SUPI to GPSI, Internal-Group-Id to External-Group-Id, etc.) and triggers the appropriate Nnef_ServiceParameter_Notify message.

Next CHANGE

#### 4.15.6.15	Provisioning of Non-3GPP Device Identifier(s) Information

This clause describes the procedures to allow an AF to provision QoS information for the non-3GPP device(s) (as defined in clause 5.52 of TS 23.501 [2]) that require differentiated QoS treatment for a UE subscription to the 5G system via NEF. The AF may belong to the operator or to an external party.

To provision QoS information for the non-3GPP device(s) that require differentiated QoS treatment, the step 1 to 4 of procedure defined in clause 4.15.6.7.2 is performed with the following considerations:

1)	Service Description. Contains an AF-Service-Identifier indicating that the request is for providing non-3GPP device Identifier information.

2)	Service Parameters. Contains the non-3GPP Device Identifier Information which has the following parameters:

-	Non-3GPP Device Identifier is a generic string, which uniquely identifies a non-3GPP device behind a specific UE subscription.

-	QoS Reference or Individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

-	Optionally, DNN and S-NSSAI. This may be provided by the AF or may be determined by the NEF based on the AF-Service-Identifier if not provided by the AF.

-	Optionally, Flow description information as specified in clause 6.1.3.6 of TS 23.503 [20]

3)	The Target UE identifier(s). A specific UE identified by e.g. GPSI or SUPI.

The NEF sets the value of Data Subset Identifier to "Non-3GPP Device Identifier Information" in Nudr_DM_Create/Update/Delete service operations when storing/updating/deleting this data. NEF determines to use this data subset based on AF-Service-Identifier.

END OF CHANGES